## Computational Fluid Dynamics (CFD) Basic Running Procedure

## Steps based on Ahmed Body, but are transferable to other geometries.

## These steps do not claim to fully explain CFD - this document has been produced to allow the reader to run an iteration on a given geometry quickly and correctly. For more information see ANSYS help pages or university tutorials.

## Max Hart     Ainsley Martin

## Loading in and setting up Geometry

Component Systems

Drag ‘geometry’ box onto screen

Right click question mark (may have to wait a bit)

New DesignModeler Geometry

File

Import External Geometry File

Generate

Sketch

Select rectangle tool and sketch around the body and feet. Don’t intersect line below geometry, draw slightly above

Click dimensions. Horizontal - full width and distance from back of geometry to back of box

Vertical - full height and distance from bottom axis line to bottom box line

Alter the dimensions to roughly what is seen below. This is the domain

### Figure 1 - domain dimensions                                 (Simanungkalit, 2016)

Generate

Create

Boolean

Operation = subtract

Expand ‘2 parts, 2 bodies’ in parts tree and use this to select the following:

Target = domain

Tool = Geometry

Generate

Click ‘select faces’

Name each of the faces of the domain as follows. Once a face is named, press F8 to hide it so it isn’t selected twice. Click Generate after naming each face

Front = ‘Velocity_Inlet’

Back = ‘Pressure_Outlet’

Side by axis = ‘Symmetry’ (‘Symmetry_Right if analysing full car)

Other side = ‘Symmetry_Left’

Top = ‘Symmetry_Top’

Bottom = ‘Road’

Right click - show all faces

Tools - Freeze - Close (X in top right)

## Integrated Computer-aided Engineering and Manufacturing CFD

## (ICEM CFD) (ANSYS, 2017)

Component Systems.

Drag ICEM CFD to white geometry box so red guidelines appear and a chain is formed.

Double click white model.

Expand ‘parts’

Deselect all apart from ‘Solid_1_1’

Expand geometry and check surfaces

Click shaded cylinder icon at the top

Select repair geometry

Set tolerance to 0.0002

Apply

Right click on part

Create part

Name the part ‘feet’

Click shining arrow and click and drag to select the feet

Follow on-screen instructions for selection - press F9 to move body freely then F9 again to return to selection controls

Click dismiss

Repeat for ‘car’

Click dismiss

Click ‘create body’

Name it ‘Body’

Click two opposite corners of the domain

Click mesh tab

Click Global Mesh Setup

Scale Factor 1 - can set to 0.5 for twice as many elements but takes much longer.

Max element 1

Global Mesh Setup (currently on) - move to red logo (shell meshing parameters)

Mesh type = all tri

Mesh method = patch independent

Move to blue logo (volume meshing parameters)

Mesh type = Tetra/Mixed

Mesh Method = Robust (Octree)

Edge Criterion = 0.06 (use very small, ≈ 0.01, for very thin geometry. Use higher, ≈ 0.2, for thicker geometry)

Apply

Move to orange logo (prism meshing parameters)

Number of layers = 5

Move to next icon on the geometry tab (part mesh setup)

Tick Prism for Body, Feet, Road and set all to Number of layers = 5

It is good practice to press enter after imputing each number to confirm its entry

Set size of mesh: Body = 0.01    Feet = 0.005

Apply and Dismiss

Click ‘Create mesh density’

Set size to 0.1

Select Density location from ‘entity bounds’

Click arrow on box symbol and select all by clicking and dragging

Apply and Dismiss

Click on geometry tab, click ‘transform geometry’

Select density box using       and clicking top left node to select entire box

Select ‘scale geometry’

Set scales to roughly same size as domain (figure 5) - rough scale factors X = 1, Y = 1.6, Z = 1.6

Set centre point to ‘centroid’

Use ‘translate geometry’         to ensure correct positioning

Select Mesh tab and click ‘compute mesh’

Mesh type and method will already be set

Click compute

Go to edit mesh tab and click ‘display mesh quality’

Criterion = skew

Apply

If Skew is large (bars don’t increase in equal parts) - element size can be reduced to increase accuracy of results - speak to more experienced user if this does not solve issue.

Criterion = TGrid Skew

Apply

If skew is too large again here, global mesh size can be reduced to increase accuracy of results - speak to more experienced user if this does not solve issue.

Go back to mesh tab, ‘compute mesh’

Select ‘prism mesh’

Compute

Edit mesh tab

Select ‘repair mesh’

Click apply to repair any imperfections in mesh

## Setting Physical Modelling Capabilities - Fluent CFD

Launch Fluent - drag from component systems to white ‘model’ box

Double click setup

Turn off Workbench Colour Scheme

Set processor to parallel

Set Processes to 4

From tree, select models

Viscous Laminar

K-Epsilon - select non-equilibrium wall functions

Boundary Conditions

Check that for body and feet type = wall and all symmetry sides type = symmetry

Double click Pressure_Outlet and check gauge pressure is 0

Double click velocity inlet check that velocity specification method = magnitude & direction

Set velocity to desired value (35 m/s is good if unsure)

Set turbulent intensity to 1%

Reports

Projected Areas

Select Ahmed and Feet

Set minimum feature size to 0.001m

Click compute to generate frontal area - copy this value

Reference Values

Compute from Velocity_Inlet

Paste in frontal area value

Methods

Scheme = coupled

Pressure = standard

Momentum = First Order Upwind

Controls

Flow Courant Number = 50

Momentum & Pressure = 0.25

Turbulent Kinetic Energy & Turbulent Dissipation Rate = 0.5

Turbulent Velocity = 0.8

Click limits

+ 4 zeros to Maximum Turbulent Viscosity Ratio

Report Definitions

New

Force Report - Drag

Name ‘cd’

Select Body and Feet

Tick ‘report file’, ‘report plot’ and ‘print to console’

Repeat this but for ‘cl’ (lift)

Initialisation

Click Initialise

If result says ‘convergence tolerance not reached’ this will hinder accuracy. Re-mesh may be required - speak to more experienced user if this does not solve issue.

Click Run Calculation (Sirius CFD, 2012)

Set number of iterations to 100

After 100 iterations change the following and run for another 100 iteration

Momentum = Second Order Upwind

Kinetic x2 = Second Order Upwind

Controls

Turbulent Kinetic Energy & Turbulent Dissipation Rate = 0.9

Turbulent Velocity = 0.95

Then run until converged

# Bibliography

ANSYS, 2017. ANSYS Workbench Help, Canonsburg, PA: ANSYS.

ANSYS, 2017. Introduction to ANSYS ICEM CFD. [Online] 
Available at: https://www.ansys.com/en-gb/services/training-center/platform/introduction-to-ansys-icem-cfd

Simanungkalit, S., 2016. CFD flow domain. [Online] 
Available at: https://www.researchgate.net/profile/Sabar_Simanungkalit

Sirius CFD, 2012. Part 3a - ANSYS Fluent for Vehicle Aerodynamics: YouTube.

With thanks to:

Ainsley Martin (UWTSD), John Hughes (UWTSD)