Workshop Skills/Learning in the Digital Era

Workshop Skills Logbook Hand-in

Joshua Ofosu Asare

2111739

BEng Embedded Electronic Systems

Figure 1: A screenshot of the circuit in proteus

The figure above shows the circuit as created in Proteus.

The circuit board was tested on a breadboard by attaching the function generator signal to the input of the circuit and CH1 of the oscilloscope to the same point. The output was connected to CH2 of the oscilloscope.

To assemble the PCB, a soldering iron, fume extractor and unleaded solder were used. To solder each of the connections, the components with just two pins were inserted into each hole and bend so they would not fall out when the PCB was inverted to solder from beneath.

LM741 – Proteus Simulation

| Frequency (Hz) |  | Output Voltage (mV) | Frequency (Hz) |  | Output Voltage (mV) |
| --- | --- | --- | --- | --- | --- |
| 100 |  | 300 | 200 |  | 300 |
| 500 |  | 300 | 1K |  | 300 |
| 2K |  | 300 | 5K |  | 300 |
| 10K |  | 300 | 20K |  | 300 |
| 50K |  | 300 | 100K |  | 288.75 |
| 200K |  | 245 | 500K |  | 115 |

Max Value = 300 * 0.707 = 212.50mV
-3dB point is 212.50mV at 250kHz
Input voltage = 100mV, Gain = 3

Gain Bandwidth Product = 750kHz

TL071 – Proteus Simulation

| Frequency (Hz) |  | Output Voltage (mV) | Frequency (Hz) |  | Output Voltage (mV) |
| --- | --- | --- | --- | --- | --- |
| 100 |  | 300 | 200 |  | 300 |
| 500 |  | 300 | 1K |  | 300 |
| 2K |  | 300 | 5K |  | 300 |
| 10K |  | 300 | 20K |  | 300 |
| 50K |  | 300 | 100K |  | 298.75 |
| 200K |  | 292.50 | 500K |  | 267.50 |
| 1M |  | 210 | 2M |  | 130 |

Max Value = 300 * 0.707 = 212.50mV
-3dB point is 212.50mV at 980kHz
Input voltage = 100mV, Gain = 3

Gain Bandwidth Product = 2940kHz

Breadboard CA741C - Practical

| Frequency (Hz) |  | Output Voltage (mV) | Frequency (Hz) |  | Output Voltage (mV) |
| --- | --- | --- | --- | --- | --- |
| 100 |  | 600 | 200 |  | 600 |
| 500 |  | 600 | 1K |  | 600 |
| 2K |  | 600 | 5K |  | 600 |
| 10K |  | 600 | 20K |  | 600 |
| 50K |  | 600 | 100K |  | 380 |
| 200K |  | 292.50 | 500K |  | 267.50 |
| 1M |  | 210 | 2M |  | 130 |

Max Value = 600 * 0.707 = 424.30mV
-3dB point is 424.30mV at 86.66kHz
Input voltage = 200mV, Gain = 3

Gain Bandwidth Product = 259.98kHz

PCB CA741C - Practical

| Frequency (Hz) |  | Output Voltage (mV) | Frequency (Hz) |  | Output Voltage (mV) |
| --- | --- | --- | --- | --- | --- |
| 100 |  | 600 | 200 |  | 600 |
| 500 |  | 600 | 1K |  | 600 |
| 2K |  | 600 | 5K |  | 600 |
| 10K |  | 600 | 20K |  | 600 |
| 50K |  | 572 | 100K |  | 348 |
| 200K |  | 180 |  |  |  |

Max Value = 600 * 0.707 = 424.30mV
-3dB point is 424.3 mV at 79.8KHz
Input voltage = 200mV, Gain = 3

Gain Bandwidth Product = 239.4kHz