Engine Performance Characteristics

University of Wales Trinity Saint David

School of Engineering

Engine Characteristics

William Hare

2219871

Motorsport Engineering BEng

20/03/2023

Motorsport Vehicle Technology

ACHA4004

# Contents Page

Contents

Contents Page	1

Introduction	1

Methods	2

Results	2

Graphs	4

Conclusion	5

Evaluation	5

References	6

Appendix	8

# Introduction

The engine dynameter is vital in the development and design of an engine, this is because it runs the engine as if it were connected to a car in real time using high powered magnets to replicate loads which affect the speed at which the engine is running at (Jiang et al., 2009). The loads experiences by the engine will simulate what the engine will experience during its real world application which it does so by altering the torque and the speed of the engine independently (Bunker et al., 1997). During the test, the engine used was a 1.6L Honda CRX 4 cylinder. The engine dynamometer was then used to find the values of load in Newtons, Speed in revolution per minute, fuel flow rate in L/min and lambda (see appendix 1). The Speed was the independent variable for the test in which the RPM was increases from 1000 going up by 50 each time. From increasing the speed the other values mentioned above can be read from sensors inside the engine dyno. All of which are given and from that other calculated vales can be found. There include power, torque, BMEP, SFC and thermal efficiency of the engine.

The purpose of the test was to find the values of brake power, torque, brake mean effective pressure (BMEP), specific fuel consumption (SFC) and the thermal efficiency on the engine. This report will discuss the aims of the test during the use of the engine dynameter. The purpose of the test was to be able to calculate a selection of values shown above. From the calculated results, Conclusions are then to be drawn with supporting evidence from graphs found from the data collected.

# Methods

The function of the dynamometer is to resist and measure the torque, simulating the change in environment the engine will experience (Grobbelaar et al., 2017). The type of dynamometer that was used to determine the data that was given is an eddy current dynamometer. This works by a stationary magnetic field, whose strength can be determined by the adjusted torque, and a solid magnetic rotating part. The rotating part is exposed to the magnetic field causing the part to decrease its speed (Sinaga et al., 2019). The dynamometer was run with varying speeds and magnet strength, connected to the 1.6L Honda engine to measure the values of load, fuel flow rate and lambda as the engine speed is increased. The load is the power of the engine (Kean et al., 2003) whilst flow rate is the amount of fuel in litres used per minute and lambda is the ratio of oxygen present during combustion (Baklacioglu, 2016; Tang et al., 2021). The engine speed is the independent variable and was increased at 50rpm each time staring at 1000rpm and ending at 5500rpm. The speed of the engine and strength of the magnets inside the dynamometer can be adjusted in the control room. The host computer sends signals like accelerate/decelerate or increased/decreased torque to the ECU which relays the command to the dynamometer to introduce a change to the engine’s working environment. Once that change has been recognised, the data showing the engine’s speed will be produced as well as the flow rate, load, and lambda (Jiang et al., 2009).

# Results

Below in table 1 are the data values that have been found from the calculations. The brake power is the useable power output from the engine (Chunkaew et al., 2016). The results from the testing show an increase in the brake power as the engine speed increases. This increase is from 8.7kW to 41.6kW which is due to the torque being increased. However, the values for toque, the engines rotational force, also increase with engine speed but only up to a point, 103.4Nm (Rizzoni, 1989). After that it drops off due to the engine not being able to consume enough air so the volumetric efficiency declines (Franco et al., 2008). The brake mean effective pressure (BMEP) can be defined as the ratio of the brake work per cycle to the cylinder volume displaced per cycle (Rahman et al., 2009). The data for break mean effective pressure increases from 658kPa to 817kPa and then also decreases back down to 571kPa like torque. This may be because of friction caused between the piston and the cylinder as the engine speed is increased. If BMEP equals indicated mean effective power (IMEP) minus frictional mean effective power (FMEP), and the value for FMEP gets larger then that results in a lower value for the BMEP.

BMEP = IMEP - FMEP

The specific fuel consumption (SFC) is defined as the fuel flow rate required by the engine per unit power produced (Costa & Sodré, 2011). It decreases in value from 0.360kg/kWh to 0.260kg/kWh which shows as the engine speed and the brake power of the car increase, the fuel consumption decreases. However, it then increases from 0.260kg/kWh to 0.374kg/kWh once the engine hits higher rpms. From this it can be understood that when a vehicle is at higher revs, it will also consume more fuel showing it is less fuel efficient. The thermal efficiency of an engine is the measure of the ability of an engine to convert the energy in fuel into mechanical energy, powering the vehicle (Ikeya et al., 2015). The thermal efficiency of the engine is the ratio of the work done output to the heat output (Ikeya et al., 2015b). The data shows the engine thermal efficiency increase to a point but then decrease after this.

Table 1

# Graphs

Graph 1

Graph 1 shows the relationship between the specific fuel consumption and the engine speed of the engine. The trendline shows a positive gradient of 2x10-5 and crosses the Y axis at 0.2447. However, at the start of the graph the SFC decreases and the Engine Speed increases. This is due to the engine not producing enough power whilst the fuel is being entered (Irawan et al., 2021). At around 1800rpm the graph starts to increase from then. The increase in fuel consumption shows that more fuel is used when the engine is at higher rpms.

Graph 2

The relationship between Engine Speed and Torque in graph 2 shows a negative correlation as stated by the trendline, with a gradient of -0.0031. The Torque initially increases as the Engine Speed increases due to the engine gaining power. It then decreases until the end point. This decrease is because of the engine not getting enough air (Zweiri et al., 1999).

Graph 3

Graph 3 above shows how the Brake Power of the engine is affected by the increase of the Engine Speed. The trendline shows a positive correlation with a gradient of 8.1291 showing an increase in the Brake Power as the Engine Speed increases. This is due to there being an increase in the power output from the engine because of the more work being done by the engine (Augusto et al., 2010).

# Conclusion

Overall, from the results conclusions can be drawn. The first being that none of the graphs or the results are inversely proportional. This shows that all the values can and will be affected by external factors whether it be friction occurred in the piston cylinder, or the quantity of the air intake. This will affect the results in different ways as some will show a positive correlation, but the graph will trail off to a more negative gradient or the opposite where its starts negative and corrects to a positive gradient (Rizzoni, 1989).  In addition, some of the values do not show strong correlations showing that the values do not change a lot when the speed of the engine increases. They tend to start strong but then reverse on themselves when they get to a certain point. This is because as the engine speed increases indefinitely the values get to a maximum value where then the engine will not run as efficiently. This is the case with thermal efficiency where it reaches the value 0.00309% where it then decreases. This is caused by the engine working a lot more as the speed of the engine is increases at which point it no longer become thermally efficient as the engine heats up because of the more work done (Ikeya et al., 2015b)

# Evaluation

The purpose of this evaluation is to see what was done well and what can be improved upon. During the tasks, I believe that I have performed well and been able to explain what the data shows and what can be interpretated as a result. The tasks ask for the methods and results to be explained from which I have done as well as explained what the values are that were to be found and why they are beneficial to be found. To improve the investigation control variables could be added to improve the validity of the raw data (Bernerth & Aguinis, 2016). Also, showing how the dynamometer is calibrated so students can see that the results given are accurate (Dynamometer Calibration and Usage, 2000).

# References

Augusto, F., Barbieri, A., Andreatta, C., Argachoy, C., & Brandao, H. (2010). Decompression Engine Brake Modeling and Design for Diesel Engine Application. Source: SAE International Journal of Engines, 3(2). https://doi.org/10.2307/26275549

Baklacioglu, T. (2016). Modeling the fuel flow-rate of transport aircraft during flight phases using genetic algorithm-optimized neural networks. Aerospace Science and Technology, 49, 52–62. https://doi.org/10.1016/J.AST.2015.11.031

Bernerth, J. B., & Aguinis, H. (2016). Education Complex. PERSONNEL PSYCHOLOGY, 69, 229–283. https://doi.org/10.1111/peps.12103

Bunker, B. J., Franchek, M. A., & Thomason, B. E. (1997). Robust Multivariable Control of an Engine-Dynamometer System. IEEE TRANSACTIONS ON CONTROL SYSTEMS TECHNOLOGY, 5(2), 189.

Chunkaew, P., Sriudom, Y., Jainoy, W., Sisa, J., Chuenprueng, K., & Chanpeng, W. (2016). Modified Compression Ratio Effect on Brake Power of Single Piston Gasoline Engine Utilizing Producer Gas. Energy Procedia, 89, 85–92. https://doi.org/10.1016/J.EGYPRO.2016.05.012

Costa, R. C., & Sodré, J. R. (2011). Compression ratio effects on an ethanol/gasoline fuelled engine performance. Applied Thermal Engineering, 31(2–3), 278–283. https://doi.org/10.1016/J.APPLTHERMALENG.2010.09.007

Dynamometer Calibration and Usage. (2000). https://apps.dtic.mil/sti/citations/ADA386680

Franco, J., Franchek, M. A., & Grigoriadis, K. (2008). Real-time brake torque estimation for internal combustion engines. Mechanical Systems and Signal Processing, 22(2), 338–361. https://doi.org/10.1016/J.YMSSP.2007.08.002

Grobbelaar, E., Richard, M., & Haines, W. (2017). The Development of a Small Diesel Engine Test Bench Employing an Electric Dynamometer. https://scholar.sun.ac.za

Hangai, Y., Kamada, H., Utsunomiya, T., Kitahara, S., Kuwazuru, O., & Yoshikawa, N. (2014). Aluminum alloy foam core sandwich panels fabricated from die casting aluminum alloy by friction stir welding route. Journal of Materials Processing Technology, 214(9), 1928–1934. https://doi.org/10.1016/J.JMATPROTEC.2014.04.010

Ikeya, K., Takazawa, M., Yamada, T., Park, S., & Tagishi, R. (2015a). Thermal Efficiency Enhancement of a Gasoline Engine. Source: SAE International Journal of Engines, 8(4), 1579–1586. https://doi.org/10.4271/2015-01-1263

Ikeya, K., Takazawa, M., Yamada, T., Park, S., & Tagishi, R. (2015b). Thermal Efficiency Enhancement of a Gasoline Engine. Source: SAE International Journal of Engines, 8(4), 1579–1586. https://doi.org/10.4271/2015-01-1263

Irawan, B., Winoko, Y. A., Puspitasari, E., & Dwiyono, T. (2021). Analysis of fuel pressure on the performance of motorcycle engine with ethanol fuel. IOP Conference Series: Materials Science and Engineering, 1073(1), 012081. https://doi.org/10.1088/1757-899X/1073/1/012081

Kean, A. J., Harley, R. A., & Kendall, G. R. (2003). Effects of Vehicle Speed and Engine Load on Motor Vehicle Emissions. Environmental Science and Technology, 37(17), 3739–3746. https://doi.org/10.1021/ES0263588

Rahman, M. M., Noor, M. M., Kadirgama, K., & Rejab, M. R. M. (2009). Study of Air Fuel Ratio on Engine Performance of Direct Injection Hydrogen Fueled Engine. European Journal of Scientific Research, 34(4), 506–513. http://www.eurojournals.com/ejsr.htm

Rizzoni, G. (1989). Estimate of indicated torque from crankshaft speed fluctuations: a model for the dynamics of the IC engine. https://doi.org/10.1109/25.45470

Sinaga, N., Syaiful, Yunianto, B., & Rifal, M. (2019). Experimental and computational study on heat transfer of a 150 kW air-cooled eddy current dynamometer. Journal of Physics: Conference Series, 1373(1), 012020. https://doi.org/10.1088/1742-6596/1373/1/012020

Tang, Q., Jiang, P., Peng, C., Chang, H., & Zhao, Z. (2021). Comparison and analysis of the effects of spark timing and lambda on a high-speed spark ignition engine fuelled with n-butanol/gasoline blends. Fuel, 287. https://doi.org/10.1016/J.FUEL.2020.119505

Zweiri, Y. H., Whidborne, J. F., & Seneviratne, L. D. (1999). Dynamic simulation of a single-cylinder diesel engine including dynamometer modelling and friction. Proceedings of the Institution of Mechanical Engineers, Part D: Journal of Automobile Engineering, 213(4), 391–402. https://www.academia.edu/3660986/Dynamic_simulation_of_a_single_cylinder_diesel_engine_including_dynamometer_modelling_and_friction

# Appendix

Appendix 1