# Networking Project

# Example Requirements

Project Overview:

Purpose: To design and implement a secure, scalable, and efficient network infrastructure for the Computing Department.

Scope: The network will support 200 wired and 150 wireless users, provide internet access, enable resource sharing, and ensure security compliance.

- Constraints:

Budget: --

Hardware: Must use Cisco networking equipment.

Timeframe: Project completion within 8 weeks.

# Functional Requirements

Network must support 200 wired and 150 wireless devices.

Implement VLANs for segmentation: Staff (VLAN 10), Students (VLAN 20), Guests (VLAN 30).

Provide DHCP and DNS services using Windows Server 2022.

Enable secure remote access via OpenVPN.

# Non-Functional Requirements

Performance: Minimum bandwidth of 100 Mbps per user.

Reliability: 99.9% uptime with redundant links.

Security: WPA3 for Wi-Fi, Cisco ASA firewall, AES-256 encryption for VPN.

Compliance: GDPR and university IT policies.

# Design Considerations

Topology: Star topology with core switch and access switches.

IP Addressing Scheme: Staff - 192.168.10.0/24, Students - 192.168.20.0/24, Guests - 192.168.30.0/24.

Protocols: TCP/IP, OSPF for routing, VLAN tagging (802.1Q).

# Hardware & Software Requirements

Routers: Cisco ISR 4331.

Switches: Cisco Catalyst 9300 Series.

Access Points: Cisco Aironet 1850 Series.

Server OS: Windows Server 2022 for DHCP/DNS.

Firewall: Cisco ASA 5506-X.

# Security Requirements

Firewall configuration with ACLs.

Access Control Lists (ACLs) for VLAN segmentation.

Intrusion Detection System (IDS) using Snort.

VPN encryption using AES-256.

# Testing & Validation

Connectivity Tests: Ping, traceroute between VLANs.

Performance Tests: iPerf for bandwidth and latency checks.

Security Tests: Penetration testing using Kali Linux, vulnerability scans with Nessus.

# Deliverables

Network design diagram using Cisco Packet Tracer.

Configuration files for routers, switches, and firewall.

Test reports for connectivity, performance, and security.

Final assignment documentation.