ACCA7029

ENTERPRISE CORE NETWORKING

- January 2023 Practical Solution

# Switch D1

interface range g1/0/1-4

switchport mode trunk

switchport trunk native vlan 999

channel-group 12 mode active

no shutdown

exit

interface range g1/0/5-6

switchport mode trunk

switchport trunk native vlan 999

channel-group 1 mode active

no shutdown

exit

spanning-tree mode rapid-pvst

spanning-tree vlan 100,102 root primary

spanning-tree vlan 101 root secondary

interface g1/0/23

switchport mode access

switchport access vlan 100

spanning-tree portfast

no shutdown

exit

end

# Switch D2

interface range g1/0/1-4

switchport mode trunk

switchport trunk native vlan 999

channel-group 12 mode active

no shutdown

exit

interface range g1/0/5-6

switchport mode trunk

switchport trunk native vlan 999

channel-group 2 mode active

no shutdown

exit

!

spanning-tree mode rapid-pvst

spanning-tree vlan 101 root primary

spanning-tree vlan 100,102 root secondary

!

interface g1/0/23

switchport mode access

switchport access vlan 102

spanning-tree portfast

no shutdown

exit

end

# Switch A1

spanning-tree mode rapid-pvst

interface range f0/1-2

switchport mode trunk

switchport trunk native vlan 999

channel-group 1 mode active

no shutdown

exit

interface range f0/3-4

switchport mode trunk

switchport trunk native vlan 999

channel-group 2 mode active

no shutdown

exit

interface f0/23

switchport mode access

switchport access vlan 101

spanning-tree portfast

no shutdown

exit

interface f0/24

switchport mode access

switchport access vlan 100

spanning-tree portfast

no shutdown

exit

end

# Part 3 Commands (Routing Protocols)

# Router R1

router ospf 4

router-id 0.0.4.1

network 10.0.10.0 0.0.0.255 area 0

network 10.0.13.0 0.0.0.255 area 0

default-information originate

exit

ipv6 router ospf 6

router-id 0.0.6.1

default-information originate

exit

interface g0/0/1

ipv6 ospf 6 area 0

exit

interface s0/1/0

ipv6 ospf 6 area 0

exit

!

ip route 10.0.0.0 255.0.0.0 null0

ipv6 route 2001:db8:100::/48 null0

!

router bgp 300

bgp router-id 1.1.1.1

neighbor 209.165.200.226 remote-as 500

neighbor 2001:db8:200::2 remote-as 500

address-family ipv4 unicast

neighbor 209.165.200.226 activate

no neighbor 2001:db8:200::2 activate

network 10.0.0.0 mask 255.0.0.0

exit-address-family

address-family ipv6 unicast

no neighbor 209.165.200.226 activate

neighbor 2001:db8:200::2 activate

network 2001:db8:100::/48

exit-address-family

# Router R2

ip route 0.0.0.0 0.0.0.0 loopback 0

ipv6 route ::/0 loopback 0

router bgp 500

bgp router-id 2.2.2.2

neighbor 209.165.200.225 remote-as 300

neighbor 2001:db8:200::1 remote-as 300

address-family ipv4

neighbor 209.165.200.225 activate

no neighbor 2001:db8:200::1 activate

network 2.2.2.2 mask 255.255.255.255

network 0.0.0.0

exit-address-family

address-family ipv6

no neighbor 209.165.200.225 activate

neighbor 2001:db8:200::1 activate

network 2001:db8:2222::/128

network ::/0

exit-address-family

# Router R3

router ospf 4

router-id 0.0.4.3

network 10.0.11.0 0.0.0.255 area 0

network 10.0.13.0 0.0.0.255 area 0

exit

ipv6 router ospf 6

router-id 0.0.6.3

exit

interface g0/0/1

ipv6 ospf 6 area 0

exit

interface s0/1/0

ipv6 ospf 6 area 0

exit

end

# Switch D1

router ospf 4

router-id 0.0.4.131

network 10.0.100.0 0.0.0.255 area 0

network 10.0.101.0 0.0.0.255 area 0

network 10.0.102.0 0.0.0.255 area 0

network 10.0.10.0 0.0.0.255 area 0

passive-interface default

no passive-interface g1/0/11

exit

ipv6 router ospf 6

router-id 0.0.6.131

passive-interface default

no passive-interface g1/0/11

exit

interface g1/0/11

ipv6 ospf 6 area 0

exit

interface vlan 100

ipv6 ospf 6 area 0

exit

interface vlan 101

ipv6 ospf 6 area 0

exit

interface vlan 102

ipv6 ospf 6 area 0

exit

end

# Switch D2

router ospf 4

router-id 0.0.4.132

network 10.0.100.0 0.0.0.255 area 0

network 10.0.101.0 0.0.0.255 area 0

network 10.0.102.0 0.0.0.255 area 0

network 10.0.11.0 0.0.0.255 area 0

passive-interface default

no passive-interface g1/0/11

exit

ipv6 router ospf 6

router-id 0.0.6.132

passive-interface default

no passive-interface g1/0/11

exit

interface g1/0/11

ipv6 ospf 6 area 0

exit

interface vlan 100

ipv6 ospf 6 area 0

exit

interface vlan 101

ipv6 ospf 6 area 0

exit

interface vlan 102

ipv6 ospf 6 area 0

exit

end

# Part 4 Commands (FHRP/SLA)

# Switch D1

ip sla 4

icmp-echo 10.0.10.1

frequency 5

exit

ip sla 6

icmp-echo 2001:db8:100:1010::1

frequency 5

exit

ip sla schedule 4 life forever start-time now

ip sla schedule 6 life-forever start-time now

track 4 ip sla 4

delay down 10 up 15

exit

track 6 ip sla 6

delay down 10 up 15

exit

interface vlan 100

standby version 2

standby 104 ip 10.0.100.254

standby 104 priority 150

standby 104 preempt

standby 104 track 4 decrement 60

standby 106 ipv6 autoconfig

standby 106 priority 150

standby 106 preempt

standby 106 track 6 decrement 60

exit

interface vlan 101

standby version 2

standby 114 ip 10.0.101.254

standby 114 preempt

standby 114 track 4 decrement 60

standby 116 ipv6 autoconfig

standby 116 preempt

standby 116 track 6 decrement 60

exit

interface vlan 102

standby version 2

standby 124 ip 10.0.102.254

standby 124 priority 150

standby 124 preempt

standby 124 track 4 decrement 60

standby 126 ipv6 autoconfig

standby 126 priority 150

standby 126 preempt

standby 126 track 6 decrement 60

exit

end

# Switch D2

ip sla 4

icmp-echo 10.0.11.1

frequency

exit

ip sla 6

icmp-echo 2001:db8:100:1011::1

frequency

exit

ip sla schedule 4 life forever start-time now

ip sla schedule 6 life forever start-time now

track 4 ip sla 4

delay down 10 up 15

exit

track 6 ip sla 6

delay down 10 up 15

exit

interface vlan 100

standby version 2

standby 104 ip 10.0.100.254

standby 104 preempt

standby 104 track 4 decrement 60

standby 106 ipv6 autoconfig

standby 106 preempt

standby 106 track 6 decrement 60

exit

interface vlan 101

standby version 2

standby 114 ip 10.0.101.254

standby 114 priority 150

standby 114 preempt

standby 114 track 4 decrement 60

standby 116 ipv6 autoconfig

standby 116 priority 150

standby 116 preempt

standby 116 track 6 decrement 60

exit

interface vlan 102

standby version 2

standby 124 ip 10.0.102.254

standby 124 preempt

standby 124 track 4 decrement 60

standby 126 ipv6 autoconfig

standby 126 preempt

standby 126 track 6 decrement 60

exit

end

# Part 5 Commands (Security)

# All Devices:

enable algorithm-type SCRYPT secret cisco12345cisco

username sadmin privilege 15 algorithm-type SCRYPT secret cisco12345cisco

! All devices except R2:

aaa new-model

radius server RADIUS

address ipv4 10.0.100.6 auth-port 1812 acct-port 1813

key $trongPass

exit

aaa authentication login default group radius local

end

# Part 6 Commands (Net Management)

Set local clock to UTC on all devices.

# Router R2:

ntp master 3

end

# Router R1

! enable and enter password

ntp server 2.2.2.2

logging trap warning

logging host 10.0.100.5

logging on

ip access-list standard SNMP-NMS

permit host 10.0.100.5

exit

snmp-server contact Cisco Student

snmp-server community ENCORSA ro SNMP-NMS

snmp-server host 10.0.100.5 version 2c ENCORSA

snmp-server ifindex persist

snmp-server enable traps bgp

snmp-server enable traps config

snmp-server enable traps ospf

end

# Router R3

ntp server 10.0.10.1

logging trap warning

logging host 10.0.100.5

logging on

ip access-list standard SNMP-NMS

permit host 10.0.100.5

exit

snmp-server contact Cisco Student

snmp-server community ENCORSA ro SNMP-NMS

snmp-server host 10.0.100.5 version 2c ENCORSA

snmp-server ifindex persist

snmp-server enable traps config

snmp-server enable traps ospf

end

# Switch D1

ntp server 10.0.10.1

logging trap warning

logging host 10.0.100.5

logging on

ip access-list standard SNMP-NMS

permit host 10.0.100.5

exit

snmp-server contact Cisco Student

snmp-server community ENCORSA ro SNMP-NMS

snmp-server host 10.0.100.5 version 2c ENCORSA

snmp-server ifindex persist

snmp-server enable traps config

snmp-server enable traps ospf

end

# Switch D2

ntp server 10.0.10.1

logging trap warning

logging host 10.0.100.5

logging on

ip access-list standard SNMP-NMS

permit host 10.0.100.5

exit

snmp-server contact Cisco Student

snmp-server community ENCORSA ro SNMP-NMS

snmp-server host 10.0.100.5 version 2c ENCORSA

snmp-server enable traps config

snmp-server enable traps ospf

end

# Switch A1

ntp server 10.0.10.1

logging trap warning

logging host 10.0.100.5

logging on

ip access-list standard SNMP-NMS

permit host 10.0.100.5

exit

snmp-server contact Cisco Student

snmp-server community ENCORSA ro SNMP-NMS

snmp-server host 10.0.100.5 version 2c ENCORSA

snmp-server ifindex persist

snmp-server enable traps config

snmp-server enable traps ospf

end

# Device Configurations (Final)

# Router R1

R1# show run

Building configuration...

Current configuration : 3406 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$0C3pnVdgrnhnY9$uzGA.WZfcLg5IhuyJu22mIf.YyZ/83VgqbO3rXBDuwo

!

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

username sadmin privilege 15 secret 9 $9$XCO4pzqbRT.3EP$ymouLOQI5/o0FOkYDtA1ztejFra67MnkJJ5Y3bhyQe6

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

ip address 209.165.200.225 255.255.255.224

negotiation auto

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:200::1/64

!

interface GigabitEthernet0/0/1

ip address 10.0.10.1 255.255.255.0

negotiation auto

ipv6 address FE80::1:2 link-local

ipv6 address 2001:DB8:100:1010::1/64

ipv6 ospf 6 area 0

!

interface Serial0/1/0

ip address 10.0.13.1 255.255.255.0

ipv6 address FE80::1:3 link-local

ipv6 address 2001:DB8:100:1013::1/64

ipv6 ospf 6 area 0

!

interface Serial0/1/1

no ip address

!

router ospf 4

router-id 0.0.4.1

network 10.0.10.0 0.0.0.255 area 0

network 10.0.13.0 0.0.0.255 area 0

default-information originate

!

router bgp 300

bgp router-id 1.1.1.1

bgp log-neighbor-changes

neighbor 2001:DB8:200::2 remote-as 500

neighbor 209.165.200.226 remote-as 500

!

address-family ipv4

network 10.0.0.0

no neighbor 2001:DB8:200::2 activate

neighbor 209.165.200.226 activate

exit-address-family

!

address-family ipv6

network 2001:DB8:100::/48

neighbor 2001:DB8:200::2 activate

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 10.0.0.0 255.0.0.0 Null0

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

logging trap warnings

logging host 10.0.100.5

ipv6 route 2001:DB8:100::/48 Null0

ipv6 router ospf 6

router-id 0.0.6.1

default-information originate

!

snmp-server community ENCORSA RO SNMP-NMS

snmp-server contact Cisco Student

snmp-server enable traps ospf state-change

snmp-server enable traps ospf errors

snmp-server enable traps ospf retransmit

snmp-server enable traps ospf lsa

snmp-server enable traps ospf cisco-specific state-change nssa-trans-change

snmp-server enable traps ospf cisco-specific state-change shamlink interface

snmp-server enable traps ospf cisco-specific state-change shamlink neighbor

snmp-server enable traps ospf cisco-specific errors

snmp-server enable traps ospf cisco-specific retransmit

snmp-server enable traps ospf cisco-specific lsa

snmp-server enable traps config

snmp-server enable traps bgp

snmp-server host 10.0.100.5 version 2c ENCORSA

!

radius server RADIUS

address ipv4 10.0.100.6 auth-port 1812 acct-port 1813

key $trongPass

!

control-plane

!

banner motd ^C R1, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

!

ntp server 2.2.2.2

!

end

# Router R2

R2# show run

Building configuration...

Current configuration : 2029 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R2

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$kWM5eeaWgcjgDk$klw0rmhA2j9zzPN13oTIYc/.yk9aczrrDxNq4rUNf5c

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

username sadmin privilege 15 secret 9 $9$xfCWZaD1xuZ5Q.$rje2SE7dafmrTg87ls/vn.PNtMXbaL3kfmN3Jr08yNU

!

redundancy

mode none

!

interface Loopback0

ip address 2.2.2.2 255.255.255.255

ipv6 address FE80::2:3 link-local

ipv6 address 2001:DB8:2222::1/128

!

interface GigabitEthernet0/0/0

ip address 209.165.200.226 255.255.255.224

negotiation auto

ipv6 address FE80::2:1 link-local

ipv6 address 2001:DB8:200::2/64

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

router bgp 500

bgp router-id 2.2.2.2

bgp log-neighbor-changes

neighbor 2001:DB8:200::1 remote-as 300

neighbor 209.165.200.225 remote-as 300

!

address-family ipv4

network 0.0.0.0

network 2.2.2.2 mask 255.255.255.255

no neighbor 2001:DB8:200::1 activate

neighbor 209.165.200.225 activate

exit-address-family

!

address-family ipv6

network ::/0

network 2001:DB8:2222::/128

neighbor 2001:DB8:200::1 activate

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 0.0.0.0 0.0.0.0 Loopback0

!

ipv6 route ::/0 Loopback0

!

control-plane

!

banner motd ^C R2, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

ntp master 3

!

end

# Router R3

R3# show run

Building configuration...

Current configuration : 2765 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$X1WR7NQHvbYXHY$HevkjyeTexlsUxwhnwaZWeh/VEB3CIoGxlPSJ9O.F6o

!

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

username sadmin privilege 15 secret 9 $9$y02cJ/kvRKO7DI$eYITN996n5QFlG2zu7OoHu2RLPwbw/8v8lO4nv/n8Aw

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.0.11.1 255.255.255.0

negotiation auto

ipv6 address FE80::3:2 link-local

ipv6 address 2001:DB8:100:1011::1/64

ipv6 ospf 6 area 0

!

interface Serial0/1/0

ip address 10.0.13.3 255.255.255.0

ipv6 address FE80::3:3 link-local

ipv6 address 2001:DB8:100:1010::2/64

ipv6 ospf 6 area 0

!

interface Serial0/1/1

no ip address

!

router ospf 4

router-id 0.0.4.3

network 10.0.11.0 0.0.0.255 area 0

network 10.0.13.0 0.0.0.255 area 0

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

logging trap warnings

logging host 10.0.100.5

ipv6 router ospf 6

router-id 0.0.6.3

!

snmp-server community ENCORSA RO SNMP-NMS

snmp-server contact Cisco Student

snmp-server enable traps ospf state-change

snmp-server enable traps ospf errors

snmp-server enable traps ospf retransmit

snmp-server enable traps ospf lsa

snmp-server enable traps ospf cisco-specific state-change nssa-trans-change

snmp-server enable traps ospf cisco-specific state-change shamlink interface

snmp-server enable traps ospf cisco-specific state-change shamlink neighbor

snmp-server enable traps ospf cisco-specific errors

snmp-server enable traps ospf cisco-specific retransmit

snmp-server enable traps ospf cisco-specific lsa

snmp-server enable traps config

snmp-server host 10.0.100.5 version 2c ENCORSA

!

radius server RADIUS

address ipv4 10.0.100.6 auth-port 1812 acct-port 1813

key $trongPass

!

control-plane

!

banner motd ^C R3, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

!

ntp server 10.0.10.1

!

end

# Switch D1

D1# show run

Building configuration...

Current configuration : 8260 bytes

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

!

hostname D1

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 9 $9$RWOFeoZQQ/zqJk$rEnKpZ9Dx6asfA/16o3cPHR3hYQvn2gFiZuybdaFo82

!

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

switch 1 provision ws-c3650-24ps

!

ip routing

!

no ip domain lookup

ip dhcp excluded-address 10.0.101.1 10.0.101.109

ip dhcp excluded-address 10.0.101.141 10.0.101.254

ip dhcp excluded-address 10.0.102.1 10.0.102.109

ip dhcp excluded-address 10.0.102.141 10.0.102.254

!

ip dhcp pool VLAN-101

network 10.0.101.0 255.255.255.0

default-router 10.0.101.254

!

ip dhcp pool VLAN-102

network 10.0.102.0 255.255.255.0

default-router 10.0.102.254

!

login on-success log

ipv6 unicast-routing

!

license boot level ipservicesk9

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 100,102 priority 24576

spanning-tree vlan 101 priority 28672

!

username sadmin privilege 15 secret 9 $9$yBNV4PYk3Zdpak$N2uvIju4cfG5jQsynRkIv0EHas6ivCZRAtkztAnLiVo

!

redundancy

mode sso

!

transceiver type all

monitoring

!

track 4 ip sla 4

delay down 10 up 15

!

track 6 ip sla 6

delay down 10 up 15

!

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data, LOGGING

class-map match-any system-cpp-default

description Inter FED, EWLC control, EWLC data

class-map match-any system-cpp-police-sys-data

description Learning cache ovfl, High Rate App, Exception, EGR Exception, NFLSAMPLED DATA, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-multicast

description Transit Traffic and MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

!

policy-map system-cpp-policy

!

!

interface Port-channel1

switchport trunk native vlan 999

switchport mode trunk

!

interface Port-channel12

switchport trunk native vlan 999

switchport mode trunk

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

!

interface GigabitEthernet1/0/1

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/2

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/3

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/4

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 10.0.10.2 255.255.255.0

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:100:1010::2/64

ipv6 ospf 6 area 0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 100

switchport mode access

spanning-tree portfast

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface Vlan1

no ip address

!

interface Vlan100

ip address 10.0.100.1 255.255.255.0

standby version 2

standby 104 ip 10.0.100.254

standby 104 priority 150

standby 104 preempt

standby 104 track 4 decrement 60

standby 106 ipv6 autoconfig

standby 106 priority 150

standby 106 preempt

standby 106 track 6 decrement 60

ipv6 address FE80::D1:2 link-local

ipv6 address 2001:DB8:100:100::1/64

ipv6 ospf 6 area 0

!

interface Vlan101

ip address 10.0.101.1 255.255.255.0

standby version 2

standby 114 ip 10.0.101.254

standby 114 preempt

standby 114 track 4 decrement 60

standby 116 ipv6 autoconfig

standby 116 preempt

standby 116 track 6 decrement 60

ipv6 address FE80::D1:3 link-local

ipv6 address 2001:DB8:100:101::1/64

ipv6 ospf 6 area 0

!

interface Vlan102

ip address 10.0.102.1 255.255.255.0

standby version 2

standby 124 ip 10.0.102.254

standby 124 priority 150

standby 124 preempt

standby 124 track 4 decrement 60

standby 126 ipv6 autoconfig

standby 126 priority 150

standby 126 preempt

standby 126 track 6 decrement 60

ipv6 address FE80::D1:4 link-local

ipv6 address 2001:DB8:100:102::1/64

ipv6 ospf 6 area 0

!

router ospf 4

router-id 0.0.4.131

passive-interface default

no passive-interface GigabitEthernet1/0/11

network 10.0.10.0 0.0.0.255 area 0

network 10.0.100.0 0.0.0.255 area 0

network 10.0.101.0 0.0.0.255 area 0

network 10.0.102.0 0.0.0.255 area 0

!

ip forward-protocol nd

ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

!

ip sla 4

icmp-echo 10.0.10.1

frequency 5

ip sla schedule 4 life forever start-time now

ip sla 6

icmp-echo 2001:DB8:100:1010::1

frequency 5

ip sla schedule 6 life forever start-time now

logging trap warnings

logging host 10.0.100.5

ipv6 router ospf 6

router-id 0.0.6.131

passive-interface default

no passive-interface GigabitEthernet1/0/11

!

snmp-server community ENCORSA RO SNMP-NMS

snmp-server contact Cisco Student

snmp-server enable traps ospf state-change

snmp-server enable traps ospf errors

snmp-server enable traps ospf retransmit

snmp-server enable traps ospf lsa

snmp-server enable traps ospf cisco-specific state-change nssa-trans-change

snmp-server enable traps ospf cisco-specific state-change shamlink interface

snmp-server enable traps ospf cisco-specific state-change shamlink neighbor

snmp-server enable traps ospf cisco-specific errors

snmp-server enable traps ospf cisco-specific retransmit

snmp-server enable traps ospf cisco-specific lsa

snmp-server enable traps config

snmp-server host 10.0.100.5 version 2c ENCORSA

snmp ifmib ifindex persist

!

radius server RADIUS

address ipv4 10.0.100.6 auth-port 1812 acct-port 1813

key $trongPass

!

control-plane

service-policy input system-cpp-policy

!

banner motd ^C D1, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

stopbits 1

line vty 5 15

!

ntp server 10.0.10.1

!

end

# Switch D2

D2# show run

Building configuration...

Current configuration : 8208 bytes

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

!

hostname D2

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 9 $9$CQubYNwHPhsPpE$QWfTfAlfzmWD3ELHkcFNzlDlp24FkpjLnGBRMPbUNow

!

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

switch 1 provision ws-c3650-24ps

!

ip routing

!

no ip domain lookup

ip dhcp excluded-address 10.0.101.1 10.0.101.209

ip dhcp excluded-address 10.0.101.241 10.0.101.254

ip dhcp excluded-address 10.0.102.1 10.0.102.209

ip dhcp excluded-address 10.0.102.241 10.0.102.254

!

ip dhcp pool VLAN-101

network 10.0.101.0 255.255.255.0

default-router 10.0.101.254

!

ip dhcp pool VLAN-102

network 10.0.102.0 255.255.255.0

default-router 10.0.102.254

!

login on-success log

ipv6 unicast-routing

!

license boot level ipservicesk9

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 100,102 priority 28672

spanning-tree vlan 101 priority 24576

!

username sadmin privilege 15 secret 9 $9$0bnG9yhbASQv9k$geQoMT2qxu1ItBXC5pl/SOR2YeWhqDOW0lsMIsicQDw

!

redundancy

mode sso

!

transceiver type all

monitoring

!

track 4 ip sla 4

delay down 10 up 15

!

track 6 ip sla 6

delay down 10 up 15

!

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data, LOGGING

class-map match-any system-cpp-default

description Inter FED, EWLC control, EWLC data

class-map match-any system-cpp-police-sys-data

description Learning cache ovfl, High Rate App, Exception, EGR Exception, NFLSAMPLED DATA, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-multicast

description Transit Traffic and MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

!

policy-map system-cpp-policy

!

interface Port-channel2

switchport trunk native vlan 999

switchport mode trunk

!

interface Port-channel12

switchport trunk native vlan 999

switchport mode trunk

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

!

interface GigabitEthernet1/0/1

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/2

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/3

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/4

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 10.0.11.2 255.255.255.0

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:100:1011::2/64

ipv6 ospf 6 area 0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 102

switchport mode access

spanning-tree portfast

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface Vlan1

no ip address

!

interface Vlan100

ip address 10.0.100.2 255.255.255.0

standby version 2

standby 104 ip 10.0.100.254

standby 104 preempt

standby 104 track 4 decrement 60

standby 106 ipv6 autoconfig

standby 106 preempt

standby 106 track 6 decrement 60

ipv6 address FE80::D2:2 link-local

ipv6 address 2001:DB8:100:100::2/64

ipv6 ospf 6 area 0

!

interface Vlan101

ip address 10.0.101.2 255.255.255.0

standby version 2

standby 114 ip 10.0.101.254

standby 114 priority 150

standby 114 preempt

standby 114 track 4 decrement 60

standby 116 ipv6 autoconfig

standby 116 priority 150

standby 116 preempt

standby 116 track 6 decrement 60

ipv6 address FE80::D2:3 link-local

ipv6 address 2001:DB8:100:101::2/64

ipv6 ospf 6 area 0

!

interface Vlan102

ip address 10.0.102.2 255.255.255.0

standby version 2

standby 124 ip 10.0.102.254

standby 124 preempt

standby 124 track 4 decrement 60

standby 126 ipv6 autoconfig

standby 126 preempt

standby 126 track 6 decrement 60

ipv6 address FE80::D2:4 link-local

ipv6 address 2001:DB8:100:102::2/64

ipv6 ospf 6 area 0

!

router ospf 4

router-id 0.0.4.132

passive-interface default

no passive-interface GigabitEthernet1/0/11

network 10.0.11.0 0.0.0.255 area 0

network 10.0.100.0 0.0.0.255 area 0

network 10.0.101.0 0.0.0.255 area 0

network 10.0.102.0 0.0.0.255 area 0

!

ip forward-protocol nd

ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

!

ip sla 4

icmp-echo 10.0.11.1

frequency 5

ip sla schedule 4 life forever start-time now

ip sla 6

icmp-echo 2001:DB8:100:1011::1

frequency 5

ip sla schedule 6 life forever start-time now

logging trap warnings

logging host 10.0.100.5

ipv6 router ospf 6

router-id 0.0.6.132

passive-interface default

no passive-interface GigabitEthernet1/0/11

!

snmp-server community ENCORSA RO SNMP-NMS

snmp-server contact Cisco Student

snmp-server enable traps ospf state-change

snmp-server enable traps ospf errors

snmp-server enable traps ospf retransmit

snmp-server enable traps ospf lsa

snmp-server enable traps ospf cisco-specific state-change nssa-trans-change

snmp-server enable traps ospf cisco-specific state-change shamlink interface

snmp-server enable traps ospf cisco-specific state-change shamlink neighbor

snmp-server enable traps ospf cisco-specific errors

snmp-server enable traps ospf cisco-specific retransmit

snmp-server enable traps ospf cisco-specific lsa

snmp-server enable traps config

snmp-server host 10.0.100.5 version 2c ENCORSA

!

radius server RADIUS

address ipv4 10.0.100.6 auth-port 1812 acct-port 1813

key $trongPass

!

control-plane

service-policy input system-cpp-policy

!

banner motd ^C D2, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

stopbits 1

line vty 5 15

!

ntp server 10.0.10.1

!

end

# Switch A1

A1# show run

Building configuration...

Current configuration : 3102 bytes

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname A1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$W4yJyY0jfUFGt3$hgWzRhouqq81DGKiSw3oN3ICGIRFKI1TF9C4Qo2BoGk

!

username sadmin privilege 15 secret 9 $9$rlz/oiC6xETwLL$4MFl7ezehKgosutkpnwabhdf83xQOcDXYyW.dvyoneY

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

system mtu routing 1500

!

no ip domain-lookup

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

interface Port-channel1

switchport trunk native vlan 999

switchport mode trunk

!

interface Port-channel2

switchport trunk native vlan 999

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/4

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/5

shutdown

!

interface FastEthernet0/6

shutdown

!

interface FastEthernet0/7

shutdown

!

interface FastEthernet0/8

shutdown

!

interface FastEthernet0/9

shutdown

!

interface FastEthernet0/10

shutdown

!

interface FastEthernet0/11

shutdown

!

interface FastEthernet0/12

shutdown

!

interface FastEthernet0/13

shutdown

!

interface FastEthernet0/14

shutdown

!

interface FastEthernet0/15

shutdown

!

interface FastEthernet0/16

shutdown

!

interface FastEthernet0/17

shutdown

!

interface FastEthernet0/18

shutdown

!

interface FastEthernet0/19

shutdown

!

interface FastEthernet0/20

shutdown

!

interface FastEthernet0/21

shutdown

!

interface FastEthernet0/22

shutdown

!

interface FastEthernet0/23

switchport access vlan 101

switchport mode access

spanning-tree portfast edge

!

interface FastEthernet0/24

switchport access vlan 100

switchport mode access

spanning-tree portfast edge

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan100

ip address 10.0.100.3 255.255.255.0

ipv6 address FE80::A1:1 link-local

ipv6 address 2001:DB8:100:100::3/64

!

ip default-gateway 10.0.100.254

ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

!

logging trap warnings

logging host 10.0.100.5

!

snmp-server community ENCORSA RO SNMP-NMS

snmp-server contact Cisco Student

snmp-server enable traps config

snmp-server host 10.0.100.5 version 2c ENCORSA

!

radius server RADIUS

address ipv4 10.0.100.6 auth-port 1812 acct-port 1813

key $trongPass

!

banner motd ^C A1, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

line vty 5 15

!

ntp server 10.0.10.1

end