University of Wales Trinity Saint David

School of Engineering

Advanced Engine Design

M6X00410

CON-ROD ACCELERATIONS, FORCES & DISPLACEMENTS

Stuart Harries

Malcolm McDonald

04/11/19

Table of Contents

1	Introduction	1

1.1	Assumptions	1

2	Background Research	2

3	Methodology	3

3.1	Peak Gas Force	3

3.2	Inertia Loads	4

3.3	Compressive Force & Tensile Load	4

3.4	Compressive & Tensile Stresses	4

3.5	Nominal Static Safety Factors	5

3.6	Compression Distance & Tensile Stretch	5

4	Results & Analysis	6

4.1	Modified Engine	6

4.2	Peak Gas Forces	6

4.3	Inertia Loads	7

4.4	Compressive Force & Tensile Load	8

4.5	Compression & Tensile Stresses	8

4.6	Nominal Static Safety Factors	8

4.7	Compression Distance & Tensile Stretch	9

5	Conclusion	10

6	References	11

7	Appendix A	12

8	Appendix B	13

9	Appendix C Further Work	14

9.1	Methodology	14

9.2	Results & Analysis	16

Table of Tables

Table 1- Dimensions & masses for ford 1.6 engine (Powder Forged Steel)	1

Table 2- For modified engine (Titanium)	1

Table 3- Peak gas force results	6

Table 4- Compressive force (N) & tensile load (N)	8

Table 5- Compressive & tensile stresses	8

Table 6- Nominal static safety factors	9

Table 7- Compression distance & tensile stretch	9

Table 8- Titanium Con-Rod	9

Table 9- Piston ring	14

Table 10- Work done & power loss	16

Table 11- Torque	16

Table of Figures

Figure 1- Forces acting on the connecting rod (Rao, 2013)	2

Figure 2- Peak gas forces	3

Figure 3- Cylinder pressure	6

Figure 4- Graph showing the peak gas force for the production engine	7

Figure 5- Graph for inertia loads on rod	7

Figure 6- Inertia loads on rod (production & modified)	8

Figure 7- Cylinder pressure data for 6000rpm	12

Figure 8- Excel calculations for 6000rpm	12

Figure 9- Modified engine pressure for 10000rpm	13

Figure 10- calculations for 10000rpm (modified)	13

Figure 11- Work done calculations - Part 1	14

Figure 12- Work done calculations - Part 2	15

Figure 13- Torque calculations	15

Figure 14- Side force @ 6000rpm	16

Figure 15- Gas torque at 6000 rpm	17

Table of Equations

Equation 1- Peak gas force (N)	3

Equation 2- Inertia load (N)	4

Equation 3- First order acceleration	4

Equation 4- Second order acceleration	4

Equation 5- Maximum compressive force on rod (N)	4

Equation 6- Maximum tensile load on rod (N)	4

Equation 7- Stress	4

Equation 8- Nominal static safety factor	5

Equation 9- Distance/Stretch	5

# Introduction

The aim of this assignment is to calculate the forces, con-rod accelerations and displacements, using the Ford 1.6-cylinder pressure data provided, and for a modified engine. A Microsoft Excel simulation model has been created which is used to accurately calculate these results. Additionally, the work done per cylinder and torque of the engine has been calculated using the cylinder pressure data from the Ford 1.6 engine at 6,000 rpm.

## Assumptions

Table 1- Dimensions & masses for ford 1.6 engine (Powder Forged Steel)

| Bore Diameter | 0.079 | m |
| --- | --- | --- |
| Cross-sectional Area of Con-Rod | 195 | mm2 |
| Ultimate Tensile Stress (UTS) | 750 | Mpa |
| Youngs Modulus (E) | 170 | Gpa |
| Stiffness (K) | 247388.06 |  |
| Crank Radius | 0.0407 | m |
| Con-Rod Length | 0.134 | m |
| Piston Mass | 0.2577 | kg |

Table 2- For modified engine (Titanium)

| Titanium |  |  |
| --- | --- | --- |
| UT S | 950 | Mpa |
| Youngs Modulus | 130 | Gpa |
| Stiffness (K) | 189179.1 |  |

# Background Research

A connecting rod connects the piston head to the crankshaft. The connecting rod is an integral part of an internal combustion engine and when designing the connecting rod, the forces acting on the rod must be taken into consideration. This is shown in figure 1. (Rao, 2013)

The connecting rod may be made of carbon steel or any other alloy. (Dh, 2017)

Figure 1- Forces acting on the connecting rod (Rao, 2013)

# Methodology

The data for the production engine is obtained from a Ford 1.6 engine with the rpm ranging from 1,000rpm to 6,000rpm. For the modified engine, the engine rpm is increased to 10,000rpm. 10,000rpm has been chosen as modern F1 engines are running at a maximum 15,000rpm (FIA, 2019), which is based on similar size engines, which are also turbo-charged. It should be reasonable to expect the modified engine therefore to achieve 10,000rpm.

To determine the increase in cylinder pressure for the modified engine; the peak gas forces were calculated for each rpm from 1,000 to 6,000; and the graph shown in figure 2 was created. The trendline shown then gives the equation y = 3.7774x + 25424 and x is replaced with the required rpm. This gives the maximum gas force for 10,000rpm. This value for the gas force is divided by the peak gas force for 6,000rpm to find the percentage increase. The mean cylinder pressure for 6,000 rpm is multiplied by the percentage increase.

Figure 2- Peak gas forces

## Peak Gas Force

To calculate the peak gas force in the cylinder equation 1 is used. Pmax is the maximum pressure in the cylinder (Pa) and d is the bore diameter (m).

Peak Gas Force=pmax×πd24

Equation 1- Peak gas force (N)

## Inertia Loads

To calculate the inertia loads that the piston exerts on the rod, equation 2 is used. First Order and Second Order are the piston accelerations and φ is the rod angle. The rod angle is included in this equation to display the inertia load across the rod.  is the rod angle.

Inertia Load=Piston Mass ×First Order+Second Order×cos

Equation 2- Inertia load (N)

First Order Acceleration=R×ω2×  cosθ

Equation 3- First order acceleration

Second Order Accleration= RL ×R×ω2×cos

Equation 4- Second order acceleration

Where first and second order are the piston accelerations.

## Compressive Force & Tensile Load

Equation 5 is used to calculate the maximum compressive force, and equation 6 is used to calculate the maximum tensile load.

Maximum Compressive Force on Rod =  pmax×(πd24)- Inertia Force (At Pmax)cos

Equation 5- Maximum compressive force on rod (N)

Maximum Tensile Load on Rod= Inertia Force-Gas Force(Max)cos

Equation 6- Maximum tensile load on rod (N)

## Compressive & Tensile Stresses

To calculate the compression stress, the maximum compressive force on rod is used. For the tensile stress, the maximum tensile load on rod is used.

σ=FA

Equation 7- Stress

## Nominal Static Safety Factors

Using the results calculated for compressive and tensile stresses, equation 8 is used to calculate the nominal static safety factor for both compressive and tensile.

Safety Factor =Ultimate Tensile Strength (UTS)Stress (σ)

Equation 8- Nominal static safety factor

## Compression Distance & Tensile Stretch

This equation is used to calculate the maximum compression and stretch, using the results calculated for compression and tensile stresses.

Maximum Distance/Stretch(x)= ( Stress (σ) ×Area) Stiffness (K)

Equation 9- Distance/Stretch

Refer to Appendix A and Appendix B for the Excel Calculations for the production engine and modified engine.

# Results & Analysis

## Modified Engine

The graph shown in figure 3 has been created to demonstrate the increase in cylinder pressure between 6,000rpm and 10,000rpm.

Figure 3- Cylinder pressure

## Peak Gas Forces

Table 3 shows the results that were calculated for the peak gas forces in the cylinder. From the calculations the peak gas force for the production engine was at 6,000rpm, and 10,000rpm for the modified engine.

Table 3- Peak gas force results

| Peak Gas Forces |  |
| --- | --- |
| Production (N) | Modified (N) |
| 46862.91 | 63198 |

The peak gas forces have been calculated for other rpms in the cylinder pressure data set and a graph (see figure 4) is created to demonstrate when the rpm is increased, the gas force in the cylinders will also increase.

Figure 4- Graph showing the peak gas force for the production engine

## Inertia Loads

Figure 5- Graph for inertia loads on rod

Figure 5 is a graph that shows the inertia loads which the piston exerts on the rod per crank angle degree. As the graphs shows, when the rpm is increased, the inertial loads will also increase. This is further shown in figure 6 which compares the inertia loads with 6,000rpm for the production engine, and 10,000rpm for the modified engine.

Figure 6- Inertia loads on rod (production & modified)

## Compressive Force & Tensile Load

Table 4 shows the highest net compressive and tensile force for the production and modified engine.

Table 4- Compressive force (N) & tensile load (N)

| Compressive Force (N) |  | Tensile Load (N) |  |
| --- | --- | --- | --- |
| Production | Modified | Production | Modified |
| 41929.46 | 49498.53 | 4080.22 | 13210.20 |

## Compression & Tensile Stresses

Table 5 shows the maximum compression and tensile stresses in the rod shank.

Table 5- Compressive & tensile stresses

| Compressive Stress (N/mm) |  | Tensile Stress (N/mm) |  |
| --- | --- | --- | --- |
| Production | Modified | Production | Modified |
| 215.02 | 253.84 | 20.92 | 67.74 |

## Nominal Static Safety Factors

Table 6 are the calculated nominal static safety factors, for both the compressive and tensile forces.

Table 6- Nominal static safety factors

| Compressive Safety Factor (N) |  | Tensile Safety Factor (N) |  |
| --- | --- | --- | --- |
| Production | Modified | Production | Modified |
| 3.49 | 2.95 | 35.84 | 11.07 |

## Compression Distance & Tensile Stretch

Table 7 shows the compression distance and tensile stretch of the rod where the material used is powder forged steel. Table 8 shows the results for the modified engine, where the material of the rod has been changed with an identical design in Titanium. The results for the modified engine show that when changing the material, from powder forged steel to titanium, the maximum compression distance and stretch have increased.

Table 7- Compression distance & tensile stretch

| Compression Distance (mm) |  | Tensile Stretch (mm) |  |
| --- | --- | --- | --- |
| Production | Modified | Production | Modified |
| 0.17 | 0.20 | 0.02 | 0.05 |

| Max Compression Distance (mm) | 0.26 |
| --- | --- |
| Tensile Stretch (mm) | 0.07 |

Table 8- Titanium Con-Rod

This would also result in manufacturing tolerances being closer on the modified engine, with a view to the increased movements in the component parts, as shown above. If these are within the operating tolerances, the modified engine should produce a significant increase in power, and, if the tolerances are adhered to, the engine should be reliable.

# Conclusion

The conclusion I have arrived at is that, by increasing the rpm, there is a significant increase in pressure. This of course exerts higher stresses on the connecting rod. This can be mitigated by utilising titanium in the components, which will give an element of safety over the standard engine. By using a higher specification material, it is possible to safely increase the maximum power available. Also, at higher rpm, the tolerances of the engine will become more important, and will bear a relationship to the overall reliability of the engine.

Whilst it may be possible to achieve such high rpm’s, these bring with it other problems such as additional cooling and the larger pumps and radiators to dissipate the heat. These need to be considered along with the specific engine needs. As always, there is the trade-off between weight and power.

# References

Dh, M. &. (2017). DESIGN AND ANALYSIS OF A CONNECTING ROD. International Journal of Pure and Applied Mathematics, 105-109.

FIA. (2019). 2019 FORMULA ONE TECHNICAL REGULATIONS. Fédération Internationale de l’Automobile.

Nigus, H. (2015). Kinematics and Load Formulation of Engine Crank. Mechanics, Materials Science & Engineering Journal.

Rao, G. N. (2013). Design Optimization and Analysis of a Connecting Rod using ANSYS. International Journal of Science and Research, 225.

# Appendix A

Figure 7- Cylinder pressure data for 6000rpm

Figure 8- Excel calculations for 6000rpm

# Appendix B

Figure 9- Modified engine pressure for 10000rpm

Figure 10- calculations for 10000rpm (modified)

# Appendix C Further Work

## Methodology

Table 9- Piston ring

| Piston Ring Height | 0.0012 | m |
| --- | --- | --- |
| Piston Ring Inner Diameter | 0.076 | m |
| Piston Ring Thickness | 0.0015 | m |
| Coefficient of Friction | 0.1 |  |

Figure 11- Work done calculations - Part 1

Figure 12- Work done calculations - Part 2

Figure 13- Torque calculations

## Results & Analysis

Table 10 below shows the results calculated for the total work done per cylinder and the power loss per cylinder at 6,000rpm.

Table 10- Work done & power loss

| Total Work Done/Cylinder | 50.25 | Joules/Cylinder |
| --- | --- | --- |
| Power Loss/Cylinder | 2512.37 | Joules/second/Cylinder |

Figure 14 is a graph showing the results for side force at 6,000rpm. From analysing the graph, the maximum side force calculated will be approximately 30 degrees after the piston hits top dead centre (TDC).

Figure 14- Side force @ 6000rpm

Table 11 shows the results calculated for gas and inertia torque at 6,000rpm. The average gas torque is per cylinder with the mean gas torque is all cylinders in the engine.

Table 11- Torque

| Average Gas Torque (N/m) | 56.06 |
| --- | --- |
| Mean Gas Torque (N/m) | 224.26 |
| Average Inertia Torque (N/m) | 0.00 |

Figure 15- Gas torque at 6000 rpm

Figure 15 shows what happens to the gas torque over a full cycle.