Disclaimer:

- This material is a simplified guideline to the format of the exam.

- In the exam, you will be asked questions on topics outside this material.

- This material on its own is not enough to cover the questions in the exam.

- The new exam format is answer 4

- Each question will have (A, B, C, D…..) sections each section is either 5 marks or 10 marks.

- No multi choices in this exam.

Q1

- If the transmission channels were error-free, do we still need the data link layer?

The data link layer is still needed for framing the data and for flow control over the transmission channel. In a multiple access medium such as a LAN, the data link layer is required to coordinate access to the shared medium among the multiple users.

- Encrypt the message GOOD with the Caesar cipher with (+4) as the key?

Plain   Alphabet: A-B-C-D-E-F-G-H-I-J-K-L-M-N-O-P-Q-R-S-T-U-V-W-X-Y-Z

Caesar Alphabet: E-F-G-H-I-J-K-L-M-N-O-P-Q-R-S-T-U-V-W-X-Y-Z-A-B-C-D

Key is (+4), The length of the plain string should be the same as the cipher string

The encrypted massage is “KSSH”

- List the seven IOS layers.

Q2.

- Definitions Bandwidth, Refraction.

- Define the EM signals and what are the main properties of EM signals?

Period, frequency, phase, amplitude

- Explain how Wireless LAN (WLANs) are different from wired networks (LANs).

LAN uses cables at the Physical layer and MAC sub-layer of the Data Link layer.

LAN connect clients to the network through an Ethernet switch.

LAN uses collision-detection for media access to proactively avoid collisions within the media.

LAN devices are plugged-in it doesn’t support mobility.

WLANs use Wireless technologies instead of cables at the Physical layer and MAC sub-layer of the Data Link layer.

WLANs connect clients to the network through a wireless access point (AP).

WLANs connect mobile devices that are often battery powered.

WLAN uses collision-avoidance to proactively avoid collisions within the media

Q3.

- What meant by  Passive attacks

Snooping on transmissions

- To obtain information

- Release of message contents

- Outsider learns content of transmission

Traffic analysis

- By monitoring frequency and length of messages, even encrypted, communication may be guessed.

Q4.

- Explain the ESS in WLANs.

ESS: Two or more BSS interconnected by DS, Typically, DS is wired backbone but can be any network, Appears as single logical LAN to LLC

- What is meant by the signal multipath?

Signal can take many different paths between sender and receiver due to reflection, scattering, diffraction.

- What are the harmful effects of excessive exposure to infrared signals?

Infrared: IR raises the internal temperature of the eye, virtually “baking” it.