Engine Performance Characteristics

Powertrain Technology (ACHA4029)

Lewis Cummings

2500612

March, 2026

Contents

1	Introduction	3

2	Testing Equipment and Method	4

2.1	Overview	4

2.2	Testing Equipment	4

2.3	Method For Testing	4

3	Data	5

3.1	Overview	5

3.2	Obtained Data through Testing	5

3.3	Torque	7

3.4	Brake Power	7

3.5	Corrected Power	7

3.6	Specific Fuel Consumption (SFC)	8

3.7	Overall Thermal Efficiency	8

3.8	Brake Mean Effective Pressure (BMEP)	8

3.9	Data	8

4	Analysis of Data and Graphs	13

4.1	Overview	13

4.2	SFC Against Engine Speed (RPM)	13

4.2.1	Graph	13

4.2.2	Analysis	13

4.3	Torque Against Engine Speed	14

4.3.1	Graph	14

4.3.2	Analysis	14

4.4	Power Against Engine Speed	15

4.4.1	Graph	15

4.4.2	Analysis	15

5	Conclusions from Data	16

6	Improvements for the future	17

7	Reference list	18

No table of figures entries found.

# Introduction

The main goal of this report is to assess the performance of a Honda internal combustion (I.C.) engine using controlled dynamometer tests. Experimental data is collected at different engine speeds to calculate key indicators such as brake power, torque, Brake Mean Effective Pressure (BMEP), and Specific Fuel Consumption (SFC). The study also seeks to evaluate the engine's overall thermal efficiency. The power measurements are corrected to standard reference conditions by accounting for ambient factors such as barometric pressure, humidity, and temperature. By relating engine speed to these performance indicators, we identify the engine's optimum efficiency "sweet spot." Furthermore, the study includes thermodynamic and mechanical analyses of the causes of torque and fuel-economy decreases across different speeds. Finally, we interpret the results to determine the engine's suitability for its intended use in a powertrain system.

# Testing Equipment and Method

## Overview

In this section, the testing equipment is outlined, as is the method required to carry out the test.

## Testing Equipment

Performance testing of the Honda internal combustion engine was conducted in a dynamometer test cell. The main equipment is a hydraulic or eddy-current dynamometer connected to the engine's output shaft. It applies controlled loads, which are measured with a load cell and a torque arm that is 0.815776m long. For combustion monitoring, a Lambda sensor measures the air/fuel ratio and a fuel flow meter measures fuel consumption in litres/minute. The combustion environment is monitored with an electronic barometer, hygrometer, and thermometer that measure barometric pressure, humidity, and ambient temperature, respectively. This ensures data can be adjusted to standard reference conditions. All raw data signals are processed through a central data acquisition (DAQ) system and displayed using dedicated dyno software for real-time analysis of speed, torque, and power.

## Method For Testing

The performance testing used a steady-state dynamometer procedure to map the Honda engine's characteristics across its operating range. The ambient conditions of the dyno room were recorded first. The pressure was 1018.9 mBar, the ambient temperature was 27°C, and the relative humidity was 46%. The engine was then started and allowed to reach a stable operating temperature. This ensured consistent frictional characteristics and stable combustion. The test began at a baseline of 1000 RPM. The dynamometer applied a braking load to maintain a steady speed. Once the engine stabilised at this RPM, key parameters were recorded, including Load (N), Fuel Flow Rate (l/min), and Lambda. This process was repeated by sweeping through the engine's speed range in 50 RPM increments up to the maximum test speed. Finally, the collected raw data was processed in a spreadsheet to calculate performance indicators such as Torque (Nm), using the 0.815776m lever arm, Brake Power (kW), and SFC (kg/kWh).

# Data

## Overview

In this section, data acquired from the test will be collected in Excel to support further calculations to obtain brake power, torque, BMEP, SFC, and Overall Thermal Efficiency.

## Obtained Data through Testing

| Load | Dyno Speed | Fuel Flow Rate | Lambda |
| --- | --- | --- | --- |
| Newtons | rpm | l/min | Lambda |
| 102.275 | 1000 | 0.0704 | 1.2658 |
| 102.756 | 1050 | 0.0724 | 1.26783 |
| 103.689 | 1100 | 0.0737 | 1.27344 |
| 104.209 | 1150 | 0.0728 | 1.28476 |
| 105.811 | 1200 | 0.0744 | 1.29181 |
| 107.51 | 1250 | 0.07895 | 1.29729 |
| 109.488 | 1300 | 0.079 | 1.29895 |
| 111.797 | 1350 | 0.0796 | 1.29469 |
| 114.268 | 1400 | 0.0835 | 1.2855 |
| 115.783 | 1450 | 0.0847 | 1.27818 |
| 116.985 | 1500 | 0.08865 | 1.2795 |
| 117.788 | 1550 | 0.0932 | 1.27766 |
| 118.471 | 1600 | 0.0959 | 1.28235 |
| 118.772 | 1650 | 0.09875 | 1.29028 |
| 118.656 | 1700 | 0.10005 | 1.29942 |
| 118.861 | 1750 | 0.10305 | 1.31053 |
| 119.584 | 1800 | 0.1093 | 1.3149 |
| 119.934 | 1850 | 0.11415 | 1.31849 |
| 120.176 | 1900 | 0.11745 | 1.31892 |
| 120.948 | 1950 | 0.12055 | 1.31484 |
| 122.157 | 2000 | 0.12555 | 1.31653 |
| 122.845 | 2050 | 0.12785 | 1.32098 |
| 123.796 | 2100 | 0.1309 | 1.32287 |
| 124.794 | 2150 | 0.13435 | 1.32164 |
| 125.137 | 2200 | 0.1377 | 1.31652 |
| 126.028 | 2250 | 0.14105 | 1.30465 |
| 126.66 | 2300 | 0.14495 | 1.29646 |
| 126.822 | 2350 | 0.14985 | 1.28229 |
| 126.69 | 2400 | 0.1537 | 1.27304 |
| 126.552 | 2450 | 0.15565 | 1.26984 |
| 126.438 | 2500 | 0.1596 | 1.27446 |
| 126.239 | 2550 | 0.1626 | 1.27876 |
| 126.161 | 2600 | 0.16805 | 1.27634 |
| 125.847 | 2650 | 0.17145 | 1.28027 |
| 124.911 | 2700 | 0.17465 | 1.28371 |
| 123.927 | 2750 | 0.1784 | 1.2812 |
| 123.689 | 2800 | 0.18005 | 1.27448 |
| 123.389 | 2850 | 0.1831 | 1.26036 |
| 122.519 | 2900 | 0.1879 | 1.2531 |
| 121.837 | 2950 | 0.19255 | 1.23943 |
| 121.709 | 3000 | 0.199 | 1.22735 |
| 121.123 | 3050 | 0.20605 | 1.2193 |
| 121.124 | 3100 | 0.21005 | 1.20692 |
| 120.851 | 3150 | 0.21605 | 1.19274 |
| 120.596 | 3200 | 0.21795 | 1.18481 |
| 120.84 | 3250 | 0.22305 | 1.17378 |
| 121.633 | 3300 | 0.2255 | 1.16118 |
| 122.181 | 3350 | 0.2285 | 1.15104 |
| 122.933 | 3400 | 0.2342 | 1.14117 |
| 123.475 | 3450 | 0.23605 | 1.12669 |
| 123.428 | 3500 | 0.24195 | 1.11975 |
| 124.205 | 3550 | 0.24545 | 1.10622 |
| 124.066 | 3600 | 0.25505 | 1.09697 |
| 123.967 | 3650 | 0.25755 | 1.09401 |
| 123.815 | 3700 | 0.26445 | 1.09659 |
| 123.767 | 3750 | 0.2651 | 1.09808 |
| 123.509 | 3800 | 0.2693 | 1.09907 |
| 122.672 | 3850 | 0.27375 | 1.10533 |
| 122.157 | 3900 | 0.2759 | 1.11704 |
| 121.184 | 3950 | 0.28285 | 1.12259 |
| 120.004 | 4000 | 0.28455 | 1.12461 |
| 118.734 | 4050 | 0.2871 | 1.1222 |
| 117.442 | 4100 | 0.28895 | 1.12394 |
| 116.5 | 4150 | 0.29305 | 1.12233 |
| 115.931 | 4200 | 0.2949 | 1.12179 |
| 115.034 | 4250 | 0.29715 | 1.12456 |
| 114.391 | 4300 | 0.3022 | 1.12278 |
| 114.016 | 4350 | 0.30605 | 1.11943 |
| 113.871 | 4400 | 0.3092 | 1.11986 |
| 113.985 | 4450 | 0.3099 | 1.11873 |
| 113.306 | 4500 | 0.31445 | 1.11563 |
| 113.074 | 4550 | 0.3171 | 1.10953 |
| 112.195 | 4600 | 0.3194 | 1.10569 |
| 111.061 | 4650 | 0.32055 | 1.09782 |
| 109.973 | 4700 | 0.3207 | 1.09558 |
| 108.654 | 4750 | 0.32485 | 1.08827 |
| 107.076 | 4800 | 0.32455 | 1.08396 |
| 105.24 | 4850 | 0.3257 | 1.08221 |
| 103.632 | 4900 | 0.3277 | 1.0846 |
| 102.335 | 4950 | 0.32845 | 1.08823 |
| 101.106 | 5000 | 0.3287 | 1.08687 |
| 99.3149 | 5050 | 0.32985 | 1.0891 |
| 97.5815 | 5100 | 0.3297 | 1.09185 |
| 95.8486 | 5150 | 0.33095 | 1.09407 |
| 94.8033 | 5200 | 0.3338 | 1.09346 |
| 93.0976 | 5250 | 0.3313 | 1.08804 |
| 91.7886 | 5300 | 0.3324 | 1.08339 |
| 90.5938 | 5350 | 0.3308 | 1.08007 |
| 89.6374 | 5400 | 0.3388 | 1.07895 |
| 88.9256 | 5450 | 0.3438 | 1.07923 |
| 88.6474 | 5500 | 0.3488 | 1.08366 |

This is the data collected during the dynamometer testing. Values for the Load, Fuel Flow rate and Lambda are taken for dyno speeds between 1000 rpm and 5500 rpm, increasing by 50 each time. The gathered data show various values, such as peak brake power and fuel efficiency, at different rpm.

## Torque

We are able to calculate the torque using the Load in Newtons and the RPM.

TNm=LoadN×Distance From Load (m)

The value for the distance is 815.776 mm obtained from the dynamometer this is then calculated across all values.

## Brake Power

Brake power can be calculated using the calculated torque values and the engine speed (rpm)

2πNT60

Where N is the engine speed, and T is the torque in NM. Engineers must account for both rotational speed and torque magnitude because, as the fundamental principle of mechanics suggests, torque is the "rotational force" that dictates the rate of energy transfer. Consequently, calculating power requires balancing these two factors, since "power is the product of torque and angular velocity."(TrueGeometry, 2024)

## Corrected Power

Corrected power is a key factor when considering powertrain performance. To ensure fair comparisons across varying environments, corrected power normalises dynamometer results by accounting for atmospheric factors such as temperature, pressure, and humidity. This process eliminates weather-related performance swings, preventing "a vehicle tested on a cool winter morning" from appearing artificially stronger than when tested on a hot summer afternoon. (Sodré and Soares, 2003) The equation for corrected power is

PsP=1.01325Pa×Ta293

Where Ps is the corrected power, P is the Brake power, Pa is the Ambient Pressure, and Ta is the ambient temperature.

## Specific Fuel Consumption (SFC)

Calculating SFC allows fuel efficiency at different engine rpm to be determined, identifying where it’s most efficient and what its best operating conditions are. SFC is calculated using the formula

SFC=V×0.75×3600t×1000×P

To calculate Specific Fuel Consumption (SFC), you multiply the measured fuel volume (Vin cc) by its specific gravity (0.75) and a time constant (3600) to convert the rate to kg/h.

## Overall Thermal Efficiency

Brake thermal efficiency represents the ratio of fuel energy converted into useful mechanical work. Most engines convert only about one-third of the fuel's heat into useful work, with the rest lost to exhaust, cooling, and radiation.  To calculate this, the fuel flow rate  must be converted to Kg/sec using the formula.

Flow Rate Kgs-1=Flow RateLitersmin×0.7560

Once the flow rate has been converted, the Overall Thermal Efficiency can be calculated using the following formula, where P is the brake power in kW, and C.V. is the calorific value of the fuel, roughly 44,000 kJ/Kg.

ηt=PFuel Flow Rate×C.V

## Brake Mean Effective Pressure (BMEP)

BMEP represents the theoretical constant pressure that, if "applied uniformly from TDC to BDC" (Pratik Ambukar, 9, 2025) during the power stroke, would produce the same brake power as the actual engine cycle. This average value allows for a direct comparison of engine performance across different displacements.

BMEP (kPa)=2π×T×nVd

Where T is the Torque in Nm, Vd is the engine displacement in (litres), and n is the Number of power strokes per revolution.

## Data

| Torque | Power | Power | Corrected Power | Corrected Power |
| --- | --- | --- | --- | --- |
| NM | KW | HP | KW | HP |
| 83.4335 | 8.7371 | 11.7120 | 8.7919 | 11.7853 |
| 83.8259 | 9.2171 | 12.3554 | 9.2749 | 12.4328 |
| 84.5870 | 9.7437 | 13.0613 | 9.8048 | 13.1431 |
| 85.0112 | 10.2377 | 13.7235 | 10.3018 | 13.8094 |
| 86.3181 | 10.8470 | 14.5403 | 10.9150 | 14.6314 |
| 87.7041 | 11.4804 | 15.3893 | 11.5523 | 15.4857 |
| 89.3177 | 12.1593 | 16.2994 | 12.2355 | 16.4015 |
| 91.2013 | 12.8933 | 17.2832 | 12.9740 | 17.3915 |
| 93.2171 | 13.6663 | 18.3195 | 13.7519 | 18.4342 |
| 94.4530 | 14.3421 | 19.2253 | 14.4319 | 19.3457 |
| 95.4336 | 14.9907 | 20.0947 | 15.0846 | 20.2206 |
| 96.0886 | 15.5967 | 20.9071 | 15.6944 | 21.0380 |
| 96.6458 | 16.1932 | 21.7066 | 16.2946 | 21.8426 |
| 96.8913 | 16.7416 | 22.4419 | 16.8465 | 22.5824 |
| 96.7967 | 17.2321 | 23.0993 | 17.3400 | 23.2440 |
| 96.9640 | 17.7696 | 23.8198 | 17.8809 | 23.9690 |
| 97.5538 | 18.3885 | 24.6494 | 18.5036 | 24.8038 |
| 97.8393 | 18.9546 | 25.4083 | 19.0733 | 25.5674 |
| 98.0367 | 19.5061 | 26.1476 | 19.6283 | 26.3114 |
| 98.6665 | 20.1480 | 27.0081 | 20.2742 | 27.1773 |
| 99.6527 | 20.8712 | 27.9775 | 21.0020 | 28.1528 |
| 100.2140 | 21.5135 | 28.8385 | 21.6482 | 29.0191 |
| 100.9898 | 22.2088 | 29.7705 | 22.3479 | 29.9570 |
| 101.8040 | 22.9209 | 30.7251 | 23.0645 | 30.9175 |
| 102.0838 | 23.5184 | 31.5260 | 23.6657 | 31.7235 |
| 102.8106 | 24.2242 | 32.4721 | 24.3759 | 32.6755 |
| 103.3262 | 24.8867 | 33.3602 | 25.0426 | 33.5691 |
| 103.4583 | 25.4602 | 34.1290 | 25.6197 | 34.3427 |
| 103.3507 | 25.9749 | 34.8188 | 26.1376 | 35.0369 |
| 103.2381 | 26.4871 | 35.5055 | 26.6530 | 35.7279 |
| 103.1451 | 27.0033 | 36.1975 | 27.1725 | 36.4242 |
| 102.9827 | 27.5000 | 36.8633 | 27.6723 | 37.0942 |
| 102.9191 | 28.0219 | 37.5629 | 28.1975 | 37.7982 |
| 102.6630 | 28.4897 | 38.1900 | 28.6682 | 38.4292 |
| 101.8994 | 28.8114 | 38.6211 | 28.9918 | 38.8631 |
| 101.0967 | 29.1138 | 39.0265 | 29.2961 | 39.2709 |
| 100.9025 | 29.5862 | 39.6597 | 29.7715 | 39.9082 |
| 100.6578 | 30.0414 | 40.2700 | 30.2296 | 40.5223 |
| 99.9481 | 30.3530 | 40.6876 | 30.5431 | 40.9425 |
| 99.3917 | 30.7044 | 41.1587 | 30.8967 | 41.4165 |
| 99.2873 | 31.1920 | 41.8124 | 31.3874 | 42.0743 |
| 98.8092 | 31.5592 | 42.3046 | 31.7569 | 42.5695 |
| 98.8101 | 32.0768 | 42.9984 | 32.2778 | 43.2678 |
| 98.5873 | 32.5207 | 43.5935 | 32.7244 | 43.8665 |
| 98.3793 | 32.9672 | 44.1920 | 33.1737 | 44.4688 |
| 98.5784 | 33.5501 | 44.9733 | 33.7602 | 45.2550 |
| 99.2253 | 34.2898 | 45.9649 | 34.5046 | 46.2528 |
| 99.6723 | 34.9662 | 46.8715 | 35.1852 | 47.1651 |
| 100.2858 | 35.7065 | 47.8639 | 35.9301 | 48.1637 |
| 100.7279 | 36.3913 | 48.7819 | 36.6193 | 49.0875 |
| 100.6896 | 36.9047 | 49.4701 | 37.1358 | 49.7799 |
| 101.3235 | 37.6675 | 50.4926 | 37.9035 | 50.8089 |
| 101.2101 | 38.1553 | 51.1465 | 38.3943 | 51.4669 |
| 101.1293 | 38.6544 | 51.8155 | 38.8965 | 52.1401 |
| 101.0053 | 39.1358 | 52.4609 | 39.3810 | 52.7895 |
| 100.9661 | 39.6493 | 53.1492 | 39.8977 | 53.4821 |
| 100.7557 | 40.0942 | 53.7456 | 40.3454 | 54.0823 |
| 100.0729 | 40.3465 | 54.0838 | 40.5992 | 54.4225 |
| 99.6527 | 40.6989 | 54.5561 | 40.9538 | 54.8979 |
| 98.8590 | 40.8923 | 54.8155 | 41.1485 | 55.1588 |
| 97.8964 | 41.0067 | 54.9688 | 41.2636 | 55.3131 |
| 96.8603 | 41.0799 | 55.0669 | 41.3372 | 55.4119 |
| 95.8064 | 41.1346 | 55.1402 | 41.3922 | 55.4855 |
| 95.0379 | 41.3022 | 55.3649 | 41.5609 | 55.7117 |
| 94.5737 | 41.5957 | 55.7583 | 41.8562 | 56.1076 |
| 93.8420 | 41.7652 | 55.9855 | 42.0268 | 56.3362 |
| 93.3174 | 42.0204 | 56.3276 | 42.2836 | 56.6804 |
| 93.0115 | 42.3696 | 56.7957 | 42.6350 | 57.1515 |
| 92.8932 | 42.8021 | 57.3755 | 43.0702 | 57.7349 |
| 92.9862 | 43.3319 | 58.0856 | 43.6033 | 58.4494 |
| 92.4323 | 43.5577 | 58.3883 | 43.8305 | 58.7541 |
| 92.2431 | 43.9515 | 58.9162 | 44.2268 | 59.2853 |
| 91.5260 | 44.0891 | 59.1006 | 44.3652 | 59.4708 |
| 90.6009 | 44.1178 | 59.1392 | 44.3942 | 59.5096 |
| 89.7133 | 44.1554 | 59.1895 | 44.4319 | 59.5603 |
| 88.6373 | 44.0899 | 59.1017 | 44.3660 | 59.4719 |
| 87.3500 | 43.9069 | 58.8565 | 44.1819 | 59.2251 |
| 85.8523 | 43.6036 | 58.4498 | 43.8767 | 58.8160 |
| 84.5405 | 43.3800 | 58.1501 | 43.6517 | 58.5144 |
| 83.4824 | 43.2742 | 58.0083 | 43.5453 | 58.3717 |
| 82.4798 | 43.1863 | 57.8905 | 43.4569 | 58.2532 |
| 81.0187 | 42.8455 | 57.4337 | 43.1139 | 57.7934 |
| 79.6046 | 42.5145 | 56.9900 | 42.7808 | 57.3469 |
| 78.1910 | 42.1689 | 56.5267 | 42.4331 | 56.8808 |
| 77.3383 | 42.1140 | 56.4531 | 42.3778 | 56.8067 |
| 75.9468 | 41.7539 | 55.9704 | 42.0155 | 56.3210 |
| 74.8789 | 41.5589 | 55.7090 | 41.8192 | 56.0579 |
| 73.9042 | 41.4049 | 55.5026 | 41.6643 | 55.8502 |
| 73.1240 | 41.3507 | 55.4299 | 41.6097 | 55.7771 |
| 72.5434 | 41.4021 | 55.4989 | 41.6615 | 55.8465 |
| 72.3164 | 41.6513 | 55.8328 | 41.9122 | 56.1825 |

| SFC | Overall Thermal Efficiency | BMEP |
| --- | --- | --- |
| Kg/kWh | % | kPa |
| 0.3626 | 22.56 | 655.2851 |
| 0.3535 | 23.15 | 658.3669 |
| 0.3404 | 24.04 | 664.3447 |
| 0.3200 | 25.57 | 667.6764 |
| 0.3087 | 26.51 | 677.9406 |
| 0.3095 | 26.44 | 688.8262 |
| 0.2924 | 27.98 | 701.4994 |
| 0.2778 | 29.45 | 716.2934 |
| 0.2749 | 29.76 | 732.1253 |
| 0.2658 | 30.79 | 741.8321 |
| 0.2661 | 30.75 | 749.5334 |
| 0.2689 | 30.43 | 754.6783 |
| 0.2665 | 30.70 | 759.0543 |
| 0.2654 | 30.82 | 760.9829 |
| 0.2613 | 31.32 | 760.2396 |
| 0.2610 | 31.35 | 761.5531 |
| 0.2675 | 30.59 | 766.1854 |
| 0.2710 | 30.19 | 768.4279 |
| 0.2710 | 30.20 | 769.9784 |
| 0.2692 | 30.39 | 774.9247 |
| 0.2707 | 30.23 | 782.6709 |
| 0.2674 | 30.59 | 787.0789 |
| 0.2652 | 30.85 | 793.1721 |
| 0.2638 | 31.02 | 799.5664 |
| 0.2635 | 31.05 | 801.764 |
| 0.2620 | 31.23 | 807.4727 |
| 0.2621 | 31.22 | 811.522 |
| 0.2649 | 30.89 | 812.5599 |
| 0.2663 | 30.73 | 811.7142 |
| 0.2644 | 30.94 | 810.83 |
| 0.2660 | 30.76 | 810.0996 |
| 0.2661 | 30.75 | 808.8246 |
| 0.2699 | 30.32 | 808.3248 |
| 0.2708 | 30.21 | 806.313 |
| 0.2728 | 29.99 | 800.316 |
| 0.2757 | 29.67 | 794.0114 |
| 0.2739 | 29.88 | 792.4865 |
| 0.2743 | 29.83 | 790.5644 |
| 0.2786 | 29.37 | 784.9902 |
| 0.2822 | 28.99 | 780.6206 |
| 0.2871 | 28.50 | 779.8005 |
| 0.2938 | 27.85 | 776.0459 |
| 0.2947 | 27.77 | 776.0523 |
| 0.2990 | 27.37 | 774.3032 |
| 0.2975 | 27.50 | 772.6694 |
| 0.2992 | 27.35 | 774.2327 |
| 0.2959 | 27.65 | 779.3135 |
| 0.2941 | 27.82 | 782.8246 |
| 0.2952 | 27.72 | 787.6428 |
| 0.2919 | 28.03 | 791.1154 |
| 0.2950 | 27.73 | 790.8143 |
| 0.2932 | 27.90 | 795.7926 |
| 0.3008 | 27.20 | 794.902 |
| 0.2998 | 27.29 | 794.2677 |
| 0.3041 | 26.91 | 793.2938 |
| 0.3009 | 27.19 | 792.9863 |
| 0.3023 | 27.07 | 791.3332 |
| 0.3053 | 26.80 | 785.9705 |
| 0.3051 | 26.82 | 782.6709 |
| 0.3113 | 26.29 | 776.4368 |
| 0.3123 | 26.20 | 768.8764 |
| 0.3145 | 26.02 | 760.7394 |
| 0.3161 | 25.88 | 752.4614 |
| 0.3193 | 25.63 | 746.426 |
| 0.3190 | 25.65 | 742.7803 |
| 0.3202 | 25.56 | 737.0332 |
| 0.3236 | 25.28 | 732.9134 |
| 0.3251 | 25.17 | 730.5107 |
| 0.3251 | 25.17 | 729.5817 |
| 0.3218 | 25.42 | 730.3121 |
| 0.3249 | 25.19 | 725.9617 |
| 0.3247 | 25.20 | 724.4753 |
| 0.3260 | 25.10 | 718.8434 |
| 0.3270 | 25.02 | 711.5778 |
| 0.3268 | 25.03 | 704.6069 |
| 0.3316 | 24.68 | 696.1559 |
| 0.3326 | 24.60 | 686.0455 |
| 0.3361 | 24.34 | 674.2821 |
| 0.3399 | 24.07 | 663.9795 |
| 0.3415 | 23.96 | 655.6695 |
| 0.3425 | 23.89 | 647.7952 |
| 0.3464 | 23.62 | 636.3195 |
| 0.3490 | 23.45 | 625.2134 |
| 0.3532 | 23.17 | 614.1106 |
| 0.3567 | 22.94 | 607.4132 |
| 0.3571 | 22.91 | 596.4847 |
| 0.3599 | 22.73 | 588.0978 |
| 0.3595 | 22.76 | 580.4426 |
| 0.3687 | 22.19 | 574.3149 |
| 0.3737 | 21.90 | 569.7543 |
| 0.3768 | 21.71 | 567.9718 |

# Analysis of Data and Graphs

## Overview

In the section, Graphs for SFC against Engine Speed (RPM), Torque against Engine Speed (RPM), and Brake Power against Engine Speed (RPM) are plotted and evaluated to determine their relationships and significance.

## SFC Against Engine Speed (RPM)

### Graph

### Analysis

The Specific Fuel Consumption (SFC) curve exhibits a characteristic U-shape (Heywood, 2018), reaching its most efficient point between 1500 and 2500 RPM. At lower speeds, efficiency decreases as slow-moving pistons allow more combustion heat to escape into the cooling system. Efficiency peaks in the mid-range, where air intake is most effective, but begins to fall again above 3000 RPM due to rising mechanical friction and shorter combustion windows. Consequently, the engine operates most economically in this mid-range "sweet spot" before becoming least efficient as it nears 5500 RPM.

## Torque Against Engine Speed

### Graph

### Analysis

This torque curve shows that the engine’s peak torque is about 103 NM at around 2500 RPM. This is where the engine achieves its best volumetric efficiency. Torque is lower at low speeds because the intake air does not move quickly enough to fill the cylinders properly. It decreases at high speeds, above 4000 RPM, because the valves open and close too quickly to allow complete air intake. Essentially, torque reflects how well the engine can "breathe" (Kamiński et al., 2023), decreasing as mechanical friction and pumping losses begin to exceed the energy produced by combustion.

## Power Against Engine Speed

### Graph

### Analysis

Power increases with engine speed because power measures how quickly the engine can do work. As RPM rises, the engine completes more power strokes each second. This causes horsepower to climb steadily from 12 HP at 1000 RPM to about 60 HP at 4700 RPM. However, this growth eventually stops because power is determined by torque and speed. As the engine spins faster, torque, which is the force per stroke, drops significantly after 4000 RPM due to pumping losses (Crolla, 2009). This means it becomes harder to push air in and out of the cylinders at high speeds. Once the loss in torque exceeds the gain from increased RPM, the power curve reaches its limit and begins to decline, as seen in the drop toward 5500 RPM.

# Conclusions from Data

The dynamometer test results confirm that the engine runs best within a specific sweet spot between 1500 and 2800 RPM. In this range, it has the lowest Specific Fuel Consumption and the highest thermal efficiency. Here, the balance between combustion stability and mechanical friction is optimised, making it the best range for steady-state operation (Renewable and Sustainable Energy Reviews, 2018). On the other hand, at higher rotational speeds, performance drops due to a noticeable decrease in torque and fuel economy. This drop is mainly caused by increased internal mechanical friction and pumping losses. These findings indicate that to achieve maximum efficiency in a powertrain application, the gearing should be set up to keep the engine within the 1500 to 2800 RPM range.

# Improvements for the future

The dynamometer was not working during the lab session, so I could not use the equipment myself. As a result, this analysis depends on a provided dataset and video recordings. While these were enough to calculate engine parameters, operating the equipment directly would have given me a better understanding of the engine's immediate physical responses, such as changes in sound and vibration under load. To improve future studies, I recommend running multiple test sweeps to average out data inconsistencies. Additionally, monitoring the environment should include intake and exhaust temperatures for a more thorough analysis of volumetric efficiency. Finally, ensuring the equipment is fully operational would enable a hands-on approach to managing throttle and load points, offering a clearer view of how factors such as Lambda and fuel flow interact in real time.

# Reference list

Crolla, D. (2009). Automotive engineering: powertrain, chassis system and vehicle body. Oxford, Uk; Burlington, Ma: Butterworth-Heinemann. [Accessed 15 Mar. 2026].

Heywood, J. (2018). Internal Combustion Engine Fundamentals. 2nd ed. McGraw-Hill Education.[Accessed 18 Mar. 2026].

https://www.facebook.com/pratik.ambukar.9 (2025). Brake mean effective pressure (BMEP): Definition, Formula. [online] Mech Content. Available at: https://mechcontent.com/internal- combustion-engine/brake-mean-effective-pressure/ [Accessed 17 Mar. 2026].

Kamiński, A., Konrad Krakowian, Skrętowicz, M. and Kupski, M. (2023). Efficiency Optimisation of a Vehicle Combustion Engine by the Adjustment of the Spark Advance Angle. Combustion Engines, [online] 196(1). doi:https://doi.org/10.19206/CE-175071.Accessed 19 Mar. 26AD].

Renewable and Sustainable Energy Reviews (2018). Engine Dynamometer - an overview | ScienceDirect Topics. [online] www.sciencedirect.com. Available at: https://www.sciencedirect.com/topics/engineering/engine-dynamometer [Accessed 17 Mar. 26AD].

Sodré, J.R. and Soares, S.M.C. (2003). Comparison of engine power correction factors for varying atmospheric conditions. Journal of the Brazilian Society of Mechanical Sciences and Engineering, [online] 25(3), pp.279–284. doi:https://doi.org/10.1590/s1678-58782003000300010.[Accessed 13 Mar. 2026].

TrueGeometry (2024). (2 (Pi) (N) (T)) / 60. [online] True Geometry’s Blog. Available at: https://blog.truegeometry.com/engineering/Analytics_Impact_of_Torque_on_Power_Calculation2_pi_n_T_60.html [Accessed 13 Mar. 2026].