| University of Wales Trinity St. David |  |  |
| --- | --- | --- |
| # An Investigation into Different Methods for Measuring the Chapman-Jouguet Pressure in a Cylinder Expansion Test |  |  |
| OME Integrated Apprenticeship Major Project Jennifer Mungham 1810444 |  |  |

# Release Conditions

This document has been prepared for MOD and, unless indicated, may be used and circulated in accordance with the conditions of the Order under which it was supplied.

It may not be used or copied for any non-Governmental or commercial purpose without the written agreement of Dstl.

© Crown Copyright 2022
Defence Science and Technology Laboratory UK

Approval for wider use or release must be sought from:

Intellectual Property Department
Defence Science and Technology Laboratory
Porton Down, Salisbury, Wiltshire SP4 0JQ

This information is released for Defence purposes and whilst it does not attract a national Security Protective Marking, elements of the information may be of a sensitive nature and therefore the information is to be:

- Handled, used and transmitted with care.

- Basic precautions against accidental compromise, opportunist or deliberate attack are to be taken.

- Disposed of sensibly by destroying in a manner to make reconstruction unlikely.

# Abstract

A series of Cylinder Expansion (CylEx) experiments were conducted to compare an experimental set-up for determining the Chapman-Jouguet pressure (PCJ) with two new methods: DALEX and McDAX. The new methods utilised heterodyne velocimetry to make the measurements with the view to replace Manganin stress sensors. Thirteen firings were conducted in total with the plastic explosive PE8; one with a steel tube and twelve with copper tubes. The resultant measurements of the velocity of detonation and calculation of the Chapman-Jouguet pressure for PE8 were compared to one another, those for other explosive materials and to previous work undertaken by the author. The existing method was shown to be affected by the planarity of the shock wave down the tube which gave a low precision or reliability in the results. The DALEX method, which involved measuring the velocity of a free-flying copper plate, allowed for two calculation methods, doubling the chance of a result being calculated and increasing the reliability of the method. The McDAX method, which involved measuring the particle velocity of a copper plate in a PMMA window, was found to be the most accurate of the methods investigated. The reliability of the DALEX method could be further improved in future work by using a thicker plate to improve its rigidity.

It is hoped that the use of DALEX and McDAX for future cylinder expansion tests will improve the quality of the data captured and therefore the models based upon that data leading to more accurate predictions of explosives behaviour, better validation and comparison of different materials and an improved ability to compare data between different government institutions.

# Table of Contents

Release Conditions	2

Abstract	3

Table of Contents	4

Figures	6

Equations	7

Tables	7

Introduction	8

Background Science	10

Explosive Theory	10

Modelling	12

Mini-Cylinder Expansion Tests	14

DALEX	14

McDAX	15

Method	16

Equipment List	16

Range Set-up	17

Firing Order	18

Instrumentation Set-up	18

Table Set-up	20

Analysis	23

Results	24

Velocity of Detonation	25

Chapman-Jouguet Pressure	32

Analysis	37

Discussion	41

Conclusion	44

References	45

Abbreviations	48

Symbols	49

Appendix	50

Shot Statistics	50

Data Capture Record	51

Tube and Plate Data	52

Stress Sensor Calibration Processs	54

CAD Drawings	55

Stemming Procedure	56

Firing Log	58

Ionisation Pin Results	59

Camera Settings	63

Camera Calibration Results	64

Camera VoD Workings	65

Heterodyne Velocimetry Chapman-Jouguet Pressure Workings	74

Uncertainties Calculation	75

# Figures

Figure 1 Pressures in a detonation	10

Figure 2 Diagram of the Rayleigh and Hugoniot Lines for explosives [12]	11

Figure 3 Trace showing relationship of particle velocity, Von-Neumann spike and Chapman-Jouguet pressure [5]	12

Figure 4 Example Heterodyne Velocimetry trace for a cylinder expansion test wall expansion [19]	13

Figure 5 CylEx test set-up [6]	14

Figure 6 Layout of the firing chamber and pad	17

Figure 7 Photograph of the layout in the chamber and surrounding area	17

Figure 8 Diagram for set-up of stress and strain sensors [33]	19

Figure 9 Layout of the tube and instrumentation for the CylEx firings	20

Figure 10 Photograph of the table set-up for a DALEX firing	21

Figure 11 Layout of the tube and instrumentation for McDAX firings	21

Figure 12 Layout of the tube and instrumentation for DALEX firings	22

Figure 13 X-rays of stemmed tubes showing lack of voids	24

Figure 14 Scatter plot of the ionisation pin results for firing 1	25

Figure 15 Scatter plot of the ionisation pins results for firing 2	26

Figure 16 Scatter plot of the ionisation pins results for firing 4	26

Figure 17 Scatter plot of the ionisation pins results for firing 5	27

Figure 18 Scatter plot of the ionisation pins results for firing 6	27

Figure 19 Scatter plot of the ionisation pins results for firing 7	28

Figure 20 Scatter plot of the ionisation pins results for firing 8	28

Figure 21 Scatter plot of the ionisation pins results for firing 9	29

Figure 22 Scatter plot of the ionisation pins results for firing 10	29

Figure 23 Images of firing 5 from the SIM ultra-high speed camera	30

Figure 24 Single frame from Phantom high speed camera	31

Figure 25 Stress sensor signal for firing 1	32

Figure 26 Stress sensor signal for firing 2	32

Figure 27 Stress sensor signal for firing 3	33

Figure 28 Stress sensor signal for firing 4	33

Figure 29 Screenshot fom HetV Tool for analysis of het-v data	34

Figure 30 Heterodyne Velocimetry results for DALEX firings	34

Figure 31 Heterodyne Velocimetry results for McDAX firings	35

Figure 32 Image taken with the SIM from firing 3	37

Figure 33 Box plot of the results for the different PCJ measurement techniques	39

Figure 34 Box plot of the results for the different VoD measurement techniques	40

Figure 35 Grpahical depiction of the density of the plastic explosive in the tubes	42

Figure 36 Density profiles for the stemmed tubes	42

# Equations

Equation 1 Equation for calculating the PCJ for CylEx tests [8]	9

Equation 2 JWL Equation of State	13

Equation 3 Equation for calculating the Gurney constant for CylEx tests [1]	13

Equation 4 Gurney equation for a cylindrical charge	13

Equation 5 The stress in copper during a DALEX experiment [5]	15

Equation 6 Copper shock state equation [5]	15

Equation 7 Chaos equation for Chapman-Jouguet pressure [19]	15

Equation 8 for finding the Chapman-Jouguet Pressure in McDAX tests [7]	15

Equation 9 Equation to determine the number of firings required for statistically valid data [30]	18

# Tables

Table 1 Examples of explosive material parameters	12

Table 2 Results of confidence level analysis to determine required number of firings	18

Table 3 Firing order devised for the trial	18

Table 4 Results for the pins for firing 1	25

Table 5 Velocity of detonation results from ionisation pin signals	30

Table 6 Velocity of detonation results from the SIM ultra-high speed camera	31

Table 7 Velocity of Detonation results from Phantom high-speed camera	31

Table 8 Summary of results from stress sensor	33

Table 9 Summary of results from DALEX configuration	35

Table 10 Summary of results from the McDAX configuration	36

Table 11 Results of Chapman-Jouguet pressures calculated from Velocity of Detonation formula	36

Table 12 Comparison of Pressure results for the different analysis methods	38

Table 13 Results for Velocity of Detonation from different instrumentation methods	40

# Introduction

Experiments are undertaken to characterise the performance of an explosive material for comparison to other materials, for validation of a new materials and for modelling. The Cylinder Expansion (CylEx) Test, developed in the 1960’s, is a well-established method of finding the parameters required to create an Equation of State (EOS) and is therefore a common precursor to the modelling of explosives. The models can be used to compare explosive performance in different circumstances, determine the moving power of the explosive for metal casings [1] (Gurney Energy) and compare unknown energetic materials to standards such as TNT. The tests are therefore used to inform energetic material choices for a wide range of military and commercial uses in the UK and internationally. The CylEx test has undergone several changes, especially in the instrumentation used, over the years since Los Alamos National Laboratory (LANL), USA published an updated Cylinder Test Specification in 1999 [2], but more work would have to be undertaken to create a more precise, reliable and accurate test standard for the future.

Other tests are available and used to determine explosive material properties, such as the mini-CylEx [3] and the Disc Acceleration Experiment (DAX) [4]. Each of the test’s differences mean they provide slightly different data; the mini-CylEx can be inaccurate for materials with a large critical diameter [3], the DAX tests are done with lower gas volumes which are not directly comparable with the John-Wilkins-Lee (JWL) [5] EOS which is the most commonly used EOS [6]. CylEx, and mini-CylEx, for small critical dimeter explosives, are the most widely used tests in the determination of a material’s properties and have a history of comparable results going back thirty years [2].

The tests measure the Velocity of Detonation (VoD) and the detonation pressure, also known as the Chapman-Jouguet Pressure (PCJ). The data can be used directly or used in numerical analysis models to provide information on a range of scenarios. As the models change parameters until the modelling data fits the experimental data it is necessary to have accurate and precise experimental data. These tests are expensive, especially CylEx, to undertake and can require large quantities, typically, 100’s g, of explosives to be produced. This can be difficult when dealing with materials which can only be made in small quantities, such as experimental or sensitive materials. It is therefore important to acquire as much data from each firing and to as high a standard as reasonably possible to reduce the number of firings required.

The PCJ can be measured directly or derived from other measured quantities such as the VoD. Using the VoD to calculate the PCJ means uncertainties in the VoD measurement are passed onto the PCJ calculation, lowering the precision and repeatability of the result. To measure it directly, a foil stress gauge is placed in a Poly-methyl methacrylate (PMMA) block directly below and in contact with the energetic material in the tube. The stress gauge is made of Manganin foil which has a low thermal response to minimise uncertainties. A constantan foil strain gauge is co-located with the stress gauge allowing the level of strain through the foil gauges to be measured throughout the experiment. If the strain is high, then the stress reading on the Manganin gauge cannot be taken as a true value: the stress gauge reading is susceptible to strain but the relationship has not been well characterised. It is important to line up the gauge to the direction explosive shock front increase the probability that the shockwave impacts linearly onto the stress gauge. If the shockwave is not linear, due to voids, an uneven mix in the material or splitting in the tube, then the stress wave will not be accurate and the PCJ cannot be measured accurately. The material being tested isn’t always a well-mixed homogenous material which may lead to unstable shock fronts, so it is desirable to find an alternative method to measure the PCJ that is less dependent on the shape of the shock front.

Two methods of measuring the PCJ pressure using heterodyne velocimetry (het-v) probes were published in public journals; dubbed here the McDAX and DALEX methods. The McDAX method, designed by Stimac et al [7], is the method utilised by Defence Science Technology Group (DSTG). The DALEX method is a combination of DAX and mini-CylEx developed by Mains et al [5]. A third method is the use of Equation 1 which relates the experimentally determined VoD to the PCJ as published by Cooper [8]. These methods were chosen as there was already access to the necessary testing equipment, the equipment could be easily incorporated into the current test regimes and the author had the ability to discuss the papers with their respective authors.

Equation 1 Equation for calculating the PCJ for CylEx tests [8]

PCJ=ρ0D2(1-0.7125ρ00.04)

The VoD (D) is the velocity at which the reaction zone, immediately behind the shock wave, passes through a material and it is always supersonic for a detonation. The PCJ is the gas pressure of the reactants at the point they start to expand behind the reaction zone. A material’s VoD and PCJ can be found from experimentally recorded measurements, and the PCJ can be calculated from the VoD using Equation 1, where VoD is represented by D and ρ0 is the density of the explosive material.

As well as the PCJ, the velocity of detonation needed to be measured in the experiments, both to provide a full data set for modelling and to provide an alternative method for calculating the PCJ. The methods available for measuring VoD are high speed cameras, chirped fibre bragg gratings (CFBGs) or time of arrival devices such as ionisation pins, optical fibre, het-v, printed circuit boards (speed strips) and piezo pins. Ionisation pins were used for measuring the velocity of detonation over other methods due to three main reasons. Pins and the equipment required to run them were readily available, unlike CFBGs or speed strips. Ionisation pins and their cabling have a much lower relative cost compared to piezo pins or heterodyne velocimetry probes and pins can achieve a significantly higher precision than ultra-high speed video or streak cameras. Ultra-high speed cameras were used as a back-up for the ionisation pins and would be the ideal method if they could match pins for precision as they are a non-invasive method with no cost per firing after the initial outlay for the camera.

In order to undertake experiments to test the different methods, a trials plan [9] and a trials risk assessment [10] were written by the author and reviewed by peers and safety representatives. The trials plan outlines the scope, objectives and methods of the experiment as well as range specific information on the safety processes of the range at which the experiments were undertaken. The risk assessment covers risks from the energetics and instrumentation as well as generic hazards, such as the weather or slips, trips and falls.

# Background Science

## Explosive Theory

Given enough energy, a material can detonate explosively to produce very large amounts of energy and large volumes of gas in very short timescales. This energy can be used to move large amounts of material or shatter and violently burst apart nearby material. Explosive materials are used around the world and each use requires different combinations of characteristics from the explosive material such as different levels of static peak overpressure, positive impulse and sensitivity.

The static peak overpressure is caused by the gases created in the reaction zone of a detonation, just behind the shockwave, rapidly expanding outwards. The gas formed from a solid or liquid material occupies a greater volume at the same temperature. The bonds between molecules are broken and the chemical energy stored in the bonds is transformed into kinetic energy which allows the molecules to do work on the environment and so surrounding cases expand. This expansion produces a pressure on the surrounding medium, which for these experiments was a steel or copper tube. With enough pressure, this can cause the expansion and then fracture of the tube material. The static peak overpressure created by the gases depends on several factors including the rate of formation, the amount of gas produced per unit mass of the products and the ambient temperature. The relative explosive power of an explosive is directly proportional to the volume of gas produced per unit of initial explosive mass.

The positive impulse is the area under the positive part of the overpressure curve (see Figure 1) and is directly related to the brisance, the pushing force of the explosive material. A higher static peak overpressure does not necessarily mean a higher impulse. A lower static peak overpressure, but high positive impulse, can occur when the gaseous expansion happens over a longer time period, such as in Ammonium-Nitrate Fuel Oil (ANFO), or when the return to ambient pressure occurs more slowly [11].

Figure 1 Pressures in a detonation

The positive impulse of an explosive can also be increased with the addition of secondary burning, to create a dynamic pressure, which occurs after the initial detonation reaction and produces more gas products. For example, Tritonal, is a mixture of TNT and aluminium, the aluminium reacts after the initial detonation to increase the total gas volume produced without increasing the static peak overpressure. As in Figure 1, with the addition of dynamic pressure, the area under the positive part of the curve is increased corresponding to a larger positive impulse.

The gas produced is assumed to follow the ideal gas laws as in the simplified Rankine-Hugoniot relationship equations which describe the reactions on both sides of the shock front as a detonating material [1]. These equations produce Rayleigh lines and Hugoniot curves which describe the behaviour of the explosive during detonation. The Rayleigh line is the straight line in Figure 2 which connects the initial and final states of the material’s pressure against specific volume when under shock. The Hugoniot curves in Figure 2 describe the possible states of the material detonating and its products, determined by the solutions of the Rankine-Hugoniot equations.

Reaction zone

Figure 2 Diagram of the Rayleigh and Hugoniot Lines for explosives [12]

The Chapman-Jouguet (C-J) point, also known as the detonation pressure, is the point where the Rayleigh and Hugoniot lines cross in Figure 2. It corresponds to detonation and the Chapman-Jouguet pressure which is lower than the initial static peak overpressure [12] (as shown in Figure 3), and is the pressure applied to a material, such as the casing, at the front of the shock wave. It is also the only point on the Hugoniot curve at which a steady state detonation can occur, further down the Hugoniot curve detonation reduces to deflagration. Higher up the curve, the detonation chokes itself and is extinguished.

The shock front compresses the explosive material from (P0,v0) through the C-J point up to (P1,v1), causing deflagrating material to detonate behind the shock front and to reach a steady state detonation down through the material. As the material becomes increasingly compressed the reaction is dampened and detonation is reduced to deflagration.

Time

Pressure

Figure 3 Trace showing relationship of particle velocity, Von-Neumann spike and Chapman-Jouguet pressure [5]

## Modelling

To model an explosive in a numerical computer program such as CTH [13], certain parameters including VoD and PCJ, are needed to build an Equation of State (EOS). Each explosive has a value for its VoD and PCJ at a particular density, some examples of which are in Table 1 alongside the relative explosive power (RE) which is an indication of gas production.

Table 1 Examples of explosive material parameters

| Material | Density ρ0 (g/cm3) | RE factor | VoD (m/s) | CJ pressure (GPa) |
| --- | --- | --- | --- | --- |
| TNT [14] | 1.63 | 1.00 | 6,930 | 21.0 |
| C4 [15] | 1.73 | 1.2 | 8,092 | 26.0 |
| PE4 [16] | 1.59 | Not Available | 8,027 | 24.7 |
| RDX [17] | 1.78 | 1.6 | 8,700 | 30.0 |

As PE8 is an RDX based explosive (88% RDX [18]) developed by Chemring, no published data on its PCJ or RE values were available but we can predict that it will have similar but slightly lower RE, VoD and PCJ values than RDX. As it is has a higher RDX content than PE4 it is also predicted that it will have higher values than PE4, C4 and TNT at a similar density.

An Equation of State describes how a material will react in certain conditions, such as with the same pressure but varying temperature. A commonly used Equation of State for energetic materials is the Jones-Wilkins-Lee (JWL) equation, Equation 2, which describes the expansion of the products, during a detonation, in the pressure-volume domain.

Equation 2 JWL Equation of State

p=A1-ωR1Ve-R1V+B1-ωR2Ve-R2V+ωe0V

To find a solution to the JWL EOS, five coefficients have to be identified; A, B, R1, R2 and ω as well as the velocity of detonation (V). Equation of State parameters are determined by iteratively running a numerical model, in programs such as CTH, until the model matches the experimental data in certain areas, such as wall expansion. The wall expansion data from the heterodyne velocity in the experiment will produce a graph similar to Figure 4 below. The numerical model can be run with successive small changes to the undetermined parameters until the model’s wall expansion traces and the experimental data align.

Figure 4 Example Heterodyne Velocimetry trace for a cylinder expansion test wall expansion [19]

The Gurney constant also known as the Gurney energy or Gurney velocity (EG) is directly proportional to the VoD (D) as can be seen in Equation 3. The Gurney constant is an indication of the energy put into the surrounding material, casing, of an explosive by the detonating products and is vital for modelling cased structures such as warheads.

Equation 3 Equation for calculating the Gurney constant for CylEx tests [1]

2EG=D2.97

A higher Gurney constant indicates a larger throwing capacity. A typical value of Gurney constant for military explosives is 2,400 – 2,800ms-1 [20] [21]. The Gurney equation for a cylindrical charge (Equation 4) can be used to estimate the wall expansion that should be seen during a cylinder expansion based experiment, where V is the velocity of the casing, M is the mass of the casing and C is the mass of the explosive charge.

Equation 4 Gurney equation for a cylindrical charge

V2EG=MC+12-0.5

As other factors (Gurney constant, PCJ) can be determined from the Velocity of Detonation, it is vital that the measurement of VoD is as precise as possible to reduce incorporating large uncertainties. The method/s used to measure VoD and wall expansion should also be as repeatable as possible otherwise there is added uncertainty on which result to align the model data to. For instance, if there are three heterodyne velocimetry traces which are significantly different, the modeller and therefore model will not be able to determine which one to align the model to and will have to produce three results to determine the uncertainty in each calculated parameter.

## Mini-Cylinder Expansion Tests

A mini-CylEx test consists of an annealed C101 half hard copper tube, with a wall thickness one tenth of the tube diameter and a length of 12 times the diameter, filled with an explosive material. The diameter of the tube is determined by the critical diameter of the explosive. For modern military or industrial grade explosives, this diameter can be small, less than 30mm, allowing for small overall tube dimensions and the development of a “mini-CylEx” [3].

When the explosive fill is detonated, the copper tube begins to expand in a cone-like shape, with the widening section at the detonator end, as can be seen in Figure 5.

Figure 5 CylEx test set-up [6]

The copper tube is filled with several hundreds of grams of the explosives to be tested (dependant on the size of the tube and density of the explosive material) and its behaviour through the test is measured using several instrumentation techniques. Heterodyne velocimetry (het-v) is used to measure the wall expansion, as in Figure 4. The pins (optical fibre, piezo or ionisation) measure the velocity of detonation. High speed cameras can also be used to measure the VoD and wall expansion, but at a lower precision.

## DALEX

This method combines elements of the Disc Acceleration Experiment (DAX) [4] [19] and mini-CylEx experiments into a new test method. It was claimed that this method reduced uncertainties found in the two different experiments to create one more precise method [5]. A copper tube is used; its wall expansion and the detonation velocity of the explosive fill are measured as in CylEx and mini-CylEx experiments. A copper flyer plate is secured to the bottom of the tube (the far end from the detonator) and het-v is used to measure the velocity of the flyer plate once the shock wave reaches it. The copper plate reverberates as the shock wave hits it and the het-v is used to measure the velocity of these reverberations. The value for P can be calculated for each reverberation in the flyer plate giving a series of pressures for each experiment. The first result is associated with the Von-Neumann spike, the peak incident overpressure, and the later ones with the adiabatic expansion of the explosive gas. The PCJ can be found by using the data from the 2nd and 3rd reverberations visible in the het-v traces for the wall expansion and the flyer plate velocity. This relationship is shown in Figure 3.

The pressure in the copper can be calculated from Equation 5 where P is the stress in the copper, pc is the density of the copper, Us is the shock wave speed and Up is the particle speed.

Equation 5 The stress in copper during a DALEX experiment [5]

P= pcUsUp

The particle speed within the copper, UP, can be found using Equation 6.

Equation 6 Copper shock state equation [5]

Us=1.489*Up+3.94

The even number states (shown as pushes in Figure 4) for the velocities are measured directly from the velocities on the flyer plate measured by het-v. The states are the particle velocities seen as reverberations in the flyer plates or copper walls caused by the shock passing into the metal and reflecting off of the internal surfaces. The PCJ falls between states 3 – 4 and so the average of the calculated pressures at states 3 and 4 can be used to estimate the PCJ.

The Chaos formula (Equation 7) for DAX [19] can also be used with the DALEX set-up to provide a second method for determining PCJ.

Equation 7 Chaos equation for Chapman-Jouguet pressure [19]

PCJ=ρ0D2γ+1

Where PCJ is the Chapman-Jouguet pressure, Po is the initial density of the explosive, D is the VoD and γ is a factor based on the ratio of container density to explosive density which can be calculated separately.

## McDAX

The McDAX Experiment designed by Stimac et al [7] is also utilised by DSTG [21] in their efforts to find an alternative method to calculate PCJ without a Manganin gauge [20]. In this experimental set-up a reflective surface is held in a PMMA window at the base of the detonating tube. The shock from the explosive passes through the window and reflective layer. The particle velocity of the reflective layer can be measured with het-v to produce a velocity-time trace. The particle velocity equates to the Von-Neumann spike of pressure and the later time traces relate to the PCJ as can be seen in Figure 3; the PCJ is found between states three and four as with DALEX. The PCJ pressure can be calculated with Equation 8 where UP is the particle velocity, ρc is the density of the copper flyer plate and UW is the velocity of the plate.

Equation 8 for finding the Chapman-Jouguet Pressure in McDAX tests [7]

PCJ= UPρcUw

In this case the particle velocity is determined as being half of the measured flyer plate velocity between states three and four.

# Method

## Equipment List

- Energetics

  - 3kg Plastic explosive (PE8)

  - 15 High voltage detonators (RP80)

- Firing Table (1.5m x 600mm x 600mm)

- 2 Mirrors & Mirror stands

- Consumables

  - 9 Copper C101 hardened tubes (25mm ID, 30mm OD, 300mm length)

  - 10 MDF Table tops (600mm x 600mm x 10mm)

  - 10 white painted MDF backgrounds (600mm x 600mm x 2mm)

  - 60 Magnesium flash bulbs and holders (Meggaflash PF300) [22]

  - 210 ionisation pins (Hi-5)

  - 30 fibre collimated probes (Acal BFI 1CL15P020LCG01-CPT2) [23]

  - 6 CylEx charge stands (See Appendix)

  - 3 DALEX charge stands (See Appendix)

  - 10 top hats (See Appendix)

  - 3 copper flyer plates (30mm diam, 0.5um thick)

  - 6 perspex blocks (consist of two blocks; the first 100mm x 100mm x 10mm and the second 100mmx 100m x 15mm)

  - 3 combined manganin stress and constantan strain gauges (Model Mn/Cn4-50-EK) [24]

- Instrumentation

  - 1 SIM 16 ultra-high speed framing camera

  - 1 Phantom v2512 high speed camera

  - 1 HBM Genesis 7t data acquisition unit

  - 1 High bandwidth oscilloscope (Agilent INFINIUM DSO-X93204A)

  - 2 Dynasen pin mixer

  - 1 Pulse power system

  - 4 Dell laptops

  - Multi-channel heterodyne velocimetry laser system

- Software

  - FAMOS 6 [25]

  - MATLAB 2019a [26]

  - Microsoft Excel 2016

  - PCC 3.4 [27]

  - Perception 6.4 [28]

  - SIM 2 [29]

## Range Set-up

The arena set-up is shown in Figure 6 and Figure 7 with the firing table holding the charge placed in the centre of a firing chamber. This set-up was the same for each of the different experimental configurations. The instrumentation demount, which has two windows and two cable ports, was placed to the side of the chamber opening to protect it from fragmentation. It held the majority of the instrumentation equipment including the Heterodyne Velocimetry (het-v) equipment and cameras. The remainder of the equipment, such as the data recorder and pulse power supply, were placed in the instrumentation control room. Long fibre optic cables and single core shielded (BNC) data cables were run from the instrumentation demount, or instrumentation control room, to the charge on the firing table. Mirrors were used to align the camera and ultra-high speed camera’s fields of view to the charge on the table.

Firing Chamber Walls

Instrumentation control room

Firing Table

Mirrors for cameras

Instrumentation Demount

Figure 6 Layout of the firing chamber and pad

Instrumentation demount

Window ports for cameras

Mirrors on stands

Firing table

Figure 7 Photograph of the layout in the chamber and surrounding area

## Firing Order

Three firings were undertaken for each type of experiment, this number was decided upon using the below formula and a 95% certainty and a 5% confidence limit.

Equation 9 Equation to determine the number of firings required for statistically valid data [30]

sample size= tvaluefor confidence of 95(standard deviation0.05margin of errormean value2

Due to the small expected sample size, t-values were used in place of the usual z-value. A mean of 8,000ms-1 and a standard deviation of 500ms-1 for the velocity of detonation was assumed based on previous experiments [31] [13]. A mean of 30GPa and a standard deviation of 1GPa was assumed for the Chapman-Jouguet pressure (PCJ) based upon results from previous experiments [32]. The results of the equation were rounded up to the nearest whole number and the largest of the values from the different measurements was used, these can be seen in Table 2.

Table 2 Results of confidence level analysis to determine required number of firings

|  | Velocity of Detonation (ms-1) | Chapman-Jouguet Pressure (GPa) |
| --- | --- | --- |
| Mean | 8000 | 30 |
| Standard Deviation | 500 | 1 |
| Certainty | 95 | 95 |
| Confidence Level | 5 | 5 |
| Number of firings Required | 3 | 3 |

A steel tube was used for the first shot to ensure all the instrumentation was working before moving on to the expensive copper tubes for the remainder of the trial.

Table 3 Firing order devised for the trial

| Shot Type | Explosive | Tube Material | No of Firings |
| --- | --- | --- | --- |
| Validation Tube | PE8 | Steel | 1 |
| Mini-CylEx | PE8 | Copper | 3 |
| McDAX | PE8 | Copper | 3 |
| DALEX | PE8 | Copper | 3 |

## Instrumentation Set-up

To ensure all the required information was captured prior to the firings, a data capture record was designed and used. It can be found in the Appendix: Data Capture Record. The data for the tube masses, explosive masses and flyer plate masses (where used) can be seen in the Appendix.

Time of Arrival Devices

The Velocity of Detonation was measured by placing two rows of ionisation pins at 1mm from the surface of the tube and at set distances from one another (nominally 10mm) down the length of the bottom two-thirds of the tube. The pins were not used in the top third to allow time for the explosive to reach a steady state and have an approximately linear wave front. The distances between the pins and the tube and between each of the pins were measured just before the firing using an electronic micrometre with a 0.01mm precision. As the shock front passed down the tube the tube expanded and made contact with the pins completing the internal circuit and sending a spike of up to 100V (powered by the Dynasen pin mixer) back to the HBM Genesis 7t data recorder. Each pin was recorded on a separate channel of the data recorder at a sample rate of 100MS/s for 0.5s

Heterodyne Velocimetry

The heterodyne velocimetry was done using a system built in house by the author and Imperial College. A class 2, 1550nm laser with less than 10mW per channel was used to measure the velocity of the tube wall as it expanded and the velocity of the flyer plate, where one was used. The signal was recorded on an Agilent INFINIUM DSO-X93204A oscilloscope at a sample rate of 80GS/s. All the signals were recorded on a single channel using a time delay circuit to delay each signal by 20µs.

Stress Gauge

A Dynasen, combined Manganin stress and constantan strain gauge (Model Mn/Cn4-50-EK) was used inside of a PMMA block. Air bubbles were reduced by placing silicon grease over the gauge inside the block. The gauge was powered by a Dynasen pulse power unit with a voltage of 100V for 500µs and the output was recorded by the HBM Genesis 7t data recorder, with each sensor on a separate channel, at a sample rate of 100MS/s for 0.5s. The layout of the circuit for the set-up of the gauge can be seen in Figure 8.

Figure 8 Diagram for set-up of stress and strain sensors [33]

Each stress gauge was calibrated prior to firing by introducing a calibrated resistance into the circuit and measuring the voltage signal. The instructions for the calibration can be seen in the Appendix: Stress Sensor Calibration Processs.

High Speed Camera

A Specialised Imaging SIM 16D ultra-high speed camera and a Phantom v2512 monochrome high speed camera were used to observe the event. These provided visual confirmation of tube rupture which would affect the results and provided a backup method for measurement of VoD and derivation of PCJ. The SIM 16D was recording the event through a 400mm lens, with a resolution of 1300x1300px at a sample rate of 654,161 frames per second and an exposure of 300ns. The Phantom v2512 used a 400mm lens and recorded the event with a resolution of 128 x 64px at a frame rate of 1 million frames per second and an exposure of 261ns.

## Table Set-up

The firing table is a steel frame, with a 10mm deep wooden top plate which was replaced after each firing, and the set-up of the table can be seen in Figure 9 and Figure 10. The differences in the table set-up for the three different experimental configurations are shown in Figure 9, Figure 11 and Figure 12.

The charge was held in an ABS 3D-printed stand, which was secured to the table top with tape. This stand also held the time of arrival devices (TOADS) for measuring the detonation velocity and het-v collimators for measuring the wall expansion of the charge. A 2mm hole was drilled through the bottom of the table tops and a het-v collimating probe glued in place for the firings where it was required to measure the plate velocity.

detonator

Ionisation pins & het-v probes

Stress & Strain Gauges

PMMA block

Table top

Copper tube stemmed with plastic explosive

Figure 9 Layout of the tube and instrumentation for the CylEx firings

Additional lighting was supplied by flash bulbs which front-lit the tube. A white background was placed on the table behind the charge to capture the majority of the light from the flash bulbs and redirect it back to the cameras as well as provide a clean background for measurements.

Figure 10 Photograph of the table set-up for a DALEX firing

A “Top Hat” was 3D printed in ABS to hold the detonator centrally on the top of the tube for each firing. It was taped to the top of the tube after the explosive was stemmed, as can be seen in Figure 10. The tubes were stemmed by the Trials Conducting Officer following procedures which can be found in the Appendix: Stemming Procedure.

CylEx Charge

Figure 11 Layout of the tube and instrumentation for McDAX firings

Figure 12 Layout of the tube and instrumentation for DALEX firings

## Analysis

Velocity of Detonation

The Velocity of Detonation (VoD) was calculated from the TOADs; the position against time of each pin and its reaching a signal of 5V was plotted on a scatter graph. Microsoft Excel was used to draw a linear line of best fit through the data and the gradient of that line was taken to be the VoD.

Heterodyne Velocimetry

The files from the oscilloscope were converted to .csv files and run through a bespoke HetV Tool MATLAB program to generate the spectrogram from a Fourier transform. The program picked out the values from within a user-defined area of interest to create a new .csv file of time and velocity values.

Stress Gauge

The Manganin Stress Gauge was analysed using the following procedure written by the Instrumentation Team [34]:

- The data Y value should initially be somewhere around zero at a point in time just after the PPS was triggered.

  - If not, something is wrong, check set up procedure, pay special attention to triggering

- Work your way along the data from PPS start time, you should find a feature which looks like a crook

  - The maxima is your stress wave arriving and then decaying, the subsequent rise is the gauge distorting and ultimately breaking

- The point of the maxima is the voltage you need to record to calculate the pressure. If the zero level is significantly different to that measured during the calibration phase, it is probably worth recording this also

- In the '4th Order' spreadsheet, enter the voltage measured at point A (in Volts) in cell C25

- Enter the zero value in cell C24

- Pressure values are calculated for both Vishay and Dynasen manufacturers’ data, in bars and Pascal

High Speed Video

The high speed video was used as a backup measurement for VoD and PCJ should any of the main instrumentation have failed and to visually inspect the tubes for premature break up during the firings.

For VoD, the imagery was calibrated prior to the firing using a checkerboard of 0.05m x 0.05m squares to provide an mm/pixel value. This was then used to measure a displacement of the shockwave between frames which was plotted against time. A linear line of best fit was created by Microsoft Excel and the gradient of the line used for the VoD.

# Results

The trial was conducted and ten firings were undertaken; one with a steel tube, then three each of the mini-CylEx, the DALEX and the McDAX configurations. No data was collected for the ionisation pins for firing three. Some of the tubes were x-rayed prior to detonation to confirm that the stemming procedure wasn’t leaving significant voids which would affect the results, the results can be seen below in Figure 13.

Figure 13 X-rays of stemmed tubes showing lack of voids

The copper tube x-rays have a resolution of 0.6mm, any voids smaller than this would not be visible using the equipment available.

## Velocity of Detonation

Ionisation Pins

The time and displacement for each row of pins on each firing was calculated as below, with A (pink) being the left-hand side, as seen by the cameras and B (blue) being the right-hand side. The full table of results for the other nine firings can be seen in the Appendix.

Table 4 Results for the pins for firing 1

| Pin | Measured Distance (m) | Cumulative Displacement (m) | Measured Time (ms) | Cumulative Time (s) |
| --- | --- | --- | --- | --- |
| A1 | 0 | 0 | 0.31548 | 0.0000000 |
| A2 | 8.4 | 0.0095 | 0.31708 | 0.0000016 |
| A3 | 8.74 | 0.01934 | 0.31851 | 0.0000030 |
| A4 | 9.14 | 0.02958 | 0.32096 | 0.0000055 |
| A5 | 8.42 | 0.0391 | 0.32183 | 0.0000064 |
| A6 | 9.53 | 0.04973 | 0.323372 | 0.0000079 |
| A7 | 10.69 | 0.06152 | 0.32655 | 0.0000111 |
| A8 | 8.23 | 0.07085 | 0.32777 | 0.0000123 |
| B1 | 0 | 0 | 0.72021 | 0.0000000 |
| B2 | 8.86 | 0.00996 | 0.72109 | 0.0000009 |
| B3 | 9.51 | 0.02057 | 0.72306 | 0.0000029 |
| B4 | 9.04 | 0.03071 | 0.72401 | 0.0000038 |
| B5 | 9.84 | 0.04165 | 0.7256 | 0.0000054 |
| B6 | 8.52 | 0.05127 | 0.7268 | 0.0000066 |
| B7 | 8.04 | 0.06041 | 0.72823 | 0.0000080 |
| B8 | 8.48 | 0.06999 | 0.7303 | 0.0000101 |

The results were plotted in Microsoft Excel using the scatter graph function and a linear line of best fit, the resulting graphs can be seen in Figure 14 to Figure 22 and a summary of the calculated VoDs can be seen in Table 5.

Figure 14 Scatter plot of the ionisation pin results for firing 1

Figure 15 Scatter plot of the ionisation pins results for firing 2

Figure 16 Scatter plot of the ionisation pins results for firing 4

Figure 17 Scatter plot of the ionisation pins results for firing 5

Figure 18 Scatter plot of the ionisation pins results for firing 6

Figure 19 Scatter plot of the ionisation pins results for firing 7

Figure 20 Scatter plot of the ionisation pins results for firing 8

Figure 21 Scatter plot of the ionisation pins results for firing 9

Figure 22 Scatter plot of the ionisation pins results for firing 10

Table 5 Velocity of detonation results from ionisation pin signals

| Firing | Line A VoD (ms-1) | Line B VoD (ms-1) | Average VoD (ms-1) |
| --- | --- | --- | --- |
| 1 | 5810.1 | 7686.8 | 6749 |
| 2 | 7900.5 | 8739.3 | 8320 |
| 4 | 7428.3 | 8101.5 | 7765 |
| 5 | 7709.5 | 7825.1 | 7767 |
| 6 | 8163.3 | 8119.9 | 8142 |
| 7 | 7488.5 | 8846.4 | 8168 |
| 8 | 7103.7 | 8140.4 | 7622 |
| 9 | 9853.7 | 6363.9 | 8109 |
| 10 | 10988.0 | 7262.7 | 9125 |

High Speed Cameras

With the SIM ultra-high speed framing camera, both sides of the tube could be seen, as shown in Figure 23, allowing for two measurements of the VoD (one either side of the tube). For the Phantom high-speed camera, only one half of the tube could be seen due to the lower resolution. Frames ten and sixteen of the SIM images were consistently overexposed due to an internal calibration error.

Point where tube starts to expand, which corresponds to the detonation wavefront, is mapped over time to find VoD

16

13

14

15

11

9

12

5

6

7

8

4

3

2

1

Figure 23 Images of firing 5 from the SIM ultra-high speed camera

Table 6 Velocity of detonation results from the SIM ultra-high speed camera

| Firing | Side A VoD (ms-1) | Side B VoD (ms-1) | Average VoD (ms-1) |
| --- | --- | --- | --- |
| 4 | 9212 | 8420 | 8816 |
| 5 | 11113 | 9736 | 10425 |
| 6 | 7518 | 8023 | 7771 |
| 7 | 6827 | 6989 | 6908 |
| 9 | 8973 |  | 8973 |
| 10 | 8179 | 8341 | 8260 |

Point where tube starts to expand, which corresponds to the detonation wavefront, is mapped over time to find VoD

Figure 24 Single frame from Phantom high speed camera

Table 7 Velocity of Detonation results from Phantom high-speed camera

| Firing | Velocity of Detonation (ms-1) |
| --- | --- |
| 1 | 8001 |
| 3 | 5906 |
| 4 | 5740 |
| 5 | 11214 |
| 7 | 7593 |
| 9 | 8207 |
| 10 | 8678 |

## Chapman-Jouguet Pressure

Stress Gauge

For the standard cylinder expansion firings, the peak voltage of the stress curve was identified using FAMOS and multiplied by the calibration factor to acquire the pressure result. The results of the stress (pink) and strain (blue) sensors can be seen in Figure 25 to Figure 28 and are summarised in Table 8.

Figure 25 Stress sensor signal for firing 1

Figure 26 Stress sensor signal for firing 2

Figure 27 Stress sensor signal for firing 3

Figure 28 Stress sensor signal for firing 4

Table 8 Summary of results from stress sensor

| Firing | Voltage (V) | Calibration Factor | Result (GPa) |
| --- | --- | --- | --- |
| 1 | N/A | N/A | N/A |
| 2 | 10.9 | 2.2004 | 23.9 |
| 3 | 9.9 | 2.0700 | 20.6 |
| 4 | 12.2 | 2.5911 | 31.7 |

DALEX

As the DALEX configuration used a free-flying plate, two equations (Equation 6 and Equation 7) were used to calculate the pressure. First, the trace was identified by using the MATLAB HetV Tool which generated a spectrogram of the voltage-time data for each het-v probe, such as in Figure 29.

pushes

Base line

Coast/final velocity

Figure 29 Screenshot fom HetV Tool for analysis of het-v data

The signal for each probe was outlined by an area of interest and the program generated a velocity-time curve such as those in Figure 30. The curve in the HetV Tool was inverted due to the configuration of the two lasers used in the het-v equipment, this was corrected before the velocity-time data was plotted.

Figure 30 Heterodyne Velocimetry results for DALEX firings

To calculate the PCJ with the Hugoniot formula, the pressures were calculated for the shock wave speed and particle speed in states three and four using Equation 5 and Equation 6. These two values were then averaged to produce the results in Table 9. The PCJ was also calculated using the Chaos formula (Equation 7) for the DAX experiments which used the densities of the explosive, the dimensions of the experiment and the terminal velocity of the flyer plate. The full workings for each method can be seen in Appendix: Heterodyne Velocimetry Chapman-Jouguet Pressure Workings, and the final results for each firing were also placed in Table 9 below.

Table 9 Summary of results from DALEX configuration

| Firing | Chaos Formula (GPa) | Hugoniot Formula (GPa) |
| --- | --- | --- |
| 5 | 29.9 | 27.7 |
| 6 | 27.7 | 35.8 |

McDAX

First the trace was identified by using the MATLAB HetV Tool, and a spectrogram of the voltage-time data for each het-v probe was generated, such as in Figure 29. The trace for each probe was outlined by an area of interest and the program generated a velocity-time curve, which was inverted, such as those below in Figure 31.

Figure 31 Heterodyne Velocimetry results for McDAX firings

Only the one equation was used to calculate the pressure for the McDAX method. The flyer plate velocities for each state were measured and then the average of states three and four and the corresponding wall velocity were placed into Equation 8. The full workings for the Hugoniot method can be seen in Appendix: Heterodyne Velocimetry Chapman-Jouguet Pressure Workings, and the final results for each firing are in Table 10 below.

Table 10 Summary of results from the McDAX configuration

| Firing | Hugoniot Formula (GPa) |
| --- | --- |
| 8 | 28.2 |
| 9 | 25.1 |
| 10 | 37.3 |

Formula

The results from the different methods of measuring the Velocity of Detonation were used in Equation 1 to calculate the Chapman-Jouguet Pressure.

The Velocity of Detonation for each method of each firing was married up to the density of the energetic stemmed into the tube for that firing. Both numbers were fed into Equation 1 to calculate a series of PCJ values which are in Table 11 below.

Table 11 Results of Chapman-Jouguet pressures calculated from Velocity of Detonation formula

| Firing | Pins A Side (GPa) | Pins B Side (GPa) | SIM A Side (GPa) | SIM B Side (GPa) | Phantom B Side (GPa) |
| --- | --- | --- | --- | --- | --- |
| 1 | 21.03 | 20.74 |  |  | 21.7 |
| 2 | 28.05 | 37.54 |  |  |  |
| 3 |  |  |  |  | 15.7 |
| 4 | 26.15 | 20.23 | 36.35 | 30.37 | 14.1 |
| 5 | 25.23 | 19.27 | 54.99 | 42.20 |  |
| 6 | 27.04 | 23.16 | 24.90 | 28.36 |  |
| 7 | 21.14 | 31.18 | 20.69 | 21.69 |  |
| 8 | 26.56 |  |  |  |  |
| 9 | 23.50 | 26.72 | 34.82 |  | 29.1 |
| 10 | 30.50 | 25.20 | 29.33 | 30.51 | 33.0 |

# Analysis

For the first firing, with the steel tube, there was a significant signal on the strain sensor at the same time as the signal for the stress sensor. This means there was strain present in the stress sensor and that the stress signal cannot be used. For the firings in the copper tubes, the strain sensor showed no significant strain until after the stress signal had occurred. The values from those three readings were significantly different leading to significantly different pressures from each other and the expected pressure from previous work.

This might be due to a misalignment from the tube to the gauge, or that the detonation wave down the tube was not planar. Looking at the wall expansion on the high speed video, it is possible to see that one side of the tube started to move before the other and therefore it is unlikely that the wave front was planar as required. This can be seen in Figure 32 below where the bright section (shock front) is not perpendicular to the tube

Figure 32 Image taken with the SIM from firing 3

A summary of the results for the PCJ can be seen below in Table 12 and a summary of the results for VoD can be seen in Table 13. The uncertainty calculations based on the experimental set-up can be seen in the Appendix: Uncertainties Calculation.

Table 12 Comparison of Pressure results for the different analysis methods

| Firing | Stress Gauge (GPa) | DALEX (GPa) | McDAX (GPa) | Pins (Gpa) | SIM (GPa) | Phantom (GPa) | Average (GPa) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N/A |  |  | 15.47 |  | 21.7 | 18.6 |
| 2 | 23.9 |  |  | 33.03 |  |  | 28.5 |
| 3 | 20.6 |  |  |  |  | 15.7 | 18.2 |
| 4 | 31.7 |  |  | 25.82 | 33.36 | 14.1 | 26.2 |
| 5 |  | 27.7 |  | 26.86 | 48.59 |  | 34.4 |
| 6 |  | 35.8 |  | 29.20 | 26.63 |  | 30.5 |
| 7 |  |  |  | 29.62 | 25.14 |  | 27.4 |
| 8 |  |  | 28.2 | 25.20 |  |  | 26.7 |
| 9 |  |  | 25.1 | 28.44 | 34.82 | 29.1 | 29.4 |
| 10 |  |  | 37.3 | 36.52 | 29.92 | 33.0 | 34.2 |
| Average (GPa) | 25.4 | 31.8 | 30.2 | 27.8 | 22.7 | 32.7 |  |
| Standard Deviation (GPa) | 5.7 | 5.7 | 6.3 | 5.8 | 8.5 | 8.2 |  |
| Coefficient of Variance (%) | 22 | 18 | 21 | 21 | 37 | 25 |  |

As there is no published and verified data on the PCJ, it isn’t possible to compare our results with a known value to judge the accuracy of the different methods, however as it is an RDX based explosive [18] it can be assumed that the PCJ would be slightly lower than 30GPa which is in line with previous work [13] [32] [31]. This implies that the formula using the ionisation pin data provides the most accurate result but this would have to be confirmed using a better characterised explosive.

The stress gauge, formula with pins, McDAX and DALEX were each equally reliable with only one piece of data being missed each with the DALEX, pins and stress gauge. The flyer plate velocity to analyse firing seven using the DALEX Hugoniot equation was missed, possibly due to the plate warping or fracturing. The high speed cameras were the least reliable method for determining the VoD due to being susceptible to misalignment to the shot, over exposure of pixels from the light produced by the explosion or problems with the flash bulbs which were providing the illumination of the front face of the tube.

The formula using SIM data had the highest coefficient of variance suggesting it is the least repeatable option and the DALEX method had the lowest implying it might be the more repeatable method for measuring the PCJ. Both the DALEX and McDAX methods have similar precision as they are both based on the highly precise het-v methods of measurement compared to the other methods which rely on a user calibration, which introduces uncertainties, and slower temporal resolution equipment. The differences in the data’s repeatability and precision can be seen below in Figure 33 which is a box plot of the results; the box demarks the interquartile range, the bars show the uncertainty in the measurement of the mean which is depicted by the x inside the boxes. The average PCJ for the technique using the phantom camera data was significantly different to the other techniques (Figure 33) and should therefore be discarded for further analysis of the results.

Figure 33 Box plot of the results for the different PCJ measurement techniques

The high speed cameras required no additional cost, beyond their initial purchase to perform the measurements, they did however require the use of up to six flash bulbs per firing at a cost of £7 per bulb and holder. The McDAX and stress gauges required the manufacture of blocks which were £100 each. The Manganin Constantan stress-strain gauges were about £400 each but the foils for DALEX and McDAX were less than 1p each. The strain gauges took time ahead of the trials to prepare, about two hours each and then a further hour on the range to calibrate. Further, the small wiring for the electrical circuits was susceptible to damage in the move, which led to loss of data. In comparison the high speed cameras and DALEX methods required no pre-trial preparation and no extra set-up time between firings. The McDAX method required some pre-trial set-up time but no extra time between firings. In doing a cost benefit analysis of the different methods, the DALEX method is the best option for financial cost, time on range and reliability.

As expected, considering their much higher precision, the ionisation pin data had the lowest coefficient of variance for the different measurement techniques suggesting it is the most repeatable option and the cameras were the least repeatable and precise method to measure the PCJ. The cost per firing for the camera methods was zero meaning after the initial purchase of the camera and ancillary equipment there is no extra cost unlike the ionisation pins which were £15 per pin. Optical fibre probes can be procured more cheaply at about 50p per probe but they require more set-up time and more complex analysis due to the different way in which they react to the energetic event.

Table 13 Results for Velocity of Detonation from different instrumentation methods

| Firing | Pins (ms-1) | SIM (ms-1) | Phantom (ms-1) | Average (ms-1) |
| --- | --- | --- | --- | --- |
| 1 | 6748 |  | 8001 | 7375 |
| 2 | 8320 |  |  | 8320 |
| 3 |  |  | 5906 | 5906 |
| 4 | 7765 | 8816 | 5740 | 7440 |
| 5 | 7767 | 10425 | 11214 | 9802 |
| 6 | 8142 | 7771 |  | 7956 |
| 7 | 8167 | 7506 | 7953 | 7876 |
| 8 | 7622 |  |  | 7622 |
| 9 | 8109 | 8617 | 8207 | 8311 |
| 10 | 9125 | 4305 | 8678 | 7370 |
| Average (ms-1) | 7974 | 7907 | 7957 |  |
| Standard Deviation (ms-1) | 637 | 2041 | 1840 |  |
| Coefficient of Variance (%) | 8% | 26% | 23% |  |

For measuring VoD, the ionisation pins were the most reliable method, having only missed one firing in comparison to the camera methods which each lost several firings of data. However, the camera methods involved no pre-trial preparation unlike the pins which were glued into the stands and measured with a micrometre, and only a calibration frame to be taken in between the firings. In between firings, the positions of the pins was double checked which was time-consuming and slowed down the running of the experiment.

The value for the VoD, as stated by the manufacturer for PE8, is 8,000ms-1 [18] which aligns, within the uncertainty margins, with the results from each method implying each method is accurate and also aligns with previous data [13] [32] [31]. As depicted in Figure 34 below, the three methods do not show a statistically significant difference in the average result of the measurement for VoD, shown by the crosses inside the boxes. Figure 34 also confirms that the uncertainty range for the ionisation pins method is the lowest implying a higher level of precision in the result.

Figure 34 Box plot of the results for the different VoD measurement techniques

# Discussion

Being able to perform more precise and accurate cylinder expansion based tests leads to more accurate models and better characterisation of the energetics and their ability to affect their surroundings. This would have an impact on a large variety of work in the energetics industry, such as more precise results for modelling of the velocities of fragmentation.

There were two electrical hazards from the instrumentation, the stress and strain gauges required a 100V high voltage power source and the ionisation pins required a 100V circuit on each pin to be shorted. The McDAX and DALEX set-ups, which replaced the stress gauge with a class 2 laser, reduced the hazards associated with the trial’s instrumentation making a more streamlined set-up. The use of fibre optic pins, whilst requiring more set-up before the trial, should also reduce the hazards and streamline the procedures on the trial.

From the X-rays of the tubes it was possible to determine the presence, or lack thereof, of voids. Voids are important as they can change a homogenous explosive fill into an in-homogenous fill. This means instead of the propagation of the explosive being driven by the shock initiation at the reaction zone, the shock imitation could occur where there are suitable voids ahead of the reaction zone; changing the rate of reaction.

For the tubes where the two rows of pins have significantly different velocity of detonation measurements, it could be inferred that the detonation front is not linear and therefore not moving down both side of the tube evenly. This could be due to the detonator not having been level in the top of the explosive, voids down one side of the tube, or uneven stemming of the tube. It was confirmed using the high speed video that this difference in velocity wasn’t a factor of the pins. To have more accurate results and ensure the VoD data is valid, only the results from tubes where the velocity of detonation down both sides of the tube were within 10% of one another were used. Using all the VoD measurements from the pins provides an average result of 7,974ms-1 with a coefficient of variance of 15%. Removing the results from tests where the two sides of pins gave results with a difference of greater than 10% gives an average result for the VoD of 7,998ms-1 and a coefficient of variance of 5%. The average value for the velocity of detonation hasn’t been altered significantly, but the precision of the result is significantly improved and so does the confidence in the measurement. It is possible to state that there is a 99% confidence that the true value for the Velocity of Detonation is within 353ms-1 of 7,998ms-1. This increase in precision is passed on to the calculation for PCJ; instead of a measurement of 27.8GPa with a coefficient of variance of 21%, the results are 28.7GPa with a coefficient of variance of 11%.

For future work, it would be worth investing in alternative measurement methods for the VoD which is non-invasive and has a higher precision; and having a method of confirming that the shockwave is planar, such as a prism at the base of the tube as used by the Atomic Weapons Establishment (AWE) [35].

The velocity of detonation is linked to the density of the explosive, as the explosives were hand-stemmed, their density was not constant down the tube as can be seen in Figure 35 and Figure 36. The density was calculated from the weight of the explosive and internal dimensions of the tube by measuring the weight of the stemmed tube, minus the weight of the tube, for every 20mm of PE stemmed into the tube.

Figure 35 Grpahical depiction of the density of the plastic explosive in the tubes

As more material is stemmed in from the bottom of the tube, the material at the top becomes more compacted leading to a high density section before a more steady-state. To account for this, the velocity measurements are made in the bottom two-thirds of the tube.

Figure 36 Density profiles for the stemmed tubes

For liquid explosives or powder explosives which can be formed into standardised pellets, this is potentially less of an issue and using these materials in the future for instrumentation verification will prevent this being a possible introduction of additional uncertainties in the measurement. It is possible to have plastic explosives manufactured to the shape of the tube to maintain a constant density, however, the author does not currently have access this capability and is unlikely to have access in the future.

The McDAX set-up gave more consistent het-v results than the DALEX set-up, this could be due to the disc being confined in the Perspex plate and therefore not as easily warped if the shock front is not planar. The initial rise of the velocity of the plate, where there is a high acceleration, is the hardest to capture clearly, the DALEX method allows for the Chapman-Jouguet pressure to be calculated from the initial rise data but also from the steady-state velocity data which is easier to measure. The DALEX method, therefore offers two data capture opportunities, doubling the likelihood of a good measurement being captured and reducing the potential need for extra repeats of firings. Also as fewer consumables and less equipment were required, it was a more cost-effective method of capturing the required data.

DSTG used a 6mm thick copper flyer plate in their experiments [20], giving it greater mass and therefore slower velocities which would be easier to measure, it would be worth considering using a heavier plate in the future to ease the physical set-up and analysis of results. The DALEX method paper by Stimac et al [7] also used a thicker copper for their experiments. A thicker and therefore sturdier plate material would make the technique less susceptible to warping from both being suspended in the tube below the explosives and from a non-linear shock front. However, it is desirable to keep the flyer plate as thin as possible to reduce it turning into a fragmentation hazard. The Gurney equations could be used to find an optimal flyer plate mass to make a slower flyer plate velocity which would improve the het-v measurements.

From an experimental outlook, the two tests were equally easy to perform and collect meaningful results from and the analysis of the data was similar. Meaning that neither set-up is of obvious preference going forwards. An important part of the cylinder expansion test is the modelling work undertaken after the experiment, therefore the repeatability and variance in the results is a key factor. There was a lack of repeatability in all the results from the tests, this could be due to the explosive not being well mixed, voids which weren’t visible in the x-rays or problems with the instrumentation introducing significant uncertainties. Other work done with hand-stemmed PE8 charges [36] have produced variable results similar to those observed here but further work would need to be undertaken to confirm this hypothesis.

The objectives from the project proposal have all been met and the plan laid out in the proposal was followed to conduct the experiment.

# Conclusion

A trial was successfully conducted to determine the Velocity of Detonation and Chapman-Jouguet pressure (PCJ) of the military grade plastic explosive material PE8. A 99% confidence interval was found to show that the true value for the Velocity of Detonation is within 353ms-1 of 7,998ms-1. The Chapman-Jouguet pressure, using the DALEX method which was determined to be the most reliable of the options was found to be 32 GPa with an uncertainty of 6GPa. Combining the results from all the different methods gave a PCJ value of 30.0GPa with an uncertainty of 7GPa. With these parameters it is possible to make some direct comparisons with other explosives to predict relative explosive power, gas production and TNT equivalence. Compared to other explosives such as TNT, C4, PE4 and RDX the material PE8 is found to be equivalent to RDX which is PE8’s main component [18].

Heterodyne velocimetry (het-v) wall expansion data was also captured which can be used alongside the measured parameters to fit a numerical analysis model and create an equation of state. This can be used to predict the behaviour of the explosives in different circumstances and can allow the material’s interaction with different warhead casings, hazardous incidents and safety cases to be modelled.

It was found during the trial that the ionisation pins were significantly more precise, reliable and repeatable than camera based methods of calculation of VoD. Of the three different methods of directly measuring the PCJ, the use of Manganin stress gauges was the least precise and repeatable. The use of the formula to derive PCJ from the VoD was found to be the most reliable but to carry over significant uncertainties from the VoD calculation leading to relatively imprecise data. This could be improved with a better precision measurement technique such as chirped fibre Bragg gratings which are currently unavailable to the author.

McDAX and DALEX were determined to both be suitable for future use in cylinder expansion trials, although more tests should be done for DALEX with plates of different thicknesses to confirm that would improve the reliability of the experiment. More het-v probes could be used in the measurement to improve the validity of the data and reliability of the test.

Further work should be undertaken to determine the effect of the varied density from hand-stemming charges on the VoD and PCJ, but this would require a higher precision VoD measurement technique.

The het-v measurement technique should also be improved before further tests are undertaken, so that the traces are not inverted, and to increase the power of the transmission laser to improve the signal to noise ratio of the signal particularly during the high acceleration phase.

The stands worked well and held the ionisation pins and het-v probes well allowing for precise measurements however, they were moved by the tube being emplaced which introduced a random uncertainty.

# References

| [1] | V. Kuznetsov, "Determination of Gurney Quantities and JWL Equation of State Parameters from the Cylinder Expansion Test," Defence Science Technology Group, Canberra, 2020. |
| --- | --- |
| [2] | R. Catanach, L. Hill, H. Harry, E. Aragon and D. Murk, "Cylinder Test Specification, Los Alamos National Laboratory Technical Report LA13643-MS," Los Alamos National Laboratory, New Mexico, 1999. |
| [3] | C. Rumchik, R. Nep, G. Butler and B. Breaux, "The Miniaturization and Reproducibility of the Cylinder Expansion Test," in AIP Conference Proceedings, 2012. |
| [4] | K. T. Lorenz, E. L. Lee and R. Chambers, "A Simple and Rapid Evaluation of Explosive Performance - The Disc Acceleration Experiment," Propellants, Explosives, Pyrotechnics, vol. 40, no. 1, p. September, 11 September 2014. |
| [5] | W. R. Maines, D. E. Kittell and M. L. Hobbs, "Combined Mini-Cylex & Disk Acceleration Tests in Type K Copper," Propellants, Explosives, Pyrotechnics, vol. 43, no. 5, pp. 506-511, 2018. |
| [6] | T. Baust, "Improving the Design and Evaluation of PDV-Based Cylinder Test Experiments for JWL-Parameter Determination," Propellants, Explosives, Pyrotechnics, vol. 45, no. 9, pp. 1344-1356, 2020. |
| [7] | B. Stimac, M. Kunzel, M. Suceska, S. Stankovic and J. Kucera, "Detonation Reaction Zone in Nitromethane: Experimental and Numerical Studies," New Treds in Research of Energetic Materials, pp. 216-228, 2019. |
| [8] | P. Cooper, Explosives Engineering, New York: Wilwy-VCH, 1996. |
| [9] | J. Mungham, "Detonics Trial Plan," Dstl, Salibsury, 2021. |
| [10] | J. Mungham, "Detontics Trial Risk Assessment," Dstl, Salisbury, 2021. |
| [11] | C. Tanner and P. Urtiew, "Theoretical and Computer Models of Detonation in Solid Explosives," in International Workshop on new Models and Numerical Codes for SHock Wave Processes in Condensed Media, Oxford, 1997. |
| [12] | A. Alia and M. Souli, "High Explosive Simulations Using Multi-Material Formulations," Applied Thermal Engineering, vol. 26, no. 10, pp. 1032-1042, 2006. |
| [13] | J. Mungham and R. Louise, "How to get a Detonating Explosive into a Computer Model," Dstl Counter Terrorism & Security Division Journal, vol. 1, no. 4, 2020. |
| [14] | Wikipedia, "TNT," Wikipedia, 7 January 2020. [Online]. Available: https://en.wikipedia.org/wiki/TNT. [Accessed January 2020]. |
| [15] | Ribbands Explosives, C4 Plastic Explosive, Ribbands Explosives, 2020. |
| [16] | M. C. Chick and L. A. Learmonth, "Determination of shock initiation and detonation characteristics of PE4 in proof test geometries," Department of Defence, Melbourne, 1985. |
| [17] | Wikipedia, "RDX," Wikipedia, 5 November 2019. [Online]. Available: https://en.wikipedia.org/wiki/RDX. [Accessed January 2020]. |
| [18] | Chemring Energetics UK, "PE8 Plastic Explosive," [Online]. Available: https://www.chemring.com/~/media/Files/C/Chemring-V3/documents/energetics/PE8%20Plastic%20Explosive%20Iss%2013%20Nov%2020.pdf. [Accessed 28 October 2020]. |
| [19] | M. Chaos, E. Lee and K. Lorenz, "Experimental Determination of Chapman-Jouguet Pressure Using Disc Acceleration eXperiment (DAX) Data," in 16th International Detonation Symposium, Cambridge, 2018. |
| [20] | A. Provatas, P. Anderson, R. Ripley, P. Cheese, S. Bibby, M. Daniel, M. Fransom and V. Kuznetsov, "TTCP Technical Report FINAL REPORT FOR WPN TP4 CP-4-52-17 High Efficiency Explosives for Extreme Environments," DSTG, Edinburgh, Australia, 2021. |
| [21] | V. Kuznetsov and J. Mungham, Private Emails, Edingburgh: DSTG, 2021. |
| [22] | Meggaflash Technologies, "Flashbulb SLow Peak PF300," Meggaflash Technologies, [Online]. Available: meggaflash.com/index.php/meggaflash-bulbs/flashbulb-slow-peak-pf300. [Accessed 6 Spetember 2022]. |
| [23] | AC Photonics, "Collimator Dat Sheet," [Online]. Available: https://acphotonics.com/downloads/Single-mode_Single_Fiber_Collimator.pdf. [Accessed 14 January 2022]. |
| [24] | Dynasen Inc, "Mn-Cn4-50-EK-20," Dynasen Inc, [Online]. Available: http://dynasen.com/product/mn-cn4-50-ek-20/. [Accessed 21 January 2022]. |
| [25] | imc, "FAMOS," imc, 2022. [Online]. Available: https://www.imc-tm.com/products/measurement-software/imc-famos/. [Accessed 31 January 2022]. |
| [26] | MathWorks, "MATLAB," MathWorks, 2021. [Online]. Available: https://uk.mathworks.com/products/matlab.html. [Accessed 20 October 2021]. |
| [27] | Vision Research Inc, "PCC Software," Vision Research Inc, 2021. [Online]. Available: https://www.phantomhighspeed.com/resourcesandsupport/phantomresources/pccsoftware. [Accessed 20 October 2021]. |
| [28] | HBM, Perception 6.4, HBM, 2012. |
| [29] | Specialised Imaging, SIM 2, Specialised Imaging, 2017. |
| [30] | Statistics How To, "Sample Size in Statistics," Statistics How To, 2002. [Online]. Available: https://www.statisticshowto.com/probability-and-statistics/find-sample-size/. [Accessed 29 July 2022]. |
| [31] | L. Rugman, "Cylinder Expansion (Cylex) Tests PE8," Defence Science Technology Laboratory, Salisbury, 2018. |
| [32] | J. Mungham and L. Rugman, "Trial Summary Report - Cylex 2," Defence Science Technology Laboratory, Salisbury, 2019. |
| [33] | T. Hutson, Manganin Stress Gauge Calibration Proceedure, Kent: Dstl, 2000. |
| [34] | T. Hutson, Manganin Stress Gauge Calibration Process, Dstl, 2010. |
| [35] | B. Sutton, F. J and A. Hodgson, "An Analytical Approach to Obtaining JWL Parameters from Cylinder Tests," in AIP Conference Proceedings, 2017. |
| [36] | L. Stockford and S. Bibby, "Reactive Materials Study," Dstl, Salibsury, 2021. |
| [37] | J. Mungham, Technical Drawing for Cylex Stand, Salisbury: Dstl, 2021. |
| [38] | J. Mungham, Technical Drawing for DALEX stand, Salisbury: Dstl, 2021. |
| [39] | K. Turner, Stemming Proceedure for Plastic Explosives, Salisbury: Dstl, 2018. |

# Abbreviations

| TLA | Three letter acronym |
| --- | --- |
| AWE CylEx | Atomic Weapons Establishment Cylinder Expansion |
| DAX | Disc Acceleration Experiment |
| DSTG | Defence Science & Technology Group |
| Dstl | Defence Science and Technology Laboratory |
| Het-V | Heterodyne Velocimetry |
| PCJ PDV PMMA VoD | Chapman-Jouguet Pressure Photon Doppler Velocimetry Poly(methyl methacrylate) Velocity of Detonation |

# Symbols

| λ | Wavelength |
| --- | --- |
| PCJ | Chapman-Jouguet Pressure |
| VoD | Velocity of Detonation |
| Δt | Difference in time |
| Δx | Displacement |
| ρ0 | Initial Density of Explosive material |
| D | Velocity of Detonation |
| A | JWL EOS Constant |
| B | JWL EOS Constant |
| ω | JWL EOS Constant |
| R2 | JWL EOS Constant |
| V | Velocity of Detonation |
| p | pressure |
| 2EG | Gurney Constant |
| M | Mass of Case |
| C | Mass of explosive |
| Pc | Stress is copper |
| US | Shock wave speed |
| UP | Particle Velocity |
| γ | Choas Formula Factor |
| UW | Velocity of Flyer Plate |
| m | Meters |
| s | Seconds |
| ms-1 | Meters per second |
| GPa | Giga-Pascal |

# Appendix

## Shot Statistics

|  | VoD | PCJ |
| --- | --- | --- |
| Expected Number of Samples | 3 | 3 |
| Expected Mean | 8000 | 30 |
| Expected Standard Deviation | 500 | 1 |
| Confidence Level | 95 | 95 |
| Confidence Limits | 566 | 1 |
| Margin of Error | 5 | 5 |
| Actual Number of Shots Required | 3 | 3 |

T Values

| n | 70 | 75 | 80 | 85 | 90 | 95 | 98 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.963 |  | 3.078 |  | 6.314 | 12.710 | 31.820 | 63.660 | 636.620 |
| 2 | 1.386 |  | 1.886 |  | 2.920 | 4.303 | 6.965 | 9.925 | 31.599 |
| 3 | 1.250 |  | 1.638 |  | 2.353 | 3.182 | 4.541 | 5.841 | 12.924 |
| 4 | 1.190 |  | 1.533 |  | 2.132 | 2.776 | 3.747 | 4.604 | 8.610 |
| 5 | 1.156 |  | 1.476 |  | 2.105 | 2.571 | 3.365 | 4.032 | 6.869 |
| 6 | 1.134 |  | 0.014 |  | 1.943 | 2.447 | 3.143 | 3.707 | 5.959 |
| 7 | 1.119 |  | 1.415 |  | 1.895 | 2.365 | 2.998 | 3.499 | 5.408 |
| 8 | 1.108 |  | 1.397 |  | 1.860 | 2.306 | 2.896 | 3.355 | 5.041 |
| 9 | 1.100 |  | 1.383 |  | 1.833 | 2.262 | 2.821 | 3.250 | 4.781 |
| 10 | 1.093 |  | 0.014 |  | 1.812 | 2.228 | 2.764 | 3.169 | 4.587 |
| 11 | 1.088 |  | 1.362 |  | 1.796 | 2.201 | 2.718 | 3.106 | 4.437 |
| 12 | 1.083 |  | 1.356 |  | 1.782 | 2.179 | 2.681 | 3.055 | 4.318 |
| 13 | 1.079 |  | 0.014 |  | 1.771 | 2.160 | 2.650 | 3.012 | 4.221 |
| 14 | 1.076 |  | 1.345 |  | 1.761 | 2.145 | 2.624 | 2.977 | 4.140 |
| 15 | 1.074 |  | 1.341 |  | 1.753 | 2.131 | 2.602 | 2.947 | 4.073 |
| 16 | 1.071 |  | 1.337 |  | 1.746 | 2.120 | 2.583 | 2.921 | 4.015 |
| 17 | 1.069 |  | 1.333 |  | 1.740 | 2.110 | 2.567 | 2.898 | 3.965 |
| 18 | 1.067 |  | 1.330 |  | 1.734 | 2.101 | 2.552 | 2.878 | 3.922 |
| 19 | 1.066 |  | 1.328 |  | 1.729 | 2.093 | 2.539 | 2.861 | 3.883 |
| 20 | 1.064 |  | 1.325 |  | 1.725 | 2.086 | 2.528 | 2.845 | 3.850 |
| 21 | 1.063 |  | 1.323 |  | 1.721 | 2.080 | 2.518 | 2.831 | 3.819 |
| 22 | 1.061 |  | 1.321 |  | 1.717 | 2.074 | 2.508 | 2.819 | 3.792 |
| 23 | 1.060 |  | 1.319 |  | 1.714 | 2.069 | 2.500 | 2.807 | 3.768 |
| 24 | 1.059 |  | 1.318 |  | 1.711 | 2.064 | 2.492 | 2.797 | 3.745 |
| 25 | 1.058 |  | 1.316 |  | 1.708 | 2.060 | 2.485 | 2.787 | 3.725 |
| 26 | 1.058 |  | 1.315 |  | 1.709 | 2.056 | 2.479 | 2.779 | 3.707 |
| 27 | 1.057 |  | 1.314 |  | 1.703 | 2.052 | 2.473 | 2.771 | 3.690 |
| 28 | 1.056 |  | 1.313 |  | 1.701 | 2.048 | 2.467 | 2.763 | 3.674 |
| 29 | 1.055 |  | 1.311 |  | 1.699 | 2.045 | 2.462 | 2.756 | 3.659 |
| 30 | 1.055 |  | 1.310 |  | 1.697 | 2.042 | 2.457 | 2.750 | 3.646 |

## Data Capture Record

Example of data sheet used to log all set-up information required for data analysis and modelling post-trial. Alignment is taken from cameras’ point of view.

## Tube and Plate Data

Tubes

| Firing | Material | Length (m) | Outer Diameter (m) | Inner Diameter (m) | Mass (kg) | Wall Thickness (m) | Density (kgm-3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Steel | 0.3001 | 0.0300 | 0.0295 | 0.506 | 0.0025 | 69409.73 |
| 2 | Copper | 0.3000 | 0.0300 | 0.0250 | 0.5068 | 0.0026 | 7821.549 |
| 3 | Copper | 0.3000 | 0.0300 | 0.0256 | 0.507 | 0.0022 | 8795.679 |
| 4 | Copper | 0.3000 | 0.0300 | 0.0260 | 0.507 | 0.002 | 9606.138 |
| 5 | Copper | 0.3000 | 0.0300 | 0.0260 | 0.51 | 0.002 | 9662.979 |
| 6 | Copper | 0.3001 | 0.0300 | 0.0260 | 0.508 | 0.002 | 9621.877 |
| 7 | Copper | 0.3001 | 0.0300 | 0.0260 | 0.507 | 0.002 | 9602.937 |
| 8 | Copper | 0.3000 | 0.0300 | 0.0260 | 0.507 | 0.002 | 9606.138 |
| 9 | Copper | 0.3000 | 0.0300 | 0.0260 | 0.507 | 0.002 | 9606.138 |
| 10 | Copper | 0.3000 | 0.0300 | 0.0260 | 0.507 | 0.002 | 9606.138 |

Plates

| Firing | Diameter (m) | Mass (kg) | Thickness (m) | Radius (m) | Cross-Sectional Area (m2) | Volume (m3) | Density (kgm-3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 0.027 | 0.00000035 | 0.00017 | 0.0135 | 0.000573 | 9.73E-08 | 3.595851 |
| 5 | 0.027 | 0.0000003 | 0.00017 | 0.0135 | 0.000573 | 9.73E-08 | 3.082158 |
| 6 |  |  |  |  |  |  |  |
| 7 | 0.026 | 0.0000002 | 0.00005 | 0.013 | 0.000531 | 2.65E-08 | 7.533962 |
| 8 | 0.027 | 0.0000002 | 0.00005 | 0.0135 | 0.000573 | 2.86E-08 | 6.986225 |
| 9 | 0.027 | 0.0000002 | 0.00005 | 0.0135 | 0.000573 | 2.86E-08 | 6.986225 |

Explosive

| Depth of tube (m) | Firing 2 |  | Firing 3 |  | Firing 4 |  | Firing 5 |  | Firing 6 |  | Firing 7 |  | Firing 8 |  | Firing 9 |  | Firing 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) | NEQ (kg) | Density (kgm-3) |
| 0.3 | 0.754 | 1678.6 | 0.752 | 1586.6 | 0.754 | 1550.7 | 0.758 | 1557.0 | 0.756 | 1557.0 | 0.754 | 1550.4 |  |  | 0.752 | 1538.2 |  |  |
| 0.28 | 0.738 | 1682.1 | 0.736 | 1588.9 | 0.740 | 1567.3 | 0.738 | 1533.7 | 0.736 | 1533.7 | 0.736 | 1540.4 | 0.736 | 1540.4 | 0.734 | 1527.0 | 0.734 | 1527.0 |
| 0.26 | 0.724 | 1701.8 | 0.722 | 1606.6 | 0.720 | 1543.0 | 0.722 | 1535.8 | 0.722 | 1550.3 | 0.720 | 1543.0 | 0.716 | 1514.0 | 0.718 | 1528.5 | 0.718 | 1528.5 |
| 0.24 | 0.706 | 1690.9 | 0.704 | 1594.7 | 0.706 | 1561.7 | 0.704 | 1522.5 | 0.706 | 1553.9 | 0.702 | 1530.3 | 0.700 | 1514.6 | 0.702 | 1530.3 | 0.702 | 1530.3 |
| 0.22 | 0.692 | 1714.9 | 0.688 | 1598.4 | 0.688 | 1549.6 | 0.690 | 1541.0 | 0.688 | 1541.0 | 0.688 | 1549.6 | 0.686 | 1532.5 | 0.686 | 1532.9 | 0.688 | 1549.6 |
| 0.20 | 0.676 | 1723.5 | 0.672 | 1602.8 | 0.672 | 1553.9 | 0.674 | 1544.5 |  |  | 0.672 | 1553.9 | 0.670 | 1535.0 | 0.670 | 1535.0 | 0.672 | 1553.9 |
| 0.18 | 0.660 | 1733.9 | 0.654 | 1586.6 | 0.656 | 1559.1 | 0.658 | 1548.7 | 0.656 | 1548.7 | 0.656 | 1559.1 | 0.654 | 1538.2 | 0.652 | 1517.3 | 0.656 | 1559.1 |
| 0.16 | 0.642 | 1721.4 | 0.648 | 1712.1 | 0.638 | 1542.1 | 0.642 | 1553.9 | 0.640 | 1553.9 | 0.640 | 1565.7 | 0.640 | 1565.7 | 0.636 | 1518.7 | 0.638 | 1542.1 |
| 0.14 | 0.626 | 1734.5 | 0.622 | 1595.9 | 0.624 | 1574.1 | 0.626 | 1560.6 | 0.622 | 1533.7 | 0.622 | 1547.2 | 0.624 | 1574.1 | 0.622 | 1547.2 | 0.622 | 1547.2 |
| 0.12 | 0.608 | 1718.0 | 0.606 | 1602.8 | 0.608 | 1585.3 | 0.608 | 1538.2 | 0.608 | 1569.6 | 0.610 | 1616.7 | 0.604 | 1522.5 | 0.608 | 1585.3 | 0.604 | 1522.5 |
| 0.10 | 0.592 | 1735.7 | 0.590 | 1612.5 | 0.000 |  | 0.592 | 1544.5 | 0.596 | 1657.5 |  |  | 0.588 | 1525.6 | 0.592 | 1601.0 | 0.588 | 1525.6 |
| 0.08 | 0.576 | 1762.2 | 0.574 | 1627.1 | 0.574 | 1577.4 | 0.578 | 1601.0 | 0.578 | 1648.1 | 0.580 | 1718.7 | 0.574 | 1577.4 | 0.574 | 1577.4 | 0.574 | 1577.4 |
| 0.06 | 0.562 | 1874.2 | 0.560 | 1716.2 | 0.558 | 1601.0 | 0.564 | 1695.1 | 0.562 | 1695.1 | 0.562 | 1726.5 | 0.560 | 1663.8 | 0.560 | 1663.8 | 0.556 | 1538.2 |
| 0.04 |  |  | 0.544 | 1797.1 | 0.544 | 1742.2 | 0.550 | 1883.5 | 0.544 | 1695.1 | 0.544 | 1742.2 | 0.544 | 1742.2 | 0.542 | 1648.1 | 0.544 | 1742.2 |
| 0.02 | 0.528 | 2159.4 | 0.526 | 1845.7 | 0.521 | 1318.4 | 0.534 | 2260.2 | 0.528 | 1883.5 | 0.528 | 1977.7 | 0.526 | 1789.3 | 0.526 | 1789.3 | 0.530 | 2166.0 |

## Stress Sensor Calibration Processs

| 1 | Connect the sensor to the end of the cable you will be using for the test, then, using an ohmmeter, measure and record the total resistance of gauge and cable. Connect the other end of the cable to the PPS, ensuring that the Decade box is in series with same. Record the resistance you measured in the '4th Order' spreadsheet in cell C23. |
| --- | --- |
| 2 | Ensure the resistance Decade Box is set to 0.00Ω. |
| 3 | Follow the procedure outlined in the PPS manual to zero the bridge, so that the output is as close to zero as practicable, I have found that the PPS display shows approx. -006 when closest to zero but this display lacks resolution, so I use the recorder to measure the offset and I can usually get it to less than 10mV easily. Small offsets are not a problem as they are subtracted from the result anyway. |
| 4 | Once you are happy with the offset, lock the control and record the voltage from 3 separate measurements. The spreadsheet "4th Order" is designed for this purpose, put the values in cells |
|  | 3C, 3D and 3E. |
| 5 | Dial up 0.1Ω on the Decade Box, then make 3 measurements of the resulting voltage and record. |
| 6 | Repeat step 5 with resistance values of 0.2, 0.5, 1, 2, 5, 10, 20 and 25Ω. Adjust the recorders sensitivity as required to measure the voltage output. |
| 7 | The spreadsheet produces a plot of resistance change versus output Voltage. It also fits a 4th order polynomial to the curve and calculates the coefficients, these will be used to convert the output voltage from the test to a percentage resistance change and hence a pressure can be found from the manufacturer’s data. |
| 8 | Re-set the resistance decade box to 0.000Ω. |

[33]

## CAD Drawings

3D printed stand for the CylEx and McDAX firings

[37]

3D printed stand for the DALEX firings.

[38]

## Stemming Procedure

Purpose [39]

This procedure details the method for the stemming of charges with the following plastic explosives:

- PE8 (Due to replace PE7 2014/15)

- PE7

- PE4

- PE4/Al

- PE6

- C4

- SEMTEX H

- SEMTEX 1A

Scope

This procedure is applicable to any member of staff who has been suitably authorised and is competent to carry out the above operation.

General requirements

Basic requirements

Any person using this procedure shall have had adequate training in the use and handling of explosives and the techniques involved and shown themselves to be competent. They must have a Certificate of competency [1] authorising them as being capable of carrying out this work or work under the direct supervision of someone who has been deemed competent to do so (e.g. a TCO or TM).

Before any attempt to proceed with this process the personnel involved shall ensure that the activity is covered by a suitable and sufficient risk assessment [2], liaising with other groups/contractors who may be involved, as necessary. The risk assessment must address, as a minimum, the following risks, which have been identified with the process:

- Toxicity

- Handling

- Compatibilty.

Any relevant requirements as listed on the Management System under working with explosives must be met.

Personal protective equipment

All personnel shall be provided with PPE relevant to the specific task as stated in the risk assessment.

Equipment and Material

Tools may be required to assist in the stemming procedure:

- Stemming stick (e.g. wood)

- Brush (soft bristle)

- Soft cloth

All materials used above must be non-sparking, compatible and anti-static.

Work description

On entering the laboratory or designated explosive preparation area the personnel carrying out the work described below should be wearing the necessary Personal Protective Equipment identified in the Risk Assessment and earth themselves by placing their bare hands on a conducting surface.

Before starting the operation, the charge case must be inspected to ensure that it is clean and free from grit or other contaminants that could affect the safety of the stemming operation.

If the explosive is cold or hard it should be kneaded by hand until it is more malleable before stemming commences.

The stemming stick or sticks used to compress the plastic explosive within the charge case should be a loose fit so that the stick cannot become stuck or jammed within the charge case.

For large charge cases, increments of 50 to 100 g of plastic explosive are taken from the bulk supply and inserted into the charge case. Each increment is consolidated by compressing within the container with a stemming stick or fingers using only manual pressure (to ensure as far as possible that no voids occur). Continue with the increments until the explosive has reached the desired height or weight. For small charge cases smaller increments of plastic explosive should be used.

The charge case should be cleaned of any loose explosive debris, especially on any threads or areas that could be compressed accidentally, using a cloth or brush. Alternatively cover the threads with tape prior to stemming to prevent ingress of explosive into the threads.

The completed charge case should then be cleaned externally with a cloth to remove any explosive debris, labelled and stored until required. For storage timescales please refer to the compatibility section in the trials plan or work instruction.

## Firing Log

| Date | Time | Temperature (oC) | Original | Shot | Description | Data Capture Log |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cameras |  |  |  | Pins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Gauges |  | Het-V |  |  |  |
|  |  |  |  |  |  | SIM | v2512 | TMX7510 | T3510 | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | Mang | Strain | 1 | 2 | 3 | 4 |
| 09/12/2021 | 08:38 | 7 | 2 | 1 | Steel Cylex | n | y | n/a | n/a | y | y | y | y | y | y | y | y | y | y | y | y | y | y | y | y | n | n | n | n | n | n |
| 08/12/2021 | 15:07 | 4 | 1 | 2 | Cu Cylex | n | n | n/a | n/a | y | y | y | y | y | y | y | y | y | y | y | y | n | y | n | y | y | y | y | y | n | n |
| 13/12/2021 | 10:49 | 10 | 6 | 3 | Cu Cylex | y | y | n/a | n/a | y | n | n | n | y | y | y | y | y | y | y | y | n | y | y | y | y | y | y | y | n | n |
| 13/12.2021 | 14:07 | 10.5 | 7 | 4 | Cu Cylex | y | y | n/a | n/a | y | y | y | y | y | y | y | n | y | y | y | y | y | y | y | y | y | y | y | y | n | n |
| 14/12/2021 | 10:54 | 10.5 | 8 | 5 | DALEX | y | y | y | n/a | y | y | n | y | y | y | y | y | y | y | y | y | y | y | y | y | n/a | n/a | y | n | n | n |
| 14/12/2021 | 13:46 | 10.5 | 9 | 6 | DALEX | y | n/a | n | n | y | y | y | y | y | y | y | y | y | n | y | y | y | y | n | y | n/a | n/a | y | n | n | n |
| 14/12/2021 | 15:17 | 10.5 | 10 | 7 | DALEX | y | n/a | y | y | y | y | y | n | n | y | y | y | y | y | n | n | y | n | y | y | n/a | n/a | y | n | n | n |
| 15/12/2021 | 10:08 | 10 | 11 | 8 | McDAX | n | n | n/a | n/a | y | y | y | n | y | y | y | y | y | y | y | y | y | y | y | n | n/a | n/a | y | n/a | n/a | n/a |
| 15/12/2021 | 12:15 | 11 | 12 | 9 | McDAX | y | y | n/a | n/a | y | y | y | y | y | y | n | y | y | y | y | y | y | n | n | y | n/a | n/a | y | n/a | n/a | n/a |
| 15/12/2021 | 13:58 | 10 | 13 | 10 | McDAX | y | y | n/a | n/a | n | y | y | n | y | y | y | y | y | y | y | y | y | y | n | y | n/a | n/a | y | n/a | n/a | n/a |

## Ionisation Pin Results

Timing signals

| Shot | Pin | Distance between Pins | Displacement |  | Time | Time Difference |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mm | mm | m | ms | ms | s |
| 1 | A1 | 0 | 0 | 0 | 0.31548 | 0.000 | 0.0000000 |
|  | A2 | 8.4 | 9.5 | 0.0095 | 0.31708 | 0.002 | 0.0000016 |
|  | A3 | 8.74 | 19.34 | 0.01934 | 0.31851 | 0.003 | 0.0000030 |
|  | A4 | 9.14 | 29.58 | 0.02958 | 0.32096 | 0.005 | 0.0000055 |
|  | A5 | 8.42 | 39.1 | 0.0391 | 0.32183 | 0.006 | 0.0000064 |
|  | A6 | 9.53 | 49.73 | 0.04973 | 0.323372 | 0.008 | 0.0000079 |
|  | A7 | 10.69 | 61.52 | 0.06152 | 0.32655 | 0.011 | 0.0000111 |
|  | A8 | 8.23 | 70.85 | 0.07085 | 0.32777 | 0.012 | 0.0000123 |
|  | B1 | 0 | 0 | 0 | 0.72021 | 0.000 | 0.0000000 |
|  | B2 | 8.86 | 9.96 | 0.00996 | 0.72109 | 0.001 | 0.0000009 |
|  | B3 | 9.51 | 20.57 | 0.02057 | 0.72306 | 0.003 | 0.0000029 |
|  | B4 | 9.04 | 30.71 | 0.03071 | 0.72401 | 0.004 | 0.0000038 |
|  | B5 | 9.84 | 41.65 | 0.04165 | 0.7256 | 0.005 | 0.0000054 |
|  | B6 | 8.52 | 51.27 | 0.05127 | 0.7268 | 0.007 | 0.0000066 |
|  | B7 | 8.04 | 60.41 | 0.06041 | 0.72823 | 0.008 | 0.0000080 |
|  | B8 | 8.48 | 69.99 | 0.06999 | 0.7303 | 0.010 | 0.0000101 |
| 2 | A1 | 185 |  | 0 | 0.84749 | 0.000 | 0.0000000 |
|  | A2 | 13 | 14.1 | 0.0141 | 0.84927 | 0.002 | 0.0000018 |
|  | A3 | 14.2 | 29.4 | 0.0294 | 0.85129 | 0.004 | 0.0000038 |
|  | A4 | 14.2 | 44.7 | 0.0447 | 0.85364 | 0.006 | 0.0000061 |
|  | A5 | 13.3 | 59.1 | 0.0591 | 0.85481 | 0.007 | 0.0000073 |
|  | A6 | 13.9 | 74.1 | 0.0741 | 0.85677 | 0.009 | 0.0000093 |
|  | A7 | 13.9 | 89.1 | 0.0891 | 0.85887 | 0.011 | 0.0000114 |
|  | A8 | 13.1 | 103.3 | 0.1033 | 0.86038 | 0.013 | 0.0000129 |
|  | B1 | 185 | 0 | 0 | 3.19982 | 0.000 | 0.0000000 |
|  | B2 | 13.8 | 14.9 | 0.0149 | 0.00000 |  |  |
|  | B3 | 14.6 | 30.6 | 0.0306 | 3.20244 | 0.003 | 0.0000026 |
|  | B4 | 14.1 | 45.8 | 0.0458 | 3.20446 | 0.005 | 0.0000046 |
|  | B5 | 14 | 60.9 | 0.0609 | 0.00000 |  |  |
|  | B6 | 13.6 | 75.6 | 0.0756 | 3.20876 | 0.009 | 0.0000089 |
|  | B7 | 13.5 | 90.2 | 0.0902 | 0.00000 |  |  |
|  | B8 | 13.4 | 105.8 | 0.1058 | 3.21213 | 0.012 | 0.0000123 |
| 3 | A1 | 0 | 0 | 0 |  | 0.000 | 0.0000000 |
|  | A2 | 8.37 | 9.47 | 0.00947 |  |  |  |
|  | A3 | 7.17 | 17.74 | 0.01774 |  |  |  |
|  | A4 | 8.44 | 27.28 | 0.02728 |  |  |  |
|  | A5 | 8.02 | 36.4 | 0.0364 |  | 0.000 | 0.0000000 |
|  | A6 | 9.11 | 46.61 | 0.04661 |  | 0.000 | 0.0000000 |
|  | A7 | 9.05 | 56.76 | 0.05676 |  | 0.000 | 0.0000000 |
|  | A8 | 28.43 | 86.29 | 0.08629 |  | 0.000 | 0.0000000 |
|  | B1 | 0 | 0 | 0 |  | 0.000 | 0.0000000 |
|  | B2 | 8.52 | 9.62 | 0.00962 |  | 0.000 | 0.0000000 |
|  | B3 | 9.09 | 19.81 | 0.01981 |  | 0.000 | 0.0000000 |
|  | B4 | 8.27 | 29.18 | 0.02918 |  | 0.000 | 0.0000000 |
|  | B5 | 8.95 | 39.23 | 0.03923 |  |  |  |
|  | B6 | 9.14 | 49.47 | 0.04947 |  | 0.000 | 0.0000000 |
|  | B7 | 8.94 | 59.51 | 0.05951 |  | 0.000 | 0.0000000 |
|  | B8 | 26.11 | 86.72 | 0.08672 |  | 0.000 | 0.0000000 |
| 4 | A1 | 0 | 0 | 0 | 4.94942 | 0.000 | 0.0000000 |
|  | A2 | 7.92 | 9.02 | 0.00902 | 4.95033 | 0.001 | 0.0000009 |
|  | A3 | 8.52 | 18.64 | 0.01864 | 4.95199 | 0.003 | 0.0000026 |
|  | A4 | 9.63 | 29.37 | 0.02937 | 4.953498 | 0.004 | 0.0000041 |
|  | A5 | 8.73 | 39.2 | 0.0392 | 4.95478 | 0.005 | 0.0000054 |
|  | A6 | 8.77 | 49.07 | 0.04907 | 4.95601 | 0.007 | 0.0000066 |
|  | A7 | 9.27 | 59.44 | 0.05944 | 4.95733 | 0.008 | 0.0000079 |
|  | A8 | 28.1 | 88.64 | 0.08864 |  |  |  |
|  | B1 | 0 | 0 | 0 | 2.85529 | 0.000 | 0.0000000 |
|  | B2 | 8.96 | 10.06 | 0.01006 | 2.85706 | 0.002 | 0.0000018 |
|  | B3 | 6.93 | 18.09 | 0.01809 | 2.85814 | 0.003 | 0.0000029 |
|  | B4 | 9.96 | 29.15 | 0.02915 | 2.85864 | 0.003 | 0.0000033 |
|  | B5 | 8.84 | 39.09 | 0.03909 | 2.860507 | 0.005 | 0.0000052 |
|  | B6 | 7.85 | 48.04 | 0.04804 | 2.86093 | 0.006 | 0.0000056 |
|  | B7 | 9.28 | 58.42 | 0.05842 | 2.86226 | 0.007 | 0.0000070 |
|  | B8 | 29.63 | 89.15 | 0.08915 | 2.86624 | 0.011 | 0.0000109 |
| 5 | A1 | 13.33 | 0 | 0 | 4.65643 | 0.000 | 0.0000000 |
|  | A2 | 14.6 | 15.7 | 0.0157 | 4.65843 | 0.002 | 0.0000020 |
|  | A3 | 11.91 | 28.71 | 0.02871 |  |  |  |
|  | A4 | 15.39 | 45.2 | 0.0452 | 4.66326 | 0.007 | 0.0000068 |
|  | A5 | 12.57 | 58.87 | 0.05887 | 4.66399 | 0.008 | 0.0000076 |
|  | A6 | 14.52 | 74.49 | 0.07449 | 4.6659 | 0.009 | 0.0000095 |
|  | A7 | 13.85 | 89.44 | 0.08944 | 4.667663 | 0.011 | 0.0000112 |
|  | A8 | 13.53 | 104.07 | 0.10407 |  |  |  |
|  | B1 | 13.47 | 0 | 0 | 5.71373 | 0.000 | 0.0000000 |
|  | B2 | 12.82 | 13.92 | 0.01392 | 5.71497 | 0.001 | 0.0000012 |
|  | B3 | 14.23 | 29.25 | 0.02925 |  |  |  |
|  | B4 | 13.26 | 43.61 | 0.04361 |  |  |  |
|  | B5 | 13.45 | 58.16 | 0.05816 | 5.72154 | 0.008 | 0.0000078 |
|  | B6 | 14.69 | 73.95 | 0.07395 | 5.72237 | 0.009 | 0.0000086 |
|  | B7 | 13.47 | 88.52 | 0.08852 | 5.72453 | 0.011 | 0.0000108 |
|  | B8 | 13.21 | 102.83 | 0.10283 | 5.72761 | 0.014 | 0.0000139 |
| 6 | A1 | 0 | 0 | 0 | 5.71353 | 0.000 | 0.0000000 |
|  | A2 | 13.12 | 14.22 | 0.01422 | 5.71464 | 0.001 | 0.0000011 |
|  | A3 | 13.66 | 28.98 | 0.02898 | 5.71784 | 0.004 | 0.0000043 |
|  | A4 | 12.67 | 42.75 | 0.04275 | 5.71853 | 0.005 | 0.0000050 |
|  | A5 | 14.17 | 58.02 | 0.05802 | 5.72043 | 0.007 | 0.0000069 |
|  | A6 | 12.61 | 71.73 | 0.07173 | 5.72237 | 0.009 | 0.0000088 |
|  | A7 | 14.07 | 86.9 | 0.0869 | 5.72418 | 0.011 | 0.0000106 |
|  | A8 | 14.11 | 102.11 | 0.10211 | 5.726 | 0.012 | 0.0000125 |
|  | B1 | 0 | 0 | 0 | 5.71369 | 0.000 | 0.0000000 |
|  | B2 | 14.94 | 16.04 | 0.01604 |  |  |  |
|  | B3 | 13.06 | 30.2 | 0.0302 | 5.71776 | 0.004 | 0.0000041 |
|  | B4 | 13.31 | 44.61 | 0.04461 | 5.71869 | 0.005 | 0.0000050 |
|  | B5 | 13.7 | 59.41 | 0.05941 | 5.7205 | 0.007 | 0.0000068 |
|  | B6 | 13.65 | 74.16 | 0.07416 | 5.72289 | 0.009 | 0.0000092 |
|  | B7 | 13.08 | 88.34 | 0.08834 |  |  |  |
|  | B8 | 12.95 | 102.39 | 0.10239 | 5.7266 | 0.013 | 0.0000129 |
| 7 | A1 | 0 | 0 | 0 | 7.8273 | 0.000 | 0.0000000 |
|  | A2 | 12.89 | 13.99 | 0.01399 | 7.82913 | 0.002 | 0.0000018 |
|  | A3 | 15.98 | 31.07 | 0.03107 | 7.83231 | 0.005 | 0.0000050 |
|  | A4 | 12.36 | 44.53 | 0.04453 |  |  |  |
|  | A5 | 13.59 | 59.22 | 0.05922 |  |  |  |
|  | A6 | 15.69 | 76.01 | 0.07601 | 7.83806 | 0.011 | 0.0000108 |
|  | A7 | 12.69 | 89.8 | 0.0898 | 7.8393 | 0.012 | 0.0000120 |
|  | A8 | 11.96 | 102.86 | 0.10286 | 7.84019 | 0.013 | 0.0000129 |
|  | B1 | 0 | 0 | 0 | 5.71459 | 0.000 | 0.0000000 |
|  | B2 | 13.59 | 14.69 | 0.01469 | 5.71682 | 0.002 | 0.0000022 |
|  | B3 | 14.89 | 30.68 | 0.03068 |  |  |  |
|  | B4 | 13.07 | 44.85 | 0.04485 |  |  |  |
|  | B5 | 13.89 | 59.84 | 0.05984 | 5.72141 | 0.007 | 0.0000068 |
|  | B6 | 13.5 | 74.44 | 0.07444 |  |  |  |
|  | B7 | 14.16 | 89.7 | 0.0897 | 5.72477 | 0.010 | 0.0000102 |
|  | B8 | 13.65 | 104.45 | 0.10445 | 5.72622 | 0.012 | 0.0000116 |
| 8 | A1 | 0 | 0 | 0 | 7.835029 | 0.000 | 0.0000000 |
|  | A2 | 0.02 | 1.12 | 0.00112 | 7.83605 | 0.001 | 0.0000010 |
|  | A3 | 7.78 | 10 | 0.01 | 7.83736 | 0.002 | 0.0000023 |
|  | A4 | 11.81 | 22.91 | 0.02291 |  |  |  |
|  | A5 | 10.71 | 34.72 | 0.03472 | 7.84042 | 0.005 | 0.0000054 |
|  | A6 | 5.31 | 41.13 | 0.04113 | 7.84069 | 0.006 | 0.0000057 |
|  | A7 | 12.22 | 54.45 | 0.05445 | 7.84258 | 0.008 | 0.0000076 |
|  | A8 | 9.05 | 64.6 | 0.0646 | 7.84364 | 0.009 | 0.0000086 |
|  | B1 | 0 | 0 | 0 | 5.721569 | 0.000 | 0.0000000 |
|  | B2 | 11.06 | 12.16 | 0.01216 | 5.7222 | 0.001 | 0.0000006 |
|  | B3 | 7.55 | 20.81 | 0.02081 | 5.72438 | 0.003 | 0.0000028 |
|  | B4 | 9.06 | 30.97 | 0.03097 | 5.72527 | 0.004 | 0.0000037 |
|  | B5 | 9.79 | 41.86 | 0.04186 | 5.72719 | 0.006 | 0.0000056 |
|  | B6 | 9.17 | 52.13 | 0.05213 | 5.72802 | 0.006 | 0.0000065 |
|  | B7 | 8.92 | 62.15 | 0.06215 | 5.72882 | 0.007 | 0.0000073 |
|  | B8 | 9.78 | 73.03 | 0.07303 |  |  |  |
| 9 | A1 | 0 | 0 | 0 | 6.510109 | 0.000 | 0.0000000 |
|  | A2 | 8.48 | 9.58 | 0.00958 | 6.511385 | 0.001 | 0.0000013 |
|  | A3 | 10.94 | 21.62 | 0.02162 | 6.512586 | 0.002 | 0.0000025 |
|  | A4 | 8.47 | 31.19 | 0.03119 | 6.513871 | 0.004 | 0.0000038 |
|  | A5 | 6.61 | 38.9 | 0.0389 | 6.514533 | 0.004 | 0.0000044 |
|  | A6 | 8.92 | 48.92 | 0.04892 | 6.514974 | 0.005 | 0.0000049 |
|  | A7 | 7.7 | 57.72 | 0.05772 |  |  |  |
|  | A8 | 10.94 | 69.76 | 0.06976 | 6.516401 | 0.006 | 0.0000063 |
|  | B1 | 0 | 0 | 0 | 5.72089 | 0.000 | 0.0000000 |
|  | B2 | 8.25 | 9.35 | 0.00935 | 5.72293 | 0.002 | 0.0000020 |
|  | B3 | 8.35 | 18.8 | 0.0188 |  |  |  |
|  | B4 | 8.84 | 28.74 | 0.02874 | 5.72615 | 0.005 | 0.0000053 |
|  | B5 | 8.01 | 37.85 | 0.03785 |  |  |  |
|  | B6 | 9.13 | 48.08 | 0.04808 |  |  |  |
|  | B7 | 9.05 | 58.23 | 0.05823 |  |  |  |
|  | B8 | 8.75 | 68.08 | 0.06808 | 5.73109 | 0.010 | 0.0000102 |
| 10 | A1 | 0 | 0 | 0 |  |  |  |
|  | A2 | 9.49 | 10.59 | 0.01059 | 6.50607 | 0.000 | 0.0000000 |
|  | A3 | 9.61 | 21.3 | 0.0213 | 6.50737 | 0.001 | 0.0000013 |
|  | A4 | 7.36 | 29.76 | 0.02976 |  |  |  |
|  | A5 | 9.43 | 40.29 | 0.04029 | 6.50938 | 0.003 | 0.0000033 |
|  | A6 | 7.24 | 48.63 | 0.04863 | 6.51067 | 0.005 | 0.0000046 |
|  | A7 | 9.35 | 59.08 | 0.05908 | 6.51182 | 0.006 | 0.0000057 |
|  | A8 | 9.72 | 69.9 | 0.0699 | 6.51228 | 0.006 | 0.0000062 |
|  | B1 | 0 | 0 | 0 | 5.72201 | 0.000 | 0.0000000 |
|  | B2 | 9.16 | 10.26 | 0.01026 | 5.72385 | 0.002 | 0.0000018 |
|  | B3 | 5.6 | 16.96 | 0.01696 | 5.72453 | 0.003 | 0.0000025 |
|  | B4 | 10.9 | 28.96 | 0.02896 | 5.72632 | 0.004 | 0.0000043 |
|  | B5 | 10.35 | 40.41 | 0.04041 | 5.72795 | 0.006 | 0.0000059 |
|  | B6 | 9.53 | 51.04 | 0.05104 | 5.72892 | 0.007 | 0.0000069 |
|  | B7 | 8.67 | 60.81 | 0.06081 |  |  |  |
|  | B8 | 5.56 | 67.47 | 0.06747 | 5.73084 | 0.009 | 0.0000088 |

## Camera Settings

SIM

| Shot | SIM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Camera | lens | aperture | Resolution | frame rate | exposure | Inter-frame | Gain | Delay |
|  |  | mm | f-stop | px | fps | us | us |  | us |
| 1 | SIM | 400 | 2.8 | 1360 x 1024 | 1,000,000 | 0.3 | 0.7 | 3 | 12.5 |
| 2 | SIM | 400 | 2.8 | 1360 x 1024 | 1,000,000 | 0.3 | 0.7 | 7 | 12.5 |
| 3 | SIM | 400 | 2.8 | 1360 x 1024 | 555,556 | 0.3 | 1.5 | 4 | 32.4 |
| 4 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.25 | 5 | 32.4 |
| 5 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.25 | 5 | 32.4 |
| 6 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.25 | 4 | 32.4 |
| 7 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.25 | 4 | 32.4 |
| 8 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.25 | 4 | 32.4 |
| 9 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.25 | 4 | 32.4 |
| 10 | SIM | 400 | 2.8 | 1360 x 1024 | 645,161 | 0.3 | 1.5 | 4 | 32.4 |

Phantom

| Phantom 2512 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camera | lens | aperature | resolution | frame rate | exposure | E Index | Filter | edr |
| 2512 | mm | f.stop | px | fps | us |  |  | us |
| 2512 | 135-400 | 11 | 128 x 32 | 1,000,000 | 0.261 | 80,000 | n/a | n/a |
| 2512 | 135-400 | 11 | 128 x 32 | 1,000,000 | 0.261 | 80,000 | n/a | n/a |
| 2512 | 135-400 | 11 | 128 x 32 | 1,000,000 | 0.261 | 80,000 | n/a | n/a |
| 2512 | 135-400 | 11 | 128 x 64 | 780,000 | 0.261 | 80,000 | n/a | n/a |
| TMX | 135-400 | 11 | 640 x 128 | 870,000 | 0.1 | 80,000 | n/a | n/a |
| TMX | 135-400 | 11 | 640 x 128 | 870,000 | 0.1 | 80,000 | n/a | n/a |
| TMX | 135-400 | 11 | 128 x 64 | 780,000 | 0.261 | 80,000 | n/a | n/a |
| 2512 | 135-400 | 11 | 768 x 64 | 870,000 | 0.261 | 80,000 | n/a | n/a |
| 2512 | 135-400 | 11 | 128 x 64 | 780,000 | 0.261 | 80,000 | n/a | n/a |
| 2512 | 135-400 | 11 | 128 x 64 | 780,000 | 0.261 | 80,000 | n/a | n/a |

## Camera Calibration Results

| Firing | Camera | Calibration Result m/px | Working |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | X1 | Y1 | X2 | Y2 | Size of Object |
|  |  |  | px | px | px | px | m |
| 1 | SIM |  |  |  |  |  |  |
| 1 | Phantom | 0.001190476 | 25 | 43 | 25 | 85 | 0.05 |
| 2 | SIM |  |  |  |  |  |  |
| 2 | Phantom | 0.001190476 | 25 | 43 | 25 | 85 | 0.05 |
| 3 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 3 | Phantom | 0.00125 | 20 | 22 | 20 | 62 | 0.05 |
| 4 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 4 | Phantom | 0.001381579 | 9 | 24 | 9 | 100 | 0.105 |
| 5 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 5 | Phantom | 0.001835059 | 53 | 244 | 58 | 301 | 0.105 |
| 6 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 6 | Phantom |  |  |  |  |  |  |
| 7 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 7 | Phantom | 0.0021 | 64 | 190 | 64 | 140 | 0.105 |
| 8 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 8 | Phantom | 0.001470588 | 53 | 60 | 53 | 26 | 0.05 |
| 9 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 9 | Phantom | 0.001561738 | 18 | 95 | 19 | 63 | 0.05 |
| 10 | SIM | 0.000199462 | 908 | 480 | 156 | 486 | 0.15 |
| 10 | Phantom | 0.001561738 | 18 | 95 | 19 | 63 | 0.05 |

## Camera VoD Workings

SIM

| Firing | Calibration | X | Y | Displacement | Cumulative Displacement | Frame | time | cumulative time | graph displacement | graph time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4A | 0.000199462 | 632 | 561 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 632 | 676 | 0.0229381 | 0.0229381 | 3 | 3.1E-06 | 3.1E-06 | 0.01146905 | 1.55E-06 |
|  | Frame Rate | 632 | 783 | 0.021342406 | 0.044280505 | 4 | 1.55E-06 | 4.65E-06 | 0.033609303 | 3.875E-06 |
|  | fps | 632 | 837 | 0.010770934 | 0.055051439 | 5 | 1.55E-06 | 6.2E-06 | 0.049665972 | 5.425E-06 |
|  | 645,161 |  |  |  |  |  |  |  |  |  |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 6.2E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.055051439 |  |  |  |  |  |  |  |  |  |
| 4B | 0.0001995 | 767 | 576 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 767 | 672 | 0.019148327 | 0.019148327 | 3 | 3.1E-06 | 3.1E-06 | 0.009574163 | 1.55E-06 |
|  | Frame Rate | 767 | 770 | 0.01954725 | 0.038695577 | 4 | 1.55E-06 | 4.65E-06 | 0.028921952 | 3.875E-06 |
|  | fps | 767 | 841 | 0.014161783 | 0.05285736 | 5 | 1.55E-06 | 6.2E-06 | 0.045776468 | 5.425E-06 |
|  | 645,161 |  |  |  |  |  |  |  |  |  |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 6.2E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0528574 |  |  |  |  |  |  |  |  |  |
| 5A | 0.0001995 | 583 | 555 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 583 | 737 | 0.036302036 | 0.036302036 | 3 | 3.1E-06 | 3.1E-06 | 0.018151018 | 1.55E-06 |
|  | Frame Rate | 583 | 820 | 0.016555324 | 0.05285736 | 4 | 1.55E-06 | 4.65E-06 | 0.044579698 | 3.875E-06 |
|  | fps | 583 | 892 | 0.014361245 | 0.067218605 | 5 | 1.55E-06 | 6.2E-06 | 0.060037983 | 5.425E-06 |
|  | 645,161 |  |  |  |  |  |  |  |  |  |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 6.2E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0672186 |  |  |  |  |  |  |  |  |  |
| 5B | 0.0001995 | 736 | 590 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 736 | 692 | 0.020345097 | 0.020345097 | 3 | 3.1E-06 | 3.1E-06 | 0.010172549 | 1.55E-06 |
|  | Frame Rate | 736 | 830 | 0.02752572 | 0.047870817 | 4 | 1.55E-06 | 4.65E-06 | 0.034107957 | 3.875E-06 |
|  | fps | 736 | 874 | 0.008776316 | 0.056647133 | 5 | 1.55E-06 | 6.2E-06 | 0.052258975 | 5.425E-06 |
|  | 645,161 |  |  |  |  |  |  |  |  |  |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 6.2E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0566471 |  |  |  |  |  |  |  |  |  |
| 6A | 0.0001995 | 606 | 598 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 606 | 724 | 0.025132179 | 0.025132179 | 3 | 3.1E-06 | 3.1E-06 | 0.012566089 | 1.55E-06 |
|  | Frame Rate | 606 | 792 | 0.013563398 | 0.038695577 | 4 | 1.55E-06 | 4.65E-06 | 0.031913878 | 3.875E-06 |
|  | fps | 606 | 827 | 0.006981161 | 0.045676738 | 5 | 1.55E-06 | 6.2E-06 | 0.042186157 | 5.425E-06 |
|  | 645,161 | 606 | 890 | 0.012566089 | 0.058242827 | 6 | 1.55E-06 | 7.75E-06 | 0.051959782 | 6.975E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 7.75E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0582428 |  |  |  |  |  |  |  |  |  |
| 6B | 0.0001995 | 766 | 581 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 766 | 722 | 0.028124105 | 0.028124105 | 3 | 3.1E-06 | 3.1E-06 | 0.014062052 | 1.55E-06 |
|  | Frame Rate | 766 | 805 | 0.016555324 | 0.044679429 | 4 | 1.55E-06 | 4.65E-06 | 0.036401767 | 3.875E-06 |
|  | fps | 766 | 808 | 0.000598385 | 0.045277814 | 5 | 1.55E-06 | 6.2E-06 | 0.044978622 | 5.425E-06 |
|  | 645,161 | 766 | 912 | 0.020744021 | 0.066021835 | 6 | 1.55E-06 | 7.75E-06 | 0.055649824 | 6.975E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 7.75E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0660218 |  |  |  |  |  |  |  |  |  |
| 7A | 0.0001995 | 766 | 581 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 766 | 722 | 0.028124105 | 0.028124105 | 3 | 3.1E-06 | 3.1E-06 | 0.014062052 | 1.55E-06 |
|  | Frame Rate | 766 | 805 | 0.016555324 | 0.044679429 | 4 | 1.55E-06 | 4.65E-06 | 0.036401767 | 3.875E-06 |
|  | fps | 766 | 808 | 0.000598385 | 0.045277814 | 5 | 1.55E-06 | 6.2E-06 | 0.044978622 | 5.425E-06 |
|  | 645,161 | 766 | 912 | 0.020744021 | 0.066021835 | 6 | 1.55E-06 | 7.75E-06 | 0.055649824 | 6.975E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 7.75E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0660218 |  |  |  |  |  |  |  |  |  |
| 7B | 0.0001995 | 790 | 549 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 790 | 642 | 0.018549941 | 0.018549941 | 3 | 3.1E-06 | 3.1E-06 | 0.009274971 | 1.55E-06 |
|  | Frame Rate | 790 | 720 | 0.015558015 | 0.034107957 | 4 | 1.55E-06 | 4.65E-06 | 0.026328949 | 3.875E-06 |
|  | fps | 790 | 744 | 0.004787082 | 0.038895039 | 5 | 1.55E-06 | 6.2E-06 | 0.036501498 | 5.425E-06 |
|  | 645,161 | 790 | 842 | 0.01954725 | 0.058442289 | 6 | 1.55E-06 | 7.75E-06 | 0.048668664 | 6.975E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 7.75E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0584423 |  |  |  |  |  |  |  |  |  |
| 9A | 0.0001995 | 409 | 408 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 409 | 610 | 0.040291271 | 0.040291271 | 3 | 3.1E-06 | 3.1E-06 | 0.020145635 | 1.55E-06 |
|  | Frame Rate | 409 | 661 | 0.010172549 | 0.050463819 | 4 | 1.55E-06 | 4.65E-06 | 0.045377545 | 3.875E-06 |
|  | fps | 409 | 767 | 0.021142944 | 0.071606763 | 6 | 3.1E-06 | 7.75E-06 | 0.061035291 | 6.2E-06 |
|  | 645,161 | 409 | 828 | 0.012167166 | 0.083773929 | 7 | 1.55E-06 | 9.3E-06 | 0.077690346 | 8.525E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 9.3E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0837739 |  |  |  |  |  |  |  |  |  |
| 10A | 0.0001995 | 550 | 528 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 550 | 702 | 0.034706342 | 0.034706342 | 3 | 3.1E-06 | 3.1E-06 | 0.017353171 | 1.55E-06 |
|  | Frame Rate | 550 | 739 | 0.007380084 | 0.042086426 | 4 | 1.55E-06 | 4.65E-06 | 0.038396384 | 3.875E-06 |
|  | fps | 550 | 807 | 0.013563398 | 0.055649824 | 5 | 1.55E-06 | 6.2E-06 | 0.048868125 | 5.425E-06 |
|  | 645,161 | 550 | 818 | 0.002194079 | 0.057843904 | 6 | 1.55E-06 | 7.75E-06 | 0.056746864 | 6.975E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 7.75E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0578439 |  |  |  |  |  |  |  |  |  |
| 10B | 0.0001995 | 686 | 522 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
|  |  | 686 | 704 | 0.036302036 | 0.036302036 | 3 | 3.1E-06 | 3.1E-06 | 0.018151018 | 1.55E-06 |
|  | Frame Rate | 686 | 739 | 0.006981161 | 0.043283197 | 4 | 1.55E-06 | 4.65E-06 | 0.039792616 | 3.875E-06 |
|  | fps | 686 | 807 | 0.013563398 | 0.056846595 | 5 | 1.55E-06 | 6.2E-06 | 0.050064896 | 5.425E-06 |
|  | 645,161 | 688 | 818 | 0.00223005 | 0.059076645 | 6 | 1.55E-06 | 7.75E-06 | 0.05796162 | 6.975E-06 |
|  | Total Time |  |  |  |  |  |  |  |  |  |
|  | s |  |  |  |  |  |  |  |  |  |
|  | 7.75E-06 |  |  |  |  |  |  |  |  |  |
|  | Total Disp |  |  |  |  |  |  |  |  |  |
|  | m |  |  |  |  |  |  |  |  |  |
|  | 0.0590766 |  |  |  |  |  |  |  |  |  |

Phantom

| Firing | Calibration | X | Y | Displacement | Cumulative Displacement | Frame | time | cumulative time | graph displacement | graph time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | m/pix | pix | pix | m | m |  | s | s | m | s |
|  | 0.0011905 | 9 | 32 | 0 | 0 | 28 | 0 | 0 | 0 | 0 |
|  |  | 9 | 38 | 0.007142857 | 0.007142857 | 29 | 0.000001 | 0.000001 | 0.003571429 | 0.0000005 |
|  | Frame Rate | 9 | 45 | 0.008333333 | 0.01547619 | 30 | 0.000001 | 0.000002 | 0.011309524 | 0.0000015 |
|  | fps | 9 | 53 | 0.00952381 | 0.025 | 31 | 0.000001 | 0.000003 | 0.020238095 | 0.0000025 |
|  | 1,000,000 | 9 | 61 | 0.00952381 | 0.03452381 | 32 | 0.000001 | 0.000004 | 0.029761905 | 0.0000035 |
|  |  | 9 | 67 | 0.007142857 | 0.041666667 | 33 | 0.000001 | 0.000005 | 0.038095238 | 0.0000045 |
|  | Total Time | 9 | 75 | 0.00952381 | 0.051190476 | 34 | 0.000001 | 0.000006 | 0.046428571 | 0.0000055 |
|  | s | 9 | 82 | 0.008333333 | 0.05952381 | 35 | 0.000001 | 0.000007 | 0.055357143 | 0.0000065 |
|  | 0.000013 | 9 | 85 | 0.003571429 | 0.063095238 | 36 | 0.000001 | 0.000008 | 0.061309524 | 0.0000075 |
|  |  | 9 | 96 | 0.013095238 | 0.076190476 | 37 | 0.000001 | 0.000009 | 0.069642857 | 0.0000085 |
|  | Total Disp | 9 | 102 | 0.007142857 | 0.083333333 | 38 | 0.000001 | 0.00001 | 0.079761905 | 0.0000095 |
|  | m | 9 | 106 | 0.004761905 | 0.088095238 | 39 | 0.000001 | 0.000011 | 0.085714286 | 0.0000105 |
|  | 0.102381 | 9 | 110 | 0.004761905 | 0.092857143 | 40 | 0.000001 | 0.000012 | 0.09047619 | 0.0000115 |
|  |  | 9 | 118 | 0.00952381 | 0.102380952 | 41 | 0.000001 | 0.000013 | 0.097619048 | 0.0000125 |
| 3 | 0.00125 | 8 | 35 | 0 | 0 | 24 | 0 | 0 | 0 | 0 |
|  |  | 8 | 37 | 0.0025 | 0.0025 | 25 | 0.000001 | 0.000001 | 0.00125 | 0.0000005 |
|  | Frame Rate | 8 | 41 | 0.005 | 0.0075 | 26 | 0.000001 | 0.000002 | 0.005 | 0.0000015 |
|  | fps | 8 | 45 | 0.005 | 0.0125 | 27 | 0.000001 | 0.000003 | 0.01 | 0.0000025 |
|  | 1,000,000 | 8 | 49 | 0.005 | 0.0175 | 28 | 0.000001 | 0.000004 | 0.015 | 0.0000035 |
|  |  | 8 | 55 | 0.0075 | 0.025 | 29 | 0.000001 | 0.000005 | 0.02125 | 0.0000045 |
|  | Total Time | 8 | 64 | 0.01125 | 0.03625 | 30 | 0.000001 | 0.000006 | 0.030625 | 0.0000055 |
|  | s | 8 | 66 | 0.0025 | 0.03875 | 31 | 0.000001 | 0.000007 | 0.0375 | 0.0000065 |
|  | 0.000012 | 8 | 69 | 0.00375 | 0.0425 | 32 | 0.000001 | 0.000008 | 0.040625 | 0.0000075 |
|  |  | 8 | 73 | 0.005 | 0.0475 | 33 | 0.000001 | 0.000009 | 0.045 | 0.0000085 |
|  | Total Disp | 8 | 78 | 0.00625 | 0.05375 | 34 | 0.000001 | 0.00001 | 0.050625 | 0.0000095 |
|  | m | 8 | 85 | 0.00875 | 0.0625 | 35 | 0.000001 | 0.000011 | 0.058125 | 0.0000105 |
|  | 0.07625 | 8 | 96 | 0.01375 | 0.07625 | 36 | 0.000001 | 0.000012 | 0.069375 | 0.0000115 |
| 4 | 0.0013816 | 9 | 49 | 0 | 0 | 18 | 0 | 0 | 0 | 0 |
|  |  | 9 | 59 | 0.013815789 | 0.013815789 | 19 | 1.28E-06 | 1.28205E-06 | 0.006907895 | 6.41E-07 |
|  | Frame Rate | 9 | 62 | 0.004144737 | 0.017960526 | 20 | 1.28E-06 | 2.5641E-06 | 0.015888158 | 1.923E-06 |
|  | fps | 9 | 67 | 0.006907895 | 0.024868421 | 21 | 1.28E-06 | 3.84615E-06 | 0.021414474 | 3.205E-06 |
|  | 780,000 | 9 | 72 | 0.006907895 | 0.031776316 | 22 | 1.28E-06 | 5.12821E-06 | 0.028322368 | 4.487E-06 |
|  |  | 9 | 78 | 0.008289474 | 0.040065789 | 23 | 1.28E-06 | 6.41026E-06 | 0.035921053 | 5.769E-06 |
|  | Total Time | 9 | 84 | 0.008289474 | 0.048355263 | 24 | 1.28E-06 | 7.69231E-06 | 0.044210526 | 7.051E-06 |
|  | s | 9 | 89 | 0.006907895 | 0.055263158 | 25 | 1.28E-06 | 8.97436E-06 | 0.051809211 | 8.333E-06 |
|  | 1.667E-05 | 9 | 94 | 0.006907895 | 0.062171053 | 26 | 1.28E-06 | 1.02564E-05 | 0.058717105 | 9.615E-06 |
|  |  | 9 | 99 | 0.006907895 | 0.069078947 | 27 | 1.28E-06 | 1.15385E-05 | 0.065625 | 1.09E-05 |
|  | Total Disp | 9 | 105 | 0.008289474 | 0.077368421 | 28 | 1.28E-06 | 1.28205E-05 | 0.073223684 | 1.218E-05 |
|  | m | 9 | 110 | 0.006907895 | 0.084276316 | 29 | 1.28E-06 | 1.41026E-05 | 0.080822368 | 1.346E-05 |
|  | 0.0967105 | 9 | 114 | 0.005526316 | 0.089802632 | 30 | 1.28E-06 | 1.53846E-05 | 0.087039474 | 1.474E-05 |
|  |  | 9 | 119 | 0.006907895 | 0.096710526 | 31 | 1.28E-06 | 1.66667E-05 | 0.093256579 | 1.603E-05 |
| 5 | 0.0018351 | 19 | 171 | 0 | 0 | -14 | 0 | 0 | 0 | 0 |
|  |  | 19 | 181 | 0.018350587 | 0.018350587 | -13 | 1.15E-06 | 1.14943E-06 | 0.009175294 | 5.747E-07 |
|  | Frame Rate | 20 | 193 | 0.022097033 | 0.04044762 | -11 | 2.3E-06 | 3.44828E-06 | 0.029399103 | 2.299E-06 |
|  | fps | 20 | 201 | 0.01468047 | 0.05512809 | -10 | 1.15E-06 | 4.5977E-06 | 0.047787855 | 4.023E-06 |
|  | 870,000 | 20 | 209 | 0.01468047 | 0.069808559 | -9 | 1.15E-06 | 5.74713E-06 | 0.062468324 | 5.172E-06 |
|  |  | 20 | 216 | 0.012845411 | 0.08265397 | -8 | 1.15E-06 | 6.89655E-06 | 0.076231265 | 6.322E-06 |
|  | Total Time | 20 | 224 | 0.01468047 | 0.09733444 | -7 | 1.15E-06 | 8.04598E-06 | 0.089994205 | 7.471E-06 |
|  | s | 20 | 232 | 0.01468047 | 0.112014909 | -6 | 1.15E-06 | 9.1954E-06 | 0.104674675 | 8.621E-06 |
|  | 1.609E-05 | 20 | 239 | 0.012845411 | 0.12486032 | -5 | 1.15E-06 | 1.03448E-05 | 0.118437615 | 9.77E-06 |
|  |  | 20 | 244 | 0.009175294 | 0.134035614 | -4 | 1.15E-06 | 1.14943E-05 | 0.129447967 | 1.092E-05 |
|  | Total Disp | 20 | 250 | 0.011010352 | 0.145045966 | -3 | 1.15E-06 | 1.26437E-05 | 0.13954079 | 1.207E-05 |
|  | m | 20 | 258 | 0.01468047 | 0.159726436 | -2 | 1.15E-06 | 1.37931E-05 | 0.152386201 | 1.322E-05 |
|  | 0.1744069 | 20 | 262 | 0.007340235 | 0.16706667 | -1 | 1.15E-06 | 1.49425E-05 | 0.163396553 | 1.437E-05 |
|  |  | 20 | 266 | 0.007340235 | 0.174406905 | 0 | 1.15E-06 | 1.6092E-05 | 0.170736788 | 1.552E-05 |
| 7 | 0.0021 | 92 | 90 | 0 | 0 | -18 | 0 | 0 | 0 | 0 |
|  |  | 92 | 96 | 0.0126 | 0.0126 | -17 | 1.28E-06 | 1.28205E-06 | 0.0063 | 6.41E-07 |
|  | Frame Rate | 92 | 101 | 0.0105 | 0.0231 | -16 | 1.28E-06 | 2.5641E-06 | 0.01785 | 1.923E-06 |
|  | fps | 92 | 105 | 0.0084 | 0.0315 | -15 | 1.28E-06 | 3.84615E-06 | 0.0273 | 3.205E-06 |
|  | 780,000 | 92 | 110 | 0.0105 | 0.042 | -14 | 1.28E-06 | 5.12821E-06 | 0.03675 | 4.487E-06 |
|  |  | 92 | 115 | 0.0105 | 0.0525 | -13 | 1.28E-06 | 6.41026E-06 | 0.04725 | 5.769E-06 |
|  | Total Time | 92 | 124 | 0.0189 | 0.0714 | -11 | 2.56E-06 | 8.97436E-06 | 0.06195 | 7.692E-06 |
|  | s | 92 | 128 | 0.0084 | 0.0798 | -10 | 1.28E-06 | 1.02564E-05 | 0.0756 | 9.615E-06 |
|  | 2.051E-05 | 92 | 132 | 0.0084 | 0.0882 | -9 | 1.28E-06 | 1.15385E-05 | 0.084 | 1.09E-05 |
|  |  | 92 | 137 | 0.0105 | 0.0987 | -8 | 1.28E-06 | 1.28205E-05 | 0.09345 | 1.218E-05 |
|  | Total Disp | 92 | 142 | 0.0105 | 0.1092 | -7 | 1.28E-06 | 1.41026E-05 | 0.10395 | 1.346E-05 |
|  | m | 92 | 147 | 0.0105 | 0.1197 | -6 | 1.28E-06 | 1.53846E-05 | 0.11445 | 1.474E-05 |
|  | 0.1575 | 92 | 152 | 0.0105 | 0.1302 | -5 | 1.28E-06 | 1.66667E-05 | 0.12495 | 1.603E-05 |
|  |  | 92 | 156 | 0.0084 | 0.1386 | -4 | 1.28E-06 | 1.79487E-05 | 0.1344 | 1.731E-05 |
|  | Velocity | 92 | 160 | 0.0084 | 0.147 | -3 | 1.28E-06 | 1.92308E-05 | 0.1428 | 1.859E-05 |
|  | m/s | 92 | 165 | 0.0105 | 0.1575 | -2 | 1.28E-06 | 2.05128E-05 | 0.15225 | 1.987E-05 |
| 9 | 0.0015617 | 7 | 28 | 0 | 0 | 19 | 0 | 0 | 0 | 0 |
|  |  | 7 | 36 | 0.012493901 | 0.012493901 | 20 | 1.28E-06 | 1.28205E-06 | 0.00624695 | 6.41E-07 |
|  | Frame Rate | 7 | 44 | 0.012493901 | 0.024987802 | 21 | 1.28E-06 | 2.5641E-06 | 0.018740851 | 1.923E-06 |
|  | fps | 7 | 48 | 0.00624695 | 0.031234752 | 22 | 1.28E-06 | 3.84615E-06 | 0.028111277 | 3.205E-06 |
|  | 780,000 | 7 | 56 | 0.012493901 | 0.043728653 | 23 | 1.28E-06 | 5.12821E-06 | 0.037481703 | 4.487E-06 |
|  |  | 7 | 62 | 0.009370426 | 0.053099079 | 24 | 1.28E-06 | 6.41026E-06 | 0.048413866 | 5.769E-06 |
|  | Total Time | 7 | 69 | 0.010932163 | 0.064031242 | 25 | 1.28E-06 | 7.69231E-06 | 0.058565161 | 7.051E-06 |
|  | s | 7 | 76 | 0.010932163 | 0.074963406 | 26 | 1.28E-06 | 8.97436E-06 | 0.069497324 | 8.333E-06 |
|  | 1.41E-05 | 7 | 82 | 0.009370426 | 0.084333831 | 27 | 1.28E-06 | 1.02564E-05 | 0.079648619 | 9.615E-06 |
|  |  | 7 | 90 | 0.012493901 | 0.096827732 | 28 | 1.28E-06 | 1.15385E-05 | 0.090580782 | 1.09E-05 |
|  | Total Disp | 7 | 96 | 0.009370426 | 0.106198158 | 29 | 1.28E-06 | 1.28205E-05 | 0.101512945 | 1.218E-05 |
|  | m | 7 | 103 | 0.010932163 | 0.117130321 | 30 | 1.28E-06 | 1.41026E-05 | 0.11166424 | 1.346E-05 |
| 10 | 0.0015617 | 9 | 47 | 0 | 0 | 22 | 0 | 0 | 0 | 0 |
|  |  | 9 | 58 | 0.017179114 | 0.017179114 | 23 | 1.28E-06 | 1.28205E-06 | 0.008589557 | 6.41E-07 |
|  | Frame Rate | 9 | 65 | 0.010932163 | 0.028111277 | 24 | 1.28E-06 | 2.5641E-06 | 0.022645195 | 1.923E-06 |
|  | fps | 9 | 73 | 0.012493901 | 0.040605178 | 25 | 1.28E-06 | 3.84615E-06 | 0.034358228 | 3.205E-06 |
|  | 780,000 | 9 | 79 | 0.009370426 | 0.049975604 | 26 | 1.28E-06 | 5.12821E-06 | 0.045290391 | 4.487E-06 |
|  |  | 9 | 85 | 0.009370426 | 0.05934603 | 27 | 1.28E-06 | 6.41026E-06 | 0.054660817 | 5.769E-06 |
|  | Total Time | 9 | 94 | 0.014055639 | 0.073401668 | 28 | 1.28E-06 | 7.69231E-06 | 0.066373849 | 7.051E-06 |
|  | s | 9 | 99 | 0.007808688 | 0.081210356 | 29 | 1.28E-06 | 8.97436E-06 | 0.077306012 | 8.333E-06 |
|  | 1.154E-05 | 9 | 106 | 0.010932163 | 0.09214252 | 30 | 1.28E-06 | 1.02564E-05 | 0.086676438 | 9.615E-06 |
|  |  | 9 | 111 | 0.007808688 | 0.099951208 | 31 | 1.28E-06 | 1.15385E-05 | 0.096046864 | 1.09E-05 |

## Heterodyne Velocimetry Chapman-Jouguet Pressure Workings

| Shot | Type | Pcj |  | Chaos Working |  |  |  |  |  |  |  |  |  |  | Hugoniot Working |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Chaos | Hugoniot | disc velocity | VoD | mass factor | ρ explosive | ρ tube | polytrophic ϒ | effective charge mass | mass of disc | radius of explosive | tube wall thickness | discount angle | Copper Density | Wall Velocity (Us) | 1st peak | 2nd peak | 3rd peak | 4th peak |  | particle velocity (Uo) |
|  |  |  |  | U (het-v) | D(pins) | z | ρ0 | ρw | ϒ(D - Pins & U - hetV) | Ceff | M | R | W | ϴ |  |  |  |  |  |  |  |  |
|  |  | Gpa | Gpa | m/s | m/s |  | kgm-3 | kgm-3 |  | kg | kg | m | m | rad | kgm-3 | m/s | m/s | m/s | m/s | m/s |  | m/s |
| 5 | DALEX | 29.9 | 27.7 | 4933.056 | 7701.712 | 2.650643 | 1628.00 | 9662.979 | 2.233064 | 1.77953E-06 | 0.00000035 | 0.000265465 | 0.002 | 0.017920609 | 9662.979 | 1730 | 1867 | 3065 | 3315 |  | 3315 | 1657.5 |
| 6 | DALEX | 27.7 | 35.8 | 4266.353 | 8013.12 | 2.82372 | 1608.64 | 9621.877 | 2.723937 | 1.76514E-06 | 0.0000003 | 0.000265465 | 0.002 | 0.01785182 | 9621.877 | 1680 |  | 2754 | 4427 |  | 4427 | 2213.5 |
| 7 | DALEX |  |  |  |  |  |  |  |  |  |  |  |  |  | 9602.937 | 1745 |  |  |  |  | 0 | 0 |
| 8 | McDAX |  | 28.2 |  |  |  |  |  |  |  |  |  |  |  | 9606.138 | 1518 | 1576 | 2316 | 3866 | 3375 | 3866 | 1933 |
| 9 | McDAX |  | 25.1 |  |  |  |  |  |  |  |  |  |  |  | 9606.138 | 1529 | 1593 | 2592 | 3421 |  | 3421 | 1710.5 |
| 10 | McDAX |  | 37.3 |  |  |  |  |  |  |  |  |  |  |  | 9606.138 | 1976 | 1685 | 2897 | 3932 | 3439 | 3932 | 1966 |

## Uncertainties Calculation

Chapman-Jouguet Pressure (het-v)

Uncertainty in time measurement = 5x10-10 s

Uncertainty in velocity measurement = 10ms-1

Uncertainty = √((10)2+(5x10-10)2) = 10ms-1

Velocity of Detonations (pins)

Uncertainty in displacement measurement

Precision of measurement tool = 0.0005m

Movement due to insertion of tube into stand = 0.005m

= 0.0005m + 0.005m = 0.0055m

Uncertainty in time measurement

Precision of data recorder = 0.0000005s

Uncertainty = √((0.0055)2+(0.0000005)2) = 0.0055ms-1

Chapman-Jouguet Pressure (pins VoD)

Uncertainty in the velocity of detonation = 0.0055ms-1

Uncertainty in the density of the energetic material = 1kgm-3

Uncertainty = √((0.0055)2+(1)2)  = 1GPa

Velocity of Detonation (cameras)

Uncertainty in displacement measurement

Precision of measurement tool = 0.05m

Movement due to insertion of tube into stand = 0.005m

= 0.05m + 0.005m = 0.055m

Uncertainty in time measurement

Precision of data recorder = 0.0000005s

Uncertainty = √((0.055)2+(0.0000005)2) = 0.055ms-1

Chapman-Jouguet Pressure (cameras VoD)

Uncertainty in the velocity of detonation = 0.055ms-1

Uncertainty in the density of the energetic material = 1kgm-3

Uncertainty = √((0.055)2+(1)2)  = 1GPa

Alternately known as Photon Doppler Velocimetry (PDV)

1.5 m tall and 600 mm wide by 600 mm deep