Nathanael Potts (2003504)

Date 03/2021

University of Wales Trinity Saint David

School of Engineering

Risk assessment technical report

g

Workshop and Practice (ACHA4011)

Contents

List of Tables	2

Introduction	2

1.0.	Research	3

1.1.	Health and Safety	3

1.2.	Tools employers can use to find risks	3

1.3.	The use of FMEA	3

1.4.	IMechE Code of Conduct	3

2.0.	Initial FMEA	4

2.1.	FMEA Results	4

2.2.	Reasons for results	4

2.2.1.	Brake calliper mounting bolts not torqued to correct level	4

2.2.2.	Incorrect brake disc size installed	4

2.2.3.	Brake pads installed in wrong orientation	5

2.2.4.	Brake line fitting not fully tightened	5

3.0.	Post-Mitigation FMEA	6

3.1.	Revised FMEA Results	6

3.2.	New Precautions in Place	6

3.2.1.	Brake calliper bolts	6

3.2.2.	Incorrect brake disc size	6

3.2.3.	Brake pads installed in wrong orientation	6

3.2.4.	Brake line fitting	7

4.0.	Conclusion	7

References	7

# List of Tables

Table 1 - Initial FMEA Results	4

Table 2 - Amended FMEA Results	6

# Introduction

In this report I will be investigating the risks to the engineers in the motorsport engineering workplace and how the risks can be reduced or so the workplace can become a safer environment. I will also be talking about the importance of all engineers understanding and spotting the risks in the workplace so that they can reduce the likelihood of an accident or worse, fatality. The aim of this report is to run an FMEA test of all the risks in the motorsport workplace and then find ways to reduce the risk of harm to ultimately make the workplace safer for the engineers.

# Research

## Health and Safety

In an motorsport engineering workshop, there are a lots of health and safety precautions that a company has to follow to minimise the risk of harm against any of their employees. In the workshop there are many factors that can harm someone from hazardous substances to heavy objects. The health and safety officers job is to ensure that the right precautions are in place to minimise the risk of harm to employees. Companies have to keep to specific Health and Safety legislations to keep their employees safe. If the company’s don’t keep to the legislations, then this can cost them with hefty fines, legal actions and the possibility of imprisonment (Martinelli, 2017).

## Tools employers can use to find risks

For an employer there are a few tools that they can use to help find and prevent risks in the workplace, FMEA being one of them. Another one very commonly used is a standard risk assessment. On the HSE website, they recommend that the minimum you should do is; identify what could cause injury/illness, decide what risk there is to the employees, and how to control/eliminate the risk (HSE, n.d.).

## The use of FMEA

One way a health and safety officer can test how safe a workplace is by running a ‘Failure Mode and Effects Analysis’ (FMEA). Porsche engineering described FMEA up by saying, “Failure Mode and Effects Analysis is an established instrument for identifying and correcting errors or potential sources of errors in the design of products and the associated manufacturing and assembly processes” (Porsche Engineering, n.d.). When running an FMEA test, each risk will get its own RPN number; RPN being “a measure used when assessing risk to help identify critical failure modes” (FMEA - FMECA, n.d.).

## IMechE Code of Conduct

The IMechE is an institution for engineers providing recourses, training and many other applications for engineers. The IMechE has a specific ‘code of conduct’ for all engineers to abide by. The code of conduct is there to not only protect the engineers but also the members of public. The code of conduct has 4 principles (IMechE, n.d.):

- Practice competently and maintain up-to-date knowledge and skills

- Act with integrity and respect for others

- Promote sustainability

- Exercise engineering

# Initial FMEA

## FMEA Results

Table 1 - Initial FMEA Results

In Table 1 below, I have presented the results obtained from the FMEA test I ran in the motorsport workshop. I ran each potential failure and produced an RPN number which tells me the extent of the risks. I then looked into how I could reduce these risks to make the workplace safer but also the final product safer for customers.

## Reasons for results

### Brake calliper mounting bolts not torqued to correct level

The first of the 4 tests I ran was the calliper bolts being fastened to the correct torque level. First of all the Likelihood of occurrence. I gave a score of 7 as I felt like it was very likely that the bolts weren’t being tightened as well as they could be. For severity I gave an 8. This is because if the bolts were to vibrate loose under load and time, then eventually there will be calliper failure which could result in an accident. In motorsport, when a car is travelling at 100+mph, the last thing the driver wants is for a brake failure. This can cause serious damage. Lastly, the likelihood of detection. For this I put 5 as it’s not possible to check the correct torque of the bolt by human feel. You need to use a torque wrench with torque ratings on it. In total, the RPN number I got out of the test was 280 which is relatively high. With the right actions, this can be hugely reduced.

The mitigation actions I would put into place would for the mechanics to use a torque wrench that would tighten the bolt to the correct torque and prevent the mechanism from over-stretching the bolt to the point of breaking (DoItYourself, 2010). This would make sure that the bolt had been tightened correctly to minimise any failure. Once the bolt has been tightened the subassembly would go through quality control where another engineer will make sure that the bolt had been tightened to the correct torque.

### Incorrect brake disc size installed

The likelihood of this happening I put was 6 as it is a possibility that the wrong brake disc can be installed. However, the likelihood of this is slightly smaller than the brake calliper bolts not torqued correctly as it’s less likely to frequently install the wrong disc size on the sub-assembly. Although it can still be done so new precautions need to be put in to reduce the likelihood to make sure it doesn’t happen at all. The severity is very high as having the wrong disc size on the car could potentially cause a huge accident. When the brakes applied at high speed, the driver trusts that the brakes will slow the velocity of the car down. Following on from this, the likelihood of detection I put was 6. This is because the company only use few amounts of brake discs so it should be relatively obvious to the engineer to spot the wrong brake pad on the car. As well as this, when the car is out for its shakedown test before going racing, the driver should be able to feel that the brakes aren’t cooperating as well as they should. In total the RPN number I got out of the test was 324 which is very high so the risk needs to be reduced to make the car and the environment safer.

The mitigation actions that I am putting into place is that when the subassembly is in manufacture, the jig only allows for the correct brake size to be installed. Before the brake disc goes onto the sub-assembly however, the measurements are checked including; the diameter and thickness. After the sub-assembly has been complete, the assembly will go through quality control where all measurements and details noted in drawings will be checked by another personal.

### Brake pads installed in wrong orientation

The likelihood of occurrence I thought was quite high as it’s not too obvious on which way round the pad should go. Therefore, this can make this potential failure mode one that can occur often especially as the pads will be changed on the cars after each race. The severity of this potential failure is very high as this could cause severe damage. If the friction material is on the wrong side then there would be no friction material in contact with the brake disc meaning that the result of the brakes being applied could cause severe damage to the driver or others around. The likelihood of detecting this is very low at a score of 4. This is because the likelihood of detecting the pad failure before the car going to track is very unlikely. When the car goes to track however, the driver will quickly work out there is an issue with the brakes. This would be very dangerous for the driver and others on and off track. Finally, the RPN number taken from this test was 252. Although it’s not as high as the other tests, the severity is very high which still means that there’s still a high risk. This risk needs to be reduced so that the likelihood of this failure is minimal.

The mitigation actions I thought were necessary are; a note on the assembly drawings which will note to the engineer that’s assembling the calliper to make sure and double check the orientation of the brake pad, as well as the correct training for the engineer so he knows how to correctly assemble the brake assembly and how to check all details. This will reduce the likelihood of this failure massively and hopefully protect the driver/engineers from harm.

### Brake line fitting not fully tightened

The likelihood of occurrence to this potential failure is very high due to the brake line fitting being a small aspect of the assembly. As the brake line fitting is relatively small, the chances of the fitting not being fully is very high. This could mean that under heavy braking, the brake line could fail resulting in the brake assembly failure. This could cause some serious harm hence the severity again is very high at a rating of 9. Lastly, the likelihood of detection is low. As it’s a small part of the assembly but very also very crucial, it is very likely that the failure won’t be detected but if the engineer does check the fitting, they would be able to tell that it’s not fully tightened. This would mean that the engineer could solve this fault. Overall, the RPN number for this failure mode was very high as it could cause some serious damage.

The mitigation actions that I am going to put in place to reduce the risk are; the first being a note on the assembly drawing notifying the engineer to check and double check the fastening, and the second being the engineer having a torqued spanner/wrench so the engineer knows when the fitting is tightened enough.

# Post-Mitigation FMEA

## Revised FMEA Results

Table 2 - Amended FMEA Results

In Table 2 below, I have presented the revised FMEA test results where I have ran the FMEA test with the new mitigation actions to generate a new RPN number to test the safety.

## New Precautions in Place

### Brake calliper bolts

The new precautions I put in place reduce the RPN number dramatically resulting in a safer environment. The likelihood of occurrence has now dropped massively to 1 as the precautions ensure that the potential failure mode won’t occur. Even though the severity is still relatively high, the chances are very low of anything happening. The chances of the engineer detecting the brake calliper bolts aren’t torqued right is very high meaning that the engineer is able to ensure the brake calliper assembly is safe not only for the engineers but also for the driver using the car. The RPN number as a result is very low meaning that the precautions have reduced the risk of harm massively.

### Incorrect brake disc size

The new precautions I put in place for the brake disc size again reduce the risk of harm. Again the likelihood of occurrence has improved massively down to 2. There is still a slim chance of the failure occurring but it’s a massive improvement than before. Yet the severity is still high, the chances of detecting the failure is still very high so it’s unlikely to happen. Therefore, the RPN number is again much lower than before as the new precautions put into place reduces the risk.

### Brake pads installed in wrong orientation

The new precautions put into place reduce the likelihood of the occurrence as the engineer has the correct tools to not get the brake pad orientation wrong. This also means that the likelihood of detection has been increased due to the tools allowing the engineer to know what orientation the brake pad should installed in. The severity is still relatively high because if the component was to fail, this could cause a serious accident. Overall, the RPN number has been reduced due to the new precautions.

### Brake line fitting

Finally, with new precautions in place for the brake line fitting, the likelihood of occurrence has been reduced. This means that the brake line fitting will be fully tightened following the new precautions minimising the chances of failure as well as making it more obvious to the engineers to detect when the brake line isn’t fully tightened. This means the chances of a failure occurring is very small so an accident, that could be highly severe, is less likely to happen. This again results in a reduced RPN number making the product safer.

# Conclusion

Overall, FMEA is a great tool to use when a company wants to reduce potential failures that can cause risks of harm others. It’s also very easy to use and very effective. It allows the company to improve the quality of their product while making their product safer for the consumers or surrounding environment. FMEA is particularly effective within the motorsport industry as the manufacturer can make the car safer. If the failures are reduced, then the less likely an accident will occur which could cause serious harm. As well as this, the reliability is improved due to the unlikely chance of a failure. This means that the car is more likely to go on and finish the race increasing chances of success.

When running the FMEA test, not only was it a quick and easy process, I was able to place suitable precautions in place to help minimise the risk of potential failure. I found that the precautions I put in place reduced the likelihood of occurrence and detections massively. The severity stays relatively high but due to the occurrence and detection being so much lower, the risk is massively reduced. This as a result makes the car safer so when it’s on track, the likelihood of an accident is very minimal. All precautions I put in place aren’t expensive and easy to use including a torqued wrench or even notes on drawings. These all make a huge difference when it comes to safety.

All in all, the FMEA test is a very valuable tool to not only improve the safety of the car but also makes the car more reliable increasing the chances of success. Finding a way of reducing the risks was key for safety and FMEA is definitely a useful/brilliant tool to use.

# References

DoItYourself. (2010, June 06). Retrieved from Doityourself: https://www.doityourself.com/stry/6-types-of-torque-wrenches-explained#:~:text=%206%20Types%20of%20Torque%20Wrenches%20Explained%20,use%20with%20couplings%20on%20soil%20pipes.%20More

FMEA - FMECA. (n.d.). FMEA RPN. Retrieved from FMEA - FMECA: https://fmea-fmeca.com/fmea-rpn.html#:~:text=Risk%20Priority%20Number%20(RPN)%20is%20a%20measure%20used,similar%20to%20the%20criticality%20numbers%20used%20in%20Mil-Std-1629A.

HSE. (n.d.). Retrieved from Health and Safety Executive: https://www.hse.gov.uk/simple-health-safety/risk/index.htm

IMechE. (n.d.). Code of Conduct Explained. Retrieved from IMechE: https://www.imeche.org/about-us/imeche-governance/governance-and-finance-reviews/code-of-conduct-explained

Martinelli, K. (2017, September 23). What are the consequences of Poor Health and Safety Procedures? Retrieved from High Speed Training: https://www.highspeedtraining.co.uk/hub/consequences-of-poor-health-safety-procedures/

Porsche Engineering. (n.d.). FMEA and Functional Safety. Retrieved from Porsche Engineering: https://www.porscheengineering.com/peg/en/services/supportingservices/fmea/

University of Wales Trinity Saint David. (2018). Technical Report Guidelines.

University of Wales Trinity Saint David. (2020). Harvard Referencing Handbook.