University of Wales Trinity Saint David

Wales Institute of Science & Art

Kinetic Energy Recovery Systems

Alternative Propulsion Systems (ACHA5010)

Bleddyn Williams

2104564

May 2023

Contents

1	Introduction	2

2	Vehicle Selection	3

2.1	Energy Recovery Methodology	3

2.2	Results	4

2.3	Evalution of Results	5

3	Kinetic Energy System Recovery Selection	6

3.1	Electronic KER System	6

3.2	Electro-mechanical KER System	6

3.3	Mechanical KER Systems	7

3.4	KERS Decision	7

4	Audi A3 with Electro-Magnetic KERS	8

5	Material Selection	9

6	Manufacturing Processes	10

8	Conclusion	11

9	Reference List	12

# Introduction

During the evolution of the Internal Combustion (ICE) engine during times of economic and environmental consciousness, certain systems have been developed to reduce vehicle emissions, improve fuel economy and develop a system that utilises, and recovers spent energy. Since the most inefficient stage of the cycle being during idling moments, where an increase of pumping losses occur due to a closed throttle and greater vacuum in the inlet side of the engine, the stop start system was developed using absorbent glass matt batteries to reduce the idling stages. As a result, the vehicle ICE will disengage, preventing the engine from running at the 800RPM of idle, and turn off. However, the engine will be primed to restart instantly following certain input, such as the release if a brake or engagement of the power steering by moving the steering wheel to minimise lag between driver input and vehicle response. Another development that will be further explored is the utilisation of the kinetic energy the vehicle can produce during stages of decelerations. While the Kinetic Energy Recovery Systems (KERS) that enable such a task reduce usage and prolong life cycle of the friction brake and discs, this system will not replace friction brakes due to limitations in pressure, rate of tolerable retardation as well as the mechanical reassurance that comes with friction brakes. Rather these systems have greater potential in recovering already spent energy to accelerate the vehicle during the deceleration stage.

# Vehicle Selection

Firstly, the vehicle I’ve selected to analyse and propose a suitable KERS system for is my own vehicle, a 2015 (8V) Audi A3 Sportback. I selected this vehicle, not just because its my own, but to gain an understanding of the capability of the common hatchback vehicles that share similar power range and Aerodynamic profile. The vehicle specifications are as follows:

- Kerb Mass – 1410 Kg

- Test Mass – 1600 Kg

- Frontal Area – 2.8m^2

- Rolling Resistance coefficient – 0.015

- Aerodynamic Drag Coefficient – 0.29

Using these dynamic characteristics, calculations of rolling resistance, power and aerodynamic resistance can be used in addition to the acceleration and deceleration data to produce a Worldwide Harmonised Light Vehicle Duty Cycle (WLTP) to produce Tractive power, Braking power and the frictional losses that occur between the two during the cycle. This information is crucial to evaluate how appropriate the vehicle is for a KERS system and if so, which system would be appropriate.

## Energy Recovery Methodology

The Methodology used to generate the accumulative Tractive and Braking power is as follows:

- Record acceleration, deceleration, and velocity data during 4 different velocity drive cycles; low, medium, high, very high

- During this, ensure the speed as well as the time is recorded.

- Also record periods of idle

- Following the data acquisition, log into a spreadsheet

- Calculate the forces the vehicle will over come during the different stages of duty cycle; acceleration, constant speed, deceleration.

- Calculate the relevant forces during the acceleration stage.

  - Acceleration resistance (N) = mass * acceleration

  - Aerodynamic Resistance (N) = 0.5 * Density of Air * Frontal Area *coefficient of drag * velocity2

  - Rolling Resistance (N) = test mass * gravity * coefficient of rolling resistance

    - Following these calculations, the force required to accelerate the vehicle can be calculated.

- The next stage of the duty cycle to be calculated is the deceleration stage:

  - Total force = Deceleration Force – (aero resistance + rolling resistance)

  - Potential energy recovery = total force * deceleration velocity

  - Energy recovered = PE recovery * KERS efficiency

During these calculations, an understanding of the power output (positive) is produced as tractive effort, and power output (negative) as Braking effort. This will be necessary to scope the potential for energy recovery as well as the KERS system appropriate to meet the needs of the vehicle.

## Results

Following the calculation of accumulative Traction ad Braking power over a period of 30 minutes. During this period extensive data on the 4 speed cycles were collected for a data plot to be as accurate as possible. As shown in the figure 2.1 below, there is a difference in output between the potential energy of tractive energy and the final kinetic energy accumulated under braking.

Figure ‎2.1 - Traction and Braking energy output per second (Williams, 2023)

This difference only increases with time, his is due to losses from the previously mentioned resistive forces as well as a loss due to heat from the brakes as well as smaller losses from seals and bearings. A greater representation of these losses are shown in figure 2.2 below.

Figure ‎2.2 - Power Losses per second (Williams, 2023)

The figure 2.2 shows across a period of 30 minutes a consistent gradient regarding the loses between the Tractive and Braking power. However, Figure 2.1 shows there is still great potential and sufficient kinetic energy (up to 0.8 mWh) to recover during deceleration and use again for another acceleration cycle.

## Evaluation of Results

Following the calculations of the forces that happen during a drive cycle, I have found limitations in the results. An example of these limitations is the final tractive and braking power outputs. The outputs of 1.4 and 0.8 mWh respectively are much higher than I had anticipated following a 30-minute duty cycle. Following research, I was unable to confirm or deny these suspicions, however the trend and gradients of these lines is consistent with that of this vehicle typical WLTP plots. As a result, the upcoming recommendations will be suitable from a theoretical aspect but possibly not from that of a practical perspective.

# Kinetic Energy System Recovery Selection

When selecting a suitable KERS system, considerations must be made regarding the life cycle of the system;

- Does it follow the life cycle trends of other powertrain components currently on the vehicle.

- Suitability for the power input and output

- System efficiency

- Ability to package the modifications necessary to introduce system.

- Compatibility with existing powertrain and driveline components

- Necessary shielding required to incorporate system (Electromagnetic noise)

- Noise vibrations from KERS system (mechanical flywheel)

- Existing or capability to introduce possible battery storage for the system in question.

## Electronic KER System

In the case if an electronic KER system, the rotational force of a decelerating vehicle is captured by an electric motor / generator unit (MGU) which is mounted to the engines crankshaft. The generator unit takes this rotational kinetic energy and converts it into electrical energy which is stored in batteries. The obstacle when selecting this type of KERS is the energy storage in the batteries and which type of batteries to use. Typically, the battery of choice is the lithium- ion battery. However, with battery technology developing this could change very quickly. A mechanical problem which must be overcome with the battery accommodation is the cooling of these battery packs, this may lead to an addition of cooling ducts and subsequent body panel modifications as a result to package the system and subsequent ducts.

## Electro-mechanical KER System

Unlike the electronic KER system, the energy recovered in an Electro-mechanical system isn’t stored in batteries. Rather, this system uses a mechanical flywheel to store the energy kinetically. An obvious advantage this provides is its packability. Usually coming in the form of a small flywheel, that can turn to speeds of up to 160,000 RPM, the flywheel acts as an electro-mechanical battery. However, considerations must be made regarding aerodynamic losses from the spinning flywheel, to overcome this problem the flywheel is stored in a vacuum when possible. The composition of the flywheel is that of fibres and metal particles, this is important as it acts as a magnet when reaching speeds between 50,000 – 160,000 RPM. As a result, it performs similarly to the previously mention MGU of the electronic KER System. the functionality of the flywheel is as follows, as the flywheel spins it induces a voltage to the MGU typically attached to the gearbox, due to its magnetic composition. As a result, it produces energy from the road to the gearbox to be used when needed or in motorsport applications, at the click of a button. Another advantage of this system is its versatility, being capable of coupling with batteries as well as flywheels to store energy together.

## Mechanical KER Systems

The final consideration for a KER System is that of a purely mechanical set up. This has a flywheel like the previously mentioned Electro-mechanical system, however this system doesn’t have any MGUs. Instead, it uses a transmission to transmit the drive to and from the flywheel. The kinetic energy built up as the car decelerates instead gets transmitted via various gears and shafts. While this system won’t need should sensors and electronic devices from the possible back emf and electronic noise that comes with the other two systems, it incurs mores losses due to its complete mechanical nature. Friction and heat losses occur as well as its poor efficiency at energy storage. Furthermore, the problem of continuous change in speed ratio between road wheels and flywheel has to be overcome by introducing a Constant Variable Transmission (CVT) which is managed by an electro-hydraulic control system. typically, a considerably heavy type of transmission.

## KERS Decision

Due to the Kinetic energy potential of the vehicle, the potentially changing scope of battery technology as well as the package constraints that currently exist on the vehicle. the versatile and efficient Electro-mechanical KER System is the system of choice for this vehicle. due to the vehicle being on the heavier end of its respective vehicle class the addition weight of the mechanical KERS made the system unviable. Furthermore, due to the Vehicle already having limited space at the driveline and power units, the electronic system was also not a viable option. Since the Electro-mechanical is versatile enough to house a small flywheel, which will be manageable to package as well as the ability to incorporate more energy storage with the use of Lithium-ion batteries or future developed batteries without compromising the system, this was the suitable choice for this vehicle.

# Audi A3 with Electro-Magnetic KERS

Following the calculation of the kinetic energy recoverable following the resistive forces lost and the accountability of lost power between tractive and braking forces, figure 4.1 below represents the power available (in kWh) to recover in order to use during an acceleration cycle.

Figure ‎4.1 - Electro-Mechanical Energy Recovery with Audi A3

As stated previously I have some scepticism on the accuracy of the data from these calculations however, following research the trends and gradients are accurate to those of active KERS systems.

Following the incorporation of this KERS system the vehicle drivability and characteristics will be changed and a period of familiarisation is to be expected.

The most noticeable characteristic will be the retardation during acceleration, this is a delicate configuration electronic and hybrid vehicle manufacturers undertake with every model containing a KER system. too much retardation and the vehicle will be unsafe and undrivable due to the significant torque changes under deceleration, however, too little retardation and the system is inefficient. Missing out on the potential energy available to recover, leading to more energy spent and a reduction in potential fuel economy.

# Reference List

Weight, Frontal area, - https://www.car.info/en-se/audi/a3/8v-82129/specs

Rolling resistance - https://www.engineeringtoolbox.com/rolling-friction-resistance-d_1303.html

Aero Drag Coefficient - https://www.audi-mediacenter.com/en/more-dynamic-than-ever-before-the-new-audi-a3-sportback-and-the-new-a3-sedan-12974/space-concept-and-body-12980

Electro-Mechanical Spec - https://mechguru.com/machine-design/kinetic-energy-recovery-system-kers-works/#:~:text=Mechanical%20KERS%20have%20a%20greater,required%20or%20in%20straight%20track.

# Manufacturing Processes

# Conclusion

# Reference List