| LOCATION Swansea | LLEOLIAD abertawe |
| --- | --- |
| LEVEL 4 / Foundation yr | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| May 2022 | Mai 2022 |

MODULE CODE: ACCA4021

Introduction to Computer Systems

TIME:  2 Hours

answer all questions

Instructions for students:

This is a timed on-line exam.

As it is ‘open book’ ensure you show all calculations/methods used in your answers.

Create a single word document in which you should answer your questions. If you need to produce any diagrams or sketches, you can do this with whatever method suits you best – e.g. use a sketching program, or manually draw it on a piece of paper, then use your phone to take a picture of it. Then Copy/paste any images into your word document.

Upload your finished word document to the link provided on moodle within the specified time limit. It is also suggested you also immediately email the document to your tutor as a backup or in the case of any moodle issues.

ANSWER ALL FIVE QUESTIONS

(total marks available = 80, final mark is multiplied by 1.25 to obtain a %)

QUESTION ONE  (20 marks total)

- Provide the name for each of the following symbols (5 marks)

- AND Gate

- OR Gate

- NOT Gate

- XOR Gate

- NAND Gate

- Give a representation of the truth table for an AND gate given the inputs A and B (5 marks)

| Inputs | Outputs |
| --- | --- |
| A B |  |
| 0 0 | 0 |
| 0 1 | 0 |
| 1 0 | 0 |
| 1 1 | 1 |

- Indicate the output at each of the stages labelled in Figure 1. Inputs A and C are at logic level 1 (4 marks)

E

D

Figure 1

The OR Gate indecates that if either of the inputs are given a supply there will be an output from this part of the circuit. The AND Gate indecates that both of the inputs are required to be on for there to be an output, if only one of the inputs are supplied a voltage there will be no output.

Iv) A domestic light can be illuminated by two different switches. Each of the switches may turn the light on independently of one another, not dissimilar to a stairs light. Represent this scenario using a logic circuit (6 marks)

| Switch 1 | Switch 2 | Lamp Position |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

QUESTION Two  (20 marks total)

Attempt all of the following. Do not simply use an Electronic Calculator and just present the final answer. Write down and show all your manual calculations in your exam book.

- Convert the following binary numbers to base 10 (2 marks each)

- 1101 0111 = 215

- 0010 0101 = 37

- Convert the following hex numbers to binary (2 marks each)

- 6F = 01101111

- EA =11101010

- Convert the following decimal numbers to Binary (2 marks each)

- 84 = 1010100

- 246 = 11110110

- Convert the following Hex numbers to decimal (2 marks each)

- A1 = 161

- F5 = 245

- How many unique values can be represented by a 6- bit number ? (2 marks)

6            64                                            There is 64 Values

7            128

8            256

9            512

- Using the base 2 number system write the number of bytes in 1 Kilobyte (2 marks)

Because computers employ binary (base two) logic rather than a decimal (base ten) system, a kilobyte (KB) equals 1,024 bytes, not 1,000 bytes as one might think.

QUESTION THREE (20 marks total)

Give a typical, cost effective specification for  a typical PC that could be used for 3D animations and software development.  Provide details for:

- A suitable CPU =

- What is a sensible amount of RAM = At least 32GB. You might want to go with 64GB of RAM if you sculpt or work with high-poly models, utilise a lot of huge textures, or have complicated scenes with thousands of objects. For many people who are just getting started with 3D, 16 GB of RAM may be plenty, but you will shortly outgrow it.

- What type and size of non-volatile storage would be suitable = RRAM, Easy fabrication, a simple metal-insulator-metal (MIM) structure, high scalability, nanosecond speed, extended data retention, and compatibility with existing CMOS technology are all benefits of RRAM.

- The input/output features/ports needed =

- The features a suitable graphics card would require. =

For all of the above areas, you should justify your choice with reasoning and where possible, explain why you chose that particular option, over the alternatives.

QUESTION FOUR (20 marks total)

a)What do the terms GPU and CPU stand for? Explain what they are used for in the context of a typical PC. (6 marks)

A graphics processing unit (GPU) is a specialised processor that was created to speed up the rendering of visuals. GPUs can handle a large amount of data at once, making them ideal for machine learning, video editing, and gaming.

Central Processing Unit (CPU) is the part of a computer system that governs how instructions are interpreted and executed.

b) Depict the  Von Neumann architecture, showing the three main busses. Also provide and explanation of the function of each bus (8 marks)

Address Bus - Memory addresses are sent from the CPU to other components such as main storage and input/output devices through the address bus. The address bus is a one-way street.

Data Bus - The data bus transports data from the CPU to other components. The data bus is a two-way system.

Control Bus - The control bus transports control signals from the CPU to the rest of the system. The clock's pulses are also carried on the control bus. The control bus is a one-way system.

c)In the context of different programming languages, what is the difference between a Compiled language and an interpreted language. Give one example of each. (6 marks)

The output of the interpreting or compiling procedure is what distinguishes an interpreted from a compiled language. An interpreter creates a result from a programme, whereas a compiler creates an assembly language programme.

An example of a Complied language could be C, C++, C#, CLEO, etc.

An example of an interpreted language could be something like JavaScript, Python, etc.