James Hunt

PIC 1 Code

main.c

//Author J.Hunt

//ACDA5018 Practical

//9th Jan 2025

//PIC 1

#include <main.h>

#include <2408.C>

#define LCD_ENABLE_PIN PIN_B2

#define LCD_RS_PIN PIN_B0

#define LCD_RW_PIN PIN_B1

#define LCD_DATA4 PIN_B4

#define LCD_DATA5 PIN_B5

#define LCD_DATA6 PIN_B6

#define LCD_DATA7 PIN_B7

#include <lcd.c>

/***********Globals**************/

void clear_i2c_mem(unsigned int data);

void ep_w(char,char);

char read_buttons(void);

char ep_r(char );

unsigned int value=0x00;

char buts=0;

char old_buts=0;

char mem_read=0;

/**************PIC 1*************************/

void main()

{

setup_adc_ports(NO_ANALOGS, VSS_VDD);

init_ext_eeprom();

lcd_init();

while(TRUE)

{

/*printf(lcd_putc,"\f welcome to the");

lcd_gotoxy(1,2);									// display on line 2

printf(lcd_putc," for Distributed ");

while(true);

//TODO: User Code*/

buts=read_buttons(); 			//call read buttons function to get button input

if(old_buts!=buts) 			//runs this sections only if the value changes

{

printf(lcd_putc,"\f");			//clears LCD

printf(lcd_putc,"Button%3u",buts); 	//displays value

}

old_buts=buts; 				//writes the values to be the same

ep_w(0x00,buts); 				//writes the value to mem  slot 00

/*****read old value stored by pic2****/

mem_read = ep_r(0x01); 			//read from mem location

lcd_gotoxy(1,2); 				//move LCD to line 2

printf(lcd_putc,"Mem%3u",mem_read); 	//displays value read from mem location

}

}

/****************functions*******************/

char read_buttons(void)

{

char value = 0; 				// used to store button inputs

while (!input_a()); 				// waits for button press

value = input_a(); 				// reads bank a

delay_ms(20);	  				// timer for button release

return (value);	  				// returns value to main loop

}

void ep_w(char where,char what)

{

while(!input(hs));				// wait for mem chip to be available

output_low(hs);					// set hs line low

i2c_start();					// issue i2c start pulse

i2c_write(0xA0);				// issue chip type and number

i2c_write(where);				// send the address to write to

i2c_write(what);				// send the data to be written

delay_ms(20);					// wait for chip

i2c_stop();					// issue an i2c stop pulse

delay_ms(20);					// allow i2c chip to store data

input(hs);				// release hs line

}

char ep_r(char where)

{

char data=0;

while(!input(hs));				// wait for mem chip to be available

output_low(hs);					// set hs line low (mem busy)

i2c_start();					// issue i2c start pulse

i2c_write(0xA0);				// issue chip type and number

i2c_write(where);				// send address to be read from

delay_ms(20);					// wait for chip to set its for read

i2c_start();					// issue another start pulse

delay_ms(20);					// wait for chip to be ready

i2c_write(0xA1);				// issue chip type and number read command

data=i2c_read();				// read data from the chip

delay_ms(20);					// wait for chip

i2c_stop();					// issue an i2c stop pulse

delay_ms(20);					// wait for chip

input(hs);				// release hs line

return(data);

}

void clear_i2c_mem(unsigned int data)

{

unsigned int qx;

while(!input(HS));				// wait for mem chip to be available

output_low(HS);					// set hs line low

for (qx=0;qx<16;qx++)				// opp 16 times to clear memory

{

i2c_start();				// issue i2c start pulse

i2c_write(0xA0);				// issue chip type and number

i2c_write(qx);				// set the address to write to

i2c_write(data);				// write the data

i2c_stop();					// issue i2c stop command

delay_ms(20);				// wait for chip to store data

}

input(HS);					// release hs line

}

main.h

#include <18F2525.h>

#device ADC=10

#FUSES NOWDT                 	//No Watch Dog Timer

#FUSES FCMEN                 	//Fail-safe clock monitor enabled

#FUSES IESO                  	//Internal External Switch Over mode enabled

#FUSES PUT                   	//Power Up Timer

#FUSES NOMCLR

#FUSES BROWNOUT              	//Reset when brownout detected

#FUSES BORV21                	//Brownout reset at 2.1V

#FUSES NOPROTECT             	//Code not protected from reading

#FUSES NOWRT                 	//Program memory not write protected

#FUSES NOEBTR                	//Memory not protected from table reads

#use delay(internal=8MHz)

#use STANDARD_IO( C )

#use i2c(Master,Fast,sda=PIN_C4,scl=PIN_C3,force_hw)

#define HS pin_B3

PIC 2 Code

main.c

//Author J.Hunt

//ACDA5018 Practical

//9th Jan 2025

//PIC 2

#include <main.h>

#include <2408.C>

#define LCD_ENABLE_PIN PIN_B2

#define LCD_RS_PIN PIN_B0

#define LCD_RW_PIN PIN_B1

#define LCD_DATA4 PIN_B4

#define LCD_DATA5 PIN_B5

#define LCD_DATA6 PIN_B6

#define LCD_DATA7 PIN_B7

#include <lcd.c>

/***********Globals**************/

char ep_r(char);

void ep_w(char where,char what);

void payload(void);

char old_buts=0;

char buts=0;

/****************PIC 2*************************/

void main()

{

setup_adc_ports(NO_ANALOGS, VSS_VDD);

init_ext_eeprom();

lcd_init();

while(TRUE)

{

/*printf(lcd_putc,"\f 2hr Practical test");

lcd_gotoxy(1,2);									// display on line 2

printf(lcd_putc," intelligent systems ");

while(true);*/

//TODO: User Code

buts=ep_r(0x00); 					//reads value from mem

if(old_buts!=buts) 				//runs this sections only if the value changes

{

printf(lcd_putc,"\f");				//clears LCD

printf(lcd_putc,"buttons -%3u",buts); 		//displays value

//delay_ms(20);

lcd_gotoxy(1,2); 				// move to line 2

printf(lcd_putc,"Old Button %3u",buts); 	//displays value

//       output_a(old_buts); 				//displays binary value via LEDs

ep_w(0x01,buts);  				// write old_buts value to mem causes issues when writing to 0x01 value changes to 255 and back.... location moved to avoid issues

}

old_buts=buts; 					// writes the values to be the same

/****** Error Display *****/

//incomplete - looking to check read values vs stored in memory - ie on 1st boot when last value stored in memory chip

//	       if(old_buts!=ep_r(0x00));

//		  {

//		   printf(lcd_putc,"\f");			//clears LCD

//		   printf(lcd_putc,"\ERROR!!!"); 		//displays ERROR!!

//		  }

//	       else if(buts!=ep_r(0x99));

//		  {

//		   printf(lcd_putc,"\f");			//clears LCD

//		   printf(lcd_putc,"\ERROR!!!"); 		//displays ERROR!!

//		  }

//	       else //something!!;

}

}

/****************functions*******************/

void ep_w(char where,char what)

{

while(!input(hs));					// wait for mem chip to be available

output_low(hs);						// set hs line low

i2c_start();						// issue i2c start pulse

i2c_write(0xA0);					// issue chip type and number

i2c_write(where);					// send the address to write to

i2c_write(what);					// send the data to be written

delay_ms(20);						// wait for chip

i2c_stop();						// issue an i2c stop pulse

delay_ms(20);						// allow i2c chip to store data

input(hs);					// release hs line

}

char ep_r(char where)

{

char data=0;

while(!input(hs));					// wait for mem chip to be available

output_low(hs);						// set hs line low (mem busy)

i2c_start();						// issue i2c start pulse

i2c_write(0xA0);					// issue chip type and number

i2c_write(where);					// send address to be read from

delay_ms(20);						// wait for chip to set its for read

i2c_start();						// issue another start pulse

delay_ms(20);						// wait for chip to be ready

i2c_write(0xA1);					// issue chip type and number read command

data=i2c_read();					// read data from the chip

delay_ms(20);						// issue an i2c stop pulse

i2c_stop();						// allow i2c chip to store data

delay_ms(20);						// wait for chip

input(hs);					// release hs line

return(data);

}

main.h

#include <18F2525.h>

#device ADC=10

#FUSES NOWDT                 	//No Watch Dog Timer

#FUSES FCMEN                 	//Fail-safe clock monitor enabled

#FUSES IESO                  	//Internal External Switch Over mode enabled

#FUSES PUT                   	//Power Up Timer

#FUSES NOMCLR

#FUSES BROWNOUT              	//Reset when brownout detected

#FUSES BORV21                	//Brownout reset at 2.1V

#FUSES NOPROTECT             	//Code not protected from reading

#FUSES NOWRT                 	//Program memory not write protected

#FUSES NOEBTR                	//Memory not protected from table reads

#use delay(internal=8MHz)

#use STANDARD_IO( C )

#use i2c(Master,Fast,sda=PIN_C4,scl=PIN_C3,force_hw)

#define HS pin_B3

Comments

Hunt James

1.. The simulation runs and puts the button number in the top of lcd 1 and writes the data to the address 0x00 in the memory,

2.. the value is recieved by pic 2 and the value is displayed on lcd 2 and written to address location 0x01

3.. Your program does not illuminate the leds correctly

4. your program does not display the key value on the 2nd led but it does put it into a feedback position in the i2c mem.

5.. Some lines on the displays agree with the displayed leds.

6.. the data is not returned to chip 1 so cannot display on lcd 1

your hand shaking works correctly

the error warning does not work correctly