Ashleigh Jones (1707088)

1707088

Abstract

The identification of a duty cycle of a standard buggy by implementing a closed track drive cycle that’s representative of the type of environment the vehicle would be subjected to.  Identify the resistive forces associated with said buggy and use graphical aids to enable the further selection and implementation of a hybrid system.

Series Hybrid vehicle part 1a

Alternative Powertrain Technology

BSc Motorsport Engineering

Mark Sanford

2nd December 2019

Table of Figures

Figure 1 Image of Buggy (Hammerhead GTS150 Buggy, n.d.)	2

Equation 1 Velocity	4

Equation 2 Acceleration	4

Equation 3 Gradient	4

Equation 4 Aero Resistance	4

Equation 5 Acceleration Resistance	4

Equation 6 Gradian Forces	4

Table 1 Table of Total Resistive Forces	5

Table 2 Distance X Altitude	5

Table 3 Time X Velocity	6

Introduction

In this report the abilities and attributes of an off-road buggy is identified to gain a understanding of What conditions the vehicle is under and again an understanding of a typical duty cycle. This is done by the request of the engineering director producing off road buggies for the recreational and corporate entertainment sector. By engineering a track, a representation of an accurate duty cycle can be identified to ensure that all aspects of the test is covered. A data logger would be utilising in the test to gain accurate data from the vehicle.

A 150cc buggy is selected carefully to ensure that all of the specification of the vehicle is provided. This is to ensure that when the calculations can be conducted with the suitable information. Below is the image of the buggy being utilised for testing which is accompanied by the appropriate information to conduct the tests.  The important information that is required is a follows:

- Vehicle mass

- Hight

- Width

- Tire specifications

- Max speed

Figure 1 Image of Buggy (Hammerhead GTS150 Buggy, n.d.)

Methodology

A track is designed to gain an understanding of the duty cycle of the vehicle.  The tracked is representative of allot of the factors that is required by the vehicle. The track combines fast sections where the vehicle velocity can reach speed of 40mph.  Gradient changes to illustrate tractive effort needed to climb majority of hills encountered. Slow were corners is also used to combine variation in acceleration and deacceleration. The track also has a medium hard soil which has a coefficient of friction of 0.4 (Descornet, 1990). this is important to calculate the rolling resistance.    Once the track is designed and setup the data on the vehicle is then identified using the vehicle specifications and this converted into standardised units. Once all the necessary information is applied into the spreadsheet the vehicle can be run around the track to allow data to be recorded.  The data on the vehicle is then identified using the vehicle specifications and this converted into standardised units. Categories that are recorded is as follows;

- Time

- Distance

- Speed

- Altitude

This data categories are essential to gain a basic snapshot of what the vehicle is doing. By using this basic data other data can be calculated. Examples of the data calculated is as follows:

- Velocity

Equation 1 Velocity

v=D ×0.44704

- Acceleration

Equation 2 Acceleration

a= ∆v ×t

- Gradient

Equation 3 Gradient

tan-1  Gradient=∆h∆d

By using these equations, the forces acting upon the vehicle can be calculated. These forces are as follows;

- Aero resistance

Equation 4 Aero Resistance

Aero Dynamic resisatnce=0.5 ×Front area ×coefficient of drag  ×density of air ×velocity 2

- Acceleration Resistance

Equation 5 Acceleration Resistance

Acceleration Resisatnce=Mass ×Acceleration (Meerkamm, 2014)

- Gradient forces

Equation 6 Gradian Forces

Gradient forces=mass ×gravity ×TAN gradient (Meerkamm, 2014)

once all the forces are calculated, it can be added together to create the tractive effort. Tractive effort gains an understanding of the resistive forces acting upon it. Without tractive effort, it will be difficult to identify force needed to overcome resistance for the vehicle to move. In this case aero, acceleration, gradient and the rolling resistance of all four tires are included in the calculations.

All of this date is then imported into excel to sift through the data utilising the math channels and the graphical aid to illustrate the duty cycle.

A force, time X velocity graph and altitude graph is created to understand the what the vehicle is doing graphically to compare and contrast the data.

Analysis

Inspecting the spreadsheet, the total forces can be added together and identified. The total tractive forces used to give an understanding of the force required for the vehicle to move. On the Graph (Table 1) shows the total forces needed at each second elapsed. The tractive force initially shows a increase in force needed due to the high acceleration rate. When the vehicle accelerates further the resistance increases because of the acceleration forces and the aero forces acting apron it . Once the maximum speed is reached the vehicle deaccelerates, decreasing the acceleration forces and aero forces, subsequently reducing the total forces acting on the vehicle.  This is due to the fact the vehicle is slowing down for the corner and the Hill. This is evident in table 2 where the distance and altitude graph illustrate an increase in altitude at that point.  Once the vehicle is proceeding up the hill, there is a peak. This is due to the initial force overcome the gradient but then quickly averages out through the decent. Once the vehicle reached the top of the hill, the tractive forces are almost zero since there is no acceleration, gradient or aero resistance.  As the vehicle travels downhill, the resistive forces decreased below zero into the negative numbers. This means that the system us using gravitational advantage to overcome the resistive forces. Once the vehicle reaches the bottom there is a negative spike possibly due to terrain and gradient changes. Once on flat ground the vehicle experiences an equilibrium when traveling through the technical corners.  For an accurate test, the tires would experience a slip angle causing an increase in friction.  Around the last section the vehicle has velocity changes evident from table 3 but the resistive forces hardly show a change in resistance.

Table 1Table of Total Resistive Forces

On the table blow Table 2 illustrates the altitude changes on the track. There is a new one Hill at a height of 1.5 metres spanning 70 meters. Only one Hill was created since a gradient was needed to gain an accurate understanding of the tractive effort when going uphill as well as down a Hill. The rest of the track is remaining sea level.

Table 2 Distance X Altitude

The time velocity graph illustrates the speed of which the vehicle is travelling a different time frame. Initially the speed increases and levels out when quick corner is, taken then the velocity increases on the straight and decreases after the straight. Comparing this graph to the Resistance table (table 1) shows a correlation in resistive forces and velocity when speed increases this is due to the aerodynamic resistances.  Once the hill is reached the velocity decreases significantly but stays relatively varied due to the gradient and aero resistance. After the Hill the velocity fluctuates as the vehicle goes around the multiple corners this is also apparent in the resistive forces in table one.

Table 3Time X Velocity

Conclusion

This data successfully achieved the goal that’s set by the engineering director by identifying the duty cycle of a standard buggy and identifying its trait. By using this data, an understanding of the duty cycle can be achieved and then recommended hybrid system can be suggested since a detailed snapshot of what the vehicle can achieve is identified.  The resistance clearly shows what resistive forces it undergoes for the vehicle to move. By using this data alone as well and the max gradient data, a suitable drivetrain can be identified.

# Bibliography

Descornet, G. (1990). Road-Surface Influence on Tire Rolling Resistance. Brussels: Centre de Recherches Routieres.

Hammerhead GTS150 Buggy. (n.d.). Retrieved from Storm Buggies : https://www.stormbuggies.com/products/hammerhead-150gts-buggy-with-usa-specs-red-1-1

Meerkamm, F. D. (2014). Schaeffler Technical Pocket Guide. Herzogenaurach: Schaeffler Technologies.