Project Name: Jayne Peirson School of Art, Design & Fashion

Project Ref: 6063

Date: 09/04/24

Proposed Structural Calculations for Jayne Perison School of Art, Design & Fashion.

Please note: All calculations are intended for preliminary design only; final calculations are to be carried out and approved by an engineer.

Prepared by Jack Keenan

The below drawings highlighted areas that have been selected to produce preliminary structural calculations. The calculations will be focused on the dead and imposed loads of the following structural elements:

- Calculations for structural columns

- Calculations & centres for structural beams

- Typical loads of columns for proposed pile foundations

Ground Floor plan:

First Floor plan:

Second Floor plan:

Ground Floor Calculations:

| Beams Spans: | DIST to structural columns: | Area: |
| --- | --- | --- |
| - 14.4m - 4.4m - 4.4m | - 4.7m - 4.4m - 4.4m - 4.1m | - 83m2 |

Beam span & centres for first floor structure:

Strength class of Glulam = GL24h

4.5m span = 180 x 320 beam at 4.5m centres

14.4m span = 240 x 800 beam at 4.5m centres

Dead Loads/Imposed Loads:

plywood – 0.7kg per mm thick

300mm x 0.7kg = 210kg m-2

Screeding – Cement 13mm = 30kg m-2

210kg x 10 = 2100N per m2 /1000 = 2.1Kn per m2

30kg x 10 = 300N per m2/ 1000 = 0.3Kn per m2

Dead load = 2.1Kn + 0.3Kn = 2.4Kn per m2

Imposed load = 3Kn per m2

First Floor Calculations:

| Beams Spans: | DIST to structural columns: | Area: |
| --- | --- | --- |
| - 14.4m - 4.4m - 4.4m | - 4.4m - 4.4m - 4.4m - 9.3m | - 83m2 |

Beam span & centres for second floor structure:

Strength class of Glulam = GL24h

4.5m span = 180 x 320 beam at 4.5m centres

14.4m span = 240 x 800 beam at 4.5m centres

Dead Loads/Imposed Loads:

plywood – 0.7kg per mm thick

300mm x 0.7kg = 210kg m-2

Screeding – Cement 13mm = 30kg m-2

210kg x 10 = 2100N per m2 /1000 = 2.1Kn per m2

30kg x 10 = 300N per m2/ 1000 = 0.3Kn per m2

Dead load = 2.1Kn + 0.3Kn = 2.4Kn per m2

Imposed load = 3Kn per m2

Second Floor Calculations:

| Beams Spans: | DIST to structural columns: |  | Area: |
| --- | --- | --- | --- |
| - 14.2m - 4.4m - 4.4m | - 4.6m - 4.5m - 9.3m - 4.1m |  | - 83m2 |

Beam spans & centres for roof beams:

Strength class of Glulam = GL24h

4.5m span = 140 x 280 beam at 4.5m centres

14.4m span = 180 x 760 beam at 4.5m centres

Dead Loads/Imposed Loads:

plywood – 0.7kg per mm thick

300mm x 0.7kg = 210kg m-2

Screeding – Cement 13mm = 30kg m-2

210kg x 10 = 2100N per m2 /1000 = 2.1Kn per m2

30kg x 10 = 300N per m2/ 1000 = 0.3Kn per m2

Dead load = 2.1Kn + 0.3Kn = 2.4Kn per m2

Imposed load = 3Kn per m2

Imposed pile foundation Calculations:

| Description: | QTY: | Height: |
| --- | --- | --- |
| Structural Columns | 3 No. | 3.6m |

Total Load carried by central column:

Equation = (Dead load + imposed load) x area x floors

(2.4Kn + 3kn) x 20.7 x 3 = 335.34Kn

Pile Foundation will be at a 12m depth x 360mm diameter based on a 335.34Kn imposed load.