University of Wales Trinity Saint David

Wales Institute of Science & Art

Application of Computer Aided Engineering Processes

Structural Design & Fluid Analysis

ACHA6010

Daniel Pewsey

1901754

November 2022

Contents Page

- Introduction

- Aims & Objectives

- Validation Study

- Bimetallic Strip

- Cantilever Beam

- CAD Modelling & Study

- Kinematic ,Thermal & Structural Analysis

- Kinematic Simulation

- Steady-State Thermal

- Static Structural Study

- Conclusion

- References & Figures

- Appendices

# Abstract

This project serves to analyse and evaluate the use of Computer-Aided Design software aswell as a comprehensive analysis on the dynamic forces, thermal conditions, and rotational moment acting on a piston in an assembly. This piston then has been improved by way of alternative materials and weight saving methods.

# 1. Introduction

This project is to simulate the testing of a real-world system modelled in computational design software. The system will then be analysed using software and will be optimized with a particular focus on structural improvement and efficient material use. This system will then be used to discuss the limitations of using Computer Aided Design Software.

- Aims & Objectives

- Conduct a validation on the software that will be used for the analysis, the validation will measure the accuracy of the results. To achieve this:

- Find a suitable CAD model that can be used for an analytical analysis.

- Complete the analytical test and then compare it back with the ANSYS results

- Study how mesh sizing affects the accuracy of results using a point of mesh independence.

- Create a realistic model assembly to use for a kinematic, thermal, and structural study.       To achieve this:

- Model will need to be as realistic as possible to obtain accurate results from the study, meaning it will have to be as accurate as possible.

- Use realistic boundary conditions and realistic material properties throughout the study.

- Improve the performance of a component in the assembly from the previous results of the FEA analysis. To achieve this:

- Set design targets to assist in improving the design.

- Reconduct studies to compare back to the benchmark study.

- Discuss the limitations of using CAE as a design tool using the previous studies. To achieve this:

- Compare to the previous models and discuss findings.

- Discuss how this may differ from a real- world scenario.

2.Validation Studies

Bimetallic Strip Study

The strip is made of 2 different metals with their own respective coefficients of thermal expansion. As the temperature changes the metals have different magnitudes of elongation, this effect can be used in instruments that control or sense temperatures such as thermistors and temperature control switches.

Figure 1 Bimetallic Strip Dimension View 1

Figure 2 Bimetallic Strip Dimension View 2

To find the ultimate deflection of the bimetallic strip analytically we can use the following formula to find us the coefficient K.

K1=4+6tcti+4tcti2+EcEitcti3+EItiECtc     (eq1)

This can then be used to find the equivalent stiffness. The deflection is then defined by eq2 below.

Uz1max=6ai-acti+tcTcts2K1L22≈-7.049mm   (eq2)

Conducting a structural study on the bimetallic strip with a thermal condition is shown below.

Figure 3 Bimetallic Strip Deformation 100C°

From the study it is deduced that at the free end of the bimetallic strip there is a deflection of approximately 7.123mm when using a mesh sizing of 0.5mm. This gives an error margin of 1.05%.

The first test run used a mesh sizing of 0.1mm, this resulted in a deflection of 7.120mm, this gives an error 1.01%

To investigate the impact that mesh sizing has on the accuracy of the deflection study test were run at other mesh sizes.

| Mesh Size (mm) | % Error compared to Analytics |
| --- | --- |
| 0.1 | 1.01 |
| 0.25 | 1.03 |
| 0.5 | 1.05 |
| 0.75 | 1.05 |
| 1 | 1.05 |

Figure 4 Graph Mesh Size Vs. % Error

The graph above shows how the percentage error relates to the mesh size. Ideally the study would have a percentage error below 1% but due to hardware limitations the meshing could not be complete.

Above the mesh sizing of 0.5mm the percentage error changes negligibly (below 2.d.p). This means that it takes a meshing value of at least 0.5mm to achieve a mesh independent result.

A mesh independent result is achieved when “a significant increase in node count does not result in a significant increase in stress” (1)

Cantilever Beam Study

The Cantilever Beam is a rigid structural beam that extends in the horizontal direction, it is fixed at one end and carries a uniform load across the rest of the beam. Cantilevers are generally used in the construction industry to allow for “overhanging structures without additional supports” (2)

Figure 5 Cantilever Diagram  (2)

The analytical calculation to find the deformation of the cantilever beam can be conducted with the equation below starting with the bending moment M for the plate under its continuous loading case.

M=q(L-x)22    (eq3)

The maximum displacement of the beam occurs at the point x=L, after calculation the validation the displacement is shown below.

Uz,max=qL48EIy=3pL42Ew2t=157.143mm    (eq4)

The cantilever made in solid works is shown below along with its respective dimensions.

Figure 6 Cantilever Beam Dimension View 1

Figure 7 Cantilever Beam Dimension View 2

Figure 8 Cantilever Beam Deformation

From the study it is deduced that the cantilever beam has a deformation of 157.15mm, when using a mesh sizing of 2.5mm this gives an error margin of 0.0045%.

| Mesh Size (mm) | % Error compared to Analytics |
| --- | --- |
| 0.625 | 0.0045 |
| 1.25 | 0.0045 |
| 2.5 | 0.0045 |
| 5 | 0.0045 |
| 10 | 0.0045 |

Figure 9 Mesh Sizing Vs. % Error

The graph above shows how the mesh sizing of the cantilever beam effects the percentage error. The mesh sizing is doubled for every study, this has no effect on the percentage error, so it has therefore achieved mesh independence.

3.CAD Modelling & Assembly

Inline 4 Engine Assembly

For further testing the assembly used will be of an inline-4 engine,

Figure 10 Inline-4 Solidworks Assembly

The assembly consists of 13 parts including all pistons, connecting rods, gudgeon pins along with the individual crankshaft. The assembly is converted to a .STEP file for the Ansys studies.

Engineering Data

The material used in the engineering data for the piston is Aluminum 2618A, this is based off an uprated piston for a 2.0 TFSI engine (EA113 BWA) (3) (4). This piston is built to withstand much higher combustion pressures and is built to “tighter tolerances on 14 critical areas of the piston” (4). The standard piston material for this engine configuration is a ‘MAHLE Aluminum 4032 alloy piston’ (5).

The values needed in the engineering data section for the Aluminum are shown below

| Engineering Data Values | Aluminum 2618A |
| --- | --- |
| Youngs Modulus | 74500Mpa |
| Poisson’s Ratio | 0.33 |
| Specific Heat Capacity | 0.875 J g-1K-1 |
| Isotropic Coef Of Thermal Expansion | 2.16×10-5K-1 |
| Isotropic Thermal Conductivity | 146 W m-1K-1 |
| Density | 2760 Kgm3 |

(6) (7)

The values for the standard piston are in the appendices

The conrod, gudgeon pin, and crankshaft material is AISI Steel-Billet 4340, this material is an ultra-high strength medium carbon low alloy steel, it is used in a performance replacement for the standard crankshaft in the EA113 engine platform.

The values needed in the engineering data section for the Billet Steel are shown below.

| Engineering Data Values | AISI Steel-Billet 4340 |
| --- | --- |
| Youngs Modulus | 205000Mpa |
| Poisson’s Ratio | 0.29 |
| Specific Heat Capacity | 0.475 J g-1K-1 |
| Isotropic Coef of Thermal Expansion | 1.23×10-5C-1@100°C |
| Density | 7.85 gcm3 |

(8)

4.Kinematic ,Thermal & Structural Analysis

The analysis will be conducted on one of the pistons using a variety of different grades of aluminum, the different grades have different advantages to their use as a piston material.

Figure 11 Piston SLDWRKS Picture

Kinematic Simulation

The kinematic simulation is conducted using ANSYS, this is used to create the loading conditions for the thermal and structural study . The simulation will be run for each of the materials that the tests will be run on. This simulation serves to define the “changes in position and orientation of the body segments” (12)aswell as the “geometry of motion in terms of displacements, velocities and accelerations” (12).

The study was conducted on 3 different grades of aluminium, the first is Aluminium 6061 T6, this is used as the base material for the study as it is a common, precipitation-hardened aluminium alloy with a high tensile strength. The second material is the standard piston material used in my personal car, this is Aluminium 4032, it is a forged application created for the higher cylinder pressures associated with forced induction engines. The third and final material choice is Aluminium 2618A, this is used in uprated pistons for my personal car, these are also a forged application but are built to withstand a further increase in load from forced induction.

The kinematic simulation measured the deformation of the piston aswell as the stress of the component.

Figure 12 Aluminium 6061 T6 Kinematic Simulation

The results above show the deformation of the piston when using Aluminium 6061 T6, the maximum deflection is in the centre of the crown of the piston and peaks at 0.3458mm.

Figure 13 Aluminium 6061 T6 Stress

The results above show the equivalent stress acting on the piston when using Aluminum 6061 T6, the peak stress is acting on the lower piston ring groove.

Figure 14 Kinematic Chart 1

Figure 15 Kinematic Chart 2

The results from the kinematic simulation show that the Aluminium 6061 T6 has the highest deformation of the three materials along with having the 2nd highest equivalent stress, The Aluminium 4032A has the lowest deformation along with the lowest equivalent stress out of the three materials. Aluminium 2618A sits in-between the other materials in terms of deformation but has the highest equivalent stress.

Steady-State Thermal Study

The next study conducted on the 3 different aluminium grades is a steady state thermal test, a steady state thermal analysis consists of “evaluating the thermal equilibrium of a system in which the temperature remains constant over time.” (13)

The data from the rigid dynamics study was carried over using the ANSYS Workbench for each respective material to ensure they have the correct engineering data, geometry, model, and inputs from previous solutions.

Figure 16 ANSYS Workbench

The results from the steady state thermal of each of the materials is shown below.

The results above show that the base aluminum 6061 T6 has the lowest maximum temperature and the smallest temperature difference between its minimum and maximum temperatures. Aluminum 2618A however has the highest maximum temperature along with the greatest temperature difference between its maximum and minimum temperatures. Aluminum 4032A sits in between both with its respective results. These results show how the thermal conductivity changes with different material compositions.

The more a piston can transfer heat to the cylinder wall the cooler the piston will be, when the piston is “operating at a lower temperature, it heats incoming fuel-air mixture less. This means the charge expands less and loses less density, resulting in more power.” (14).

Static Structural Analysis

The static-structural analysis combines the solution from the thermal study and the engineering data, geometry, and model from the rigid dynamics study. This will then combine to produce a comprehensive study on the dynamic forces conducted on the piston.

Using the solution from the thermal study along with the engineering data, geometry, and model from the rigid dynamics study,

The materials have performed in a similar manner to how they did in the rigid dynamics study.

From this study it can be deduced that the best performing material is the Aluminium 4032A as it has the least deformation, after excessive piston deformation and stress the material will be more susceptible to damage, this could consist of damage to the crown or cracks in the piston itself. Damage such as this will cause a loss of compression and therefore a loss of power. It can also cause damage to other components in the system such as the cylinder bore for example.

As the Aluminium 2618A piston is supposedly a higher performance piston how does it deform more?, Aluminium 2618 pistons expand as their temperature increases as they do not feature silicon like the 4032 counterparts, this silicon reduces ‘piston slap’ when the engine is operating at colder temperatures, the advantage being that it “drastically reduces the noise factor” as it has a “tighter wall clearance” (15). Aluminium 2618 also tends to “bend while retaining its original shape” (15) so it withstands the increased deformations shown in the study.

Improvements

To improve the piston the design has been changed with the aim of reducing the piston mass, this will in turn reduce the rotational forces occurring in the assembly.

Figure 17 Lightened Rotating Assembly                                                                                               Figure 18 Lightened Piston

The weight saving design features the cutting of some materials on the underside of the piston as shown above, the cuts are 4mm in radius and approx 3mm in depth, there are 4 per piston, this calculates to a volume of 600mm3 so for the entire rotating assembly this calculates to 0.0000006m3.

| Material | Total Weight Saving (kg) | Total Weight Saving (g) |
| --- | --- | --- |
| Aluminium 2618A | 0.001656 | 1.656 |
| Aluminium 4032 | 0.001614 | 1.614 |
| Aluminium 6061 T6 | 0.001620 | 1.620 |

For Aluminium 2618A the standard piston has a weight of 260.34 g while the lightweight counterparts weigh 258.68g, this is a saving of 0.64%. This will reduce the load on the gudgeon pin and conrod as a result and can allow an engine to accelerate quicker. The weight could also be decreased further by taking more material from underneath the piston or shortening the skirt. Further weight reduction could result in a structural compromise but could still be applicable depending on the piston loading of the application e.g., High-revving NA engine.

The piston could also be changed by changing the geometry of the crown of the piston, with this the compression of the engine can be changed in favour of the application by adding material to increase compression or removing material to decrease compression.

CAD Discussion

Computer-aided design is the use of software to aid in design processes, it is used in a broad array of professions to produce fast, cheap, and accurate engineering schematics for use in analysis, production, and design areas. CAD is used as a cheaper alternative to physical testing as it reduces material cost and provides an ease of repeatability when making designs, this means that small alterations can be made to enhance the design without starting from the ground up, an example of this is shown above where the material of the piston was easily changed, and the tests were re-run, in a physical scenario this would require multiple engines, pistons, crankshafts and more to complete an effective analysis. It also makes it possible to change design geometries such as with material weight saving shown in the project above.  The limitations of CAD software show when files can become corrupt or project licensing needs to be purchased, sometimes an industrial license could cost more than completing some physical testing especially when testing is on a smaller scale. Another drawback of using CAD is that “It is limited in context” (16), for example when covering a bolt hole therefore making it inaccessible. Especially in the automotive industry this is a prevalent issue as there are quite often many moving parts with their own respective fittings.

5.Conclusion

To conclude on computer aided design software in its entirety it is an incredibly useful tool which can be a far superior, cost-effective alternative method of designing and testing products. This, however, depends heavily on the application that it is required to do as it can be a more labour intensive, time-consuming way to conduct an analysis.

To conclude on the piston analysis the standard piston improved by using different Aluminium alloys to improve the durability of the piston, depending on the purpose of the engine a lighter and stronger piston can be used (2618), or a stronger but more effective for daily use piston can be used (4032). The piston was also improved by adding weight saving measure to the underside of the piston, this reduced weight and therefore reduced the rotating mass in the assembly which will in turn reduce load on other components.

# 6. References & Figures

1, F. (2019, 3 31). FEATips. Retrieved from FEATips: featips.com/2019/03/31/what-is-a-mesh-independance-study/

2, T. (2022). Theconstructor. Retrieved from Theconstructor: Theconstructor.org/structural-engg/cantilever-beams/167474/

3, r.-t. (2020). r-techperformance. Retrieved from r-techperformance: https://r-techperformance.co.uk/2-0-tfsi-tuning/

4, p. (2020). performancebyie. Retrieved from performancebyie: https://performancebyie.com/products/je-pistons-vw-audi-20t-fsi-tsi

5, A. j. (2009). goapr.jpn. Retrieved from goapr.jpn: http://goapr.jp/products/pistons_20t_ea113.html

6, R. M. (2022). Referans Metal. Retrieved from Referans Metal: http://www.referansmetal.com/alasimli-aluminyum/product/21/uzay-havacilik-savunma/levha-plaka/air-9048-640-alcu2mg1-5ni-2618-2618a-t851?lang=en

7, a. (2012, 11 13). azom. Retrieved from azom: https://www.azom.com/article.aspx?ArticleID=6617

8, T. W. (2020). The World Material. Retrieved from The World Material: https://www.theworldmaterial.com/astm-sae-aisi-4340-alloy-steel/

9, A. (2020, 11 13). Azom. Retrieved from Azom: https://www.azom.com/article.aspx?ArticleID=6622

10, A. N. (2022). Piston Study Data.

11, A. (2020). Azom. Retrieved from Azom: https://www.azom.com/article.aspx?ArticleID=6636

12, B. R. (2014). Science Direct . Retrieved from Science Direct : https://www.sciencedirect.com/topics/engineering/kinematic-analysis

13, J. W. (2020, 08 11). 6 Sigma ET. Retrieved from 6 Sigma ET: https://www.6sigmaet.info/blog/what-is-steady-state-    thermal-analysis/

14, K. C. (2021, 12 1). Pistons, Hot and Cold. Retrieved from Cycleworld: https://www.cycleworld.com/story/blogs/ask-kevin/how-motorcycle-piston-temperature-affects-engine-power/

15, J. S. (2019). OnAllCylinders. Retrieved from https://www.onallcylinders.com/2019/12/20/ask-away-with-jeff-smith-how-2618-vs-4032-alloys-affect-forged-piston-noise/#:~:text=The%202618%20alloy%20expands%20more,greater%20ability%20to%20absorb%20abuse.

16, C. G. (2019, 10 9). Cad guide . Retrieved from https://www.computeraideddesignguide.com/limitations-cad-2017-onwards/

Figures

Figure 1 Bimetallic Strip Dimension View 1	3

Figure 2 Bimetallic Strip Dimension View 2	3

Figure 3 Bimetallic Strip Deformation 100C°	3

Figure 4 Graph Mesh Size Vs. % Error	4

Figure 5 Cantilever Diagram  (2)	5

Figure 6 Cantilever Beam Dimension View 1	5

Figure 7 Cantilever Beam Dimension View 2	5

Figure 8 Cantilever Beam Deformation	6

Figure 9 Mesh Sizing Vs. % Error	6

Figure 10 Inline-4 Solidworks Assembly	7

Figure 11 Piston SLDWRKS Picture	8

Figure 12 Aluminium 6061 T6 Kinematic Simulation	8

Figure 13 Aluminium 6061 T6 Stress	9

Figure 14 Kinematic Chart 1	9

Figure 15 Kinematic Chart 2	9

Figure 16 ANSYS Workbench	10

Figure 17 Lightened Rotating Assembly                                                                                                  Figure 18 Lightened Piston	12

Figure 19 Aluminium 2618A Deformation	17

Figure 20 Aluminium 2618A Stress	17

Figure 21 Aluminium 4032 Deformation	17

Figure 22 Aluminium 4032 Stress	17

Figure 23 Thermal Study Aluminium 2618A	18

Figure 24 Thermal Study Aluminium 4032	18

Figure 25 Thermal Study Aluminium 6061 T6	18

Figure 26 Aluminium 2618A Structural Deformation	19

Figure 27 Aluminium 2618A Structural Stress	19

Figure 28 Aluminium 4032A Structural Deformation	20

Figure 29 Aluminium 4032A Structural Stress	20

Figure 30 Aluminium 6061 T6 Structural Deformation	21

Figure 31 Aluminium 6061 T6 Structural Stress	21

7.Appendices

Equations

- K1=4+6tcti+4tcti2+EcEitcti3+EItiECtc

- Uz1max=6ai-acti+tcTcts2K1L22≈Max Deflection

- M=q(L-x)22

- Uz,max=qL48EIy=3pL42Ew2t=Displacement

Standard Piston values

| Engineering Data Values | Aluminum 4032A |
| --- | --- |
| Youngs Modulus | 75000Mpa-12000Mpa |
| Poisson’s Ratio | 0.33 |
| Specific Heat Capacity | 0.875 J g-1K-1 |
| Isotropic Coef Of Thermal Expansion | 1.94×10-5K-1 |
| Isotropic Thermal Conductivity | 155 W m-1K-1 |
| Density | 2.69 g cm3 |

(9)

6061 T6 Aluminum values

| Engineering Data Values | Aluminum 6061 T6 |
| --- | --- |
| Youngs Modulus | 67000Mpa |
| Poisson’s Ratio | 0.33 |
| Specific Heat Capacity | 0.896 J g-1K-1 |
| Isotropic Coef Of Thermal Expansion | 2.36×10-5K-1 |
| Isotropic Thermal Conductivity | 167 W m-1K-1 |
| Density | 2.7 g cm3 |

(10) (11)

Piston Rigid (Kinematic study)

Aluminium 2618A – uprated forged

Figure 19 Aluminium 2618A Deformation

Figure 20 Aluminium 2618A Stress

Aluminium 4032 – Standard Forged

Figure 21 Aluminium 4032 Deformation

Figure 22 Aluminium 4032 Stress

Piston Thermal

Aluminium 2618A

Figure 23 Thermal Study Aluminium 2618A

Aluminium 4032

Figure 24 Thermal Study Aluminium 4032

Aluminium 6061 T6

Figure 25 Thermal Study Aluminium 6061 T6

Piston Static Structural

Aluminium 2618A

Total Deformation

Figure 26 Aluminium 2618A Structural Deformation

Equivalent stress

Figure 27 Aluminium 2618A Structural Stress

Aluminium 4032

Total Deformation

Figure 28 Aluminium 4032A Structural Deformation

Equivalent stress

Figure 29 Aluminium 4032A Structural Stress

Aluminium 6061 T6

Total deformation

Figure 30 Aluminium 6061 T6 Structural Deformation

Equivalent Stress

Figure 31 Aluminium 6061 T6 Structural Stress