Applied Computing

Wales Institute of Science and Art (WISA)

# Advanced Networking (ACCA5024)

Lab 8: Static Routing

n.kaliyamurthy@uwtsd.ac.uk                                                                                                                            1

Design the topology and execute basic configuration as stated below

Refer to the above topology for addressing information.

Tasks:

- Set Up the Topology and Initialize Devices

- Configure Devices and Verify Connectivity

Assign static IPv4 information to the PC interfaces.

Configure basic router settings. (Refer to our previous labs. Basic Configurations includes hostname, passwords, line configurations, device security, domain lookups, banners)

Configuring your network with complete information is highly recommended.

Verify network connectivity.

- Configure Static, Default and Loopback addresses for all the routers Configure IPv4 Address for all the router interfaces.

All completed

Configure loopback address (Lo1) as 50.50.50.50/32 at Router R2.

DDoneDsadsdasd

On R1, R2 and R3 configure IPv4 static route to connect other networks. Do not include Lo1 network.

Configure default static route to reach Lo1 network using next-hop ip address at R1

Configure default static route to reach Lo1 network using exit interface at R3

Interpret the output from the routing table.

R1# show ip route Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 10.0.0.0/30 is directly connected, Serial0/1/0 L 10.0.0.1/32 is directly connected, Serial0/1/0 192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.10.0/24 is directly connected, GigabitEthernet0/0/0 L 192.168.10.1/32 is directly connected, GigabitEthernet0/0/0 S 192.168.20.0/24 [1/0] via 10.0.0.2 S 192.168.30.0/24 [1/0] via 10.0.0.2 200.0.0.0/24 is variably subnetted, 2 subnets, 2 masks C 200.0.0.0/24 is directly connected, Loopback1 L 200.0.0.1/32 is directly connected, Loopback1 R2(config)#do show ip route Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 10.0.0.0/30 is directly connected, Serial0/1/0 L 10.0.0.2/32 is directly connected, Serial0/1/0 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 20.0.0.0/30 is directly connected, Serial0/1/1 L 20.0.0.1/32 is directly connected, Serial0/1/1 50.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 50.50.50.48/30 is directly connected, Loopback1 L 50.50.50.50/32 is directly connected, Loopback1 S 192.168.10.0/24 [1/0] via 10.0.0.1 192.168.20.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.20.0/24 is directly connected, GigabitEthernet0/0/0 L 192.168.20.1/32 is directly connected, GigabitEthernet0/0/0 S 192.168.30.0/24 [1/0] via 20.0.0.2 R3# show ip route Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 20.0.0.0/30 is directly connected, Serial0/1/1 L 20.0.0.2/32 is directly connected, Serial0/1/1 S 192.168.10.0/24 [1/0] via 20.0.0.1 S 192.168.20.0/24 [1/0] via 20.0.0.1 192.168.30.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.30.0/24 is directly connected, GigabitEthernet0/0/0 L 192.168.30.1/32 is directly connected, GigabitEthernet0/0/0 200.0.0.0/24 is variably subnetted, 2 subnets, 2 masks C 200.0.0.0/24 is directly connected, Loopback1 L 200.0.0.20/32 is directly connected, Loopback1

Verify the status of the interfaces.

R1#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0/0 192.168.10.1 YES manual up up GigabitEthernet0/0/1 unassigned YES unset administratively down down Serial0/1/0 10.0.0.1 YES manual up up Serial0/1/1 unassigned YES unset administratively down down Loopback1 200.0.0.1 YES manual up up Vlan1 unassigned YES unset administratively down down R2(config)# do show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0/0 192.168.20.1 YES manual up up GigabitEthernet0/0/1 unassigned YES unset administratively down down Serial0/1/0 10.0.0.2 YES manual up up Serial0/1/1 20.0.0.1 YES manual up up Loopback1 50.50.50.50 YES manual up up Vlan1 unassigned YES unset administratively down down R3#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0/0 192.168.30.1 YES manual up up GigabitEthernet0/0/1 unassigned YES unset administratively down down Serial0/1/0 unassigned YES unset administratively down down Serial0/1/1 20.0.0.2 YES manual up up Loopback1 200.0.0.20 YES manual up up Vlan1 unassigned YES unset administratively down down

Verify the status of the network connectivity.

Network connectivity looks fine, as I was able to ping all the devices from any device.

Record your routers running configurations here!!!  Do it separately for R1, R2 and R3

R1#show run

Building configuration...

Current configuration : 1192 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R1

!

!

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name R1

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback1

ip address 200.0.0.1 255.255.255.0

!

interface GigabitEthernet0/0/0

ip address 192.168.10.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

ip address 10.0.0.1 255.255.255.252

clock rate 64000

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.20.0 255.255.255.0 10.0.0.2

ip route 192.168.30.0 255.255.255.0 10.0.0.2

!

ip flow-export version 9

!

!

!

banner motd ^C

Unauthorized access is strictly prohibited and prosecuted to the full extent of the law.^C

!

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A1613030B

logging synchronous

login

!

!

!

end

R2#show run

Building configuration...

Current configuration : 1192 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R2

!

!

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name R2

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback1

ip address 50.50.50.50 255.255.255.252

!

interface GigabitEthernet0/0/0

ip address 192.168.20.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

ip address 10.0.0.2 255.255.255.252

!

interface Serial0/1/1

ip address 20.0.0.1 255.255.255.252

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.10.0 255.255.255.0 10.0.0.1

ip route 192.168.30.0 255.255.255.0 20.0.0.2

!

ip flow-export version 9

!

!

!

banner motd ^C.

Unauthorized access is strictly prohibited and prosecuted to the full extent of the law.^C

!

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A1613030B

logging synchronous

login

transport input ssh

!

!

!

end

R3#show run

Building configuration...

Current configuration : 1260 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R3

!

!

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name R3

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback1

ip address 200.0.0.20 255.255.255.0

!

interface GigabitEthernet0/0/0

ip address 192.168.30.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

ip address 20.0.0.2 255.255.255.252

clock rate 64000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.30.0 255.255.255.0 20.0.0.1

ip route 192.168.20.0 255.255.255.0 20.0.0.1

ip route 192.168.10.0 255.255.255.0 20.0.0.1

!

ip flow-export version 9

!

!

!

banner motd ^C

Unauthorized access is strictly prohibited and prosecuted to the full extent of the law.^C

!

!

!

!

!

line con 0

exec-timeout 5 0

password 7 0822455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A1613030B

logging synchronous

login

transport input ssh

!

!

!

end