MOCK EXAM - Marking KEY

ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- A power supply requires 25W of input power and produces an output power of 20W. Calculate its efficiency in percentage and the total power loss.	[5 Marks]

Answer:

Efficiency =POUTPIN= 2020=0.8							[2 Marks]

% Efficiency = 0.8×100=80%						[1 Mark]

Total Power Loss PLoss=PIN-POUT=25-20=5W			[2 Marks]

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?		[4 Marks]

Answer:

I=PV=13 									[2 Marks]

I=0.33A 									[2 Marks]

- For the circuit shown in fig.1, calculate the following.

- Equivalent Resistance,						[6 Marks]

- Current flowing through 20Ω Resistor				[3 Marks]

Fig:1

Answer:

- Equivalent Resistance

R4+R3 in parallel with R5

Req1=(R3+R4)×R5R3+R4+R5=30+50×5030+50+50=30.369Ω					[2 Marks]

Req1+R2 in parallel with R6

Req2=(Req1+R2)×R6Req1+R2+R6=30.369+25×5030.369+25+50=26.274Ω				[2 Marks]

Req2 is series with R1

Req=R1+Req2=20+26.264=46.264Ω 				[2 Marks]

- Current flowing through 20Ω Resistor

I=VReq=1546.264=0.324A							[3 Marks]

- Define Kirchoff’s Current Law.						[3 Marks]

Answer:

At any node or junction in an electrical circuit, the sum of currents flowing into that node is equal to the sum of currents flowing out of that node.				[3 Marks]

- Define the following terms

- Impedance								[2 Marks]

- Peak inverse Voltage							[2 Marks]

Answer:

- Impedance

Vector sum of resistance and reactance – Describes total opposition to current – AC and DC circuit property.								[2 Marks]

- Peak inverse voltage

The maximum reverse voltage a diode can withstand without breaking down. Exceeding the PIV can cause permanent damage to the diode.				[2 Marks]

Question 2 [25 Marks]

- Determine the current across each resistor in the circuit shown in fig.2, using mesh analysis (Kirchhoff’s Voltage Law)		[16 Marks]

Fig.2

Answer:

Apply KVL in ABEFA

-5I1-3I1-I2-10-6I1+15=0 					[3 Marks]

-14I1+3I2=-5 …………(1)							[2 Marks]

Apply KVL in BCDEB

-6I2-8I2+10+3I1-I2=0 						[3 Marks]

+3I1-17I2=-10 …………(2)						[2 Marks]

Simplifying Equation (1) and (2)

I2=0.677A									[3 Marks]

Substitute, I2 in equation (1)

-14I1+30.677=-5

I1=7.03114=0.5022A								[2 Marks]

Current across branch BE

I1-I2=0.5022-0.677=-0.1678A						[1 Mark]

Current Across R1and R3=I1=0.5022A

Current Across R2=I1-I2=-0.1678A

Current Across R4 R5=I2=0.677A

- For the circuit shown in fig 3, determine the following

- Total impedance							[5 Marks]

- Circuit current							[2 Marks]

- Phase angle								[2 Marks]

Fig.3

Answer:

- Total impedance

XL=2πfL=2×π×50×1 mH=0.3142 Ω					[2 Marks] XC=12πfC=12×π×50×470 μF=6.773 Ω  						[2 Marks]

Z=R2+XL-XC2=1002+0.3142-6.7732=100.208 Ω		[1 Mark]

- Circuit Current

I=VZ=120100.208=1.198 A								[2 Marks]

- Phase Angle

φ=tan-1XTR=tan-10.3142-6.773100=-3.695°					[2 Marks]

Question 3 [25 Marks]

- A 12 V Battery has an internal Resistance of 12Ω, Calculate the power dissipated by various load resistances given in the below table and complete the table.		[10 Marks]

| Load Resistance (Ω) | Load power (W) |
| --- | --- |
| 4 |  |
| 8 |  |
| 12 |  |
| 16 |  |
| 20 |  |

Answer:

| Load Resistance (Ω) | Load power (W) |
| --- | --- |
| 4 | 2.25 |
| 8 | 2.88 |
| 12 | 3 |
| 16 | 2.938 |
| 20 | 2.813 |

[2 marks for each, maximum of 10 marks]

- A sine wave is observed on a oscilloscope as shown in fig 4. The time setting is 10ms/div and voltage is 3V / div. Estimate the following.

- Peak Voltage								[2 Marks]

- Peak to peak voltage							[2 Marks]

- Time period of the waveform						[2 Marks]

- Frequency								[2 Marks]

Fig.4

Answer:

- Peak voltage

Peak voltage=3×2div=6V		[2 Marks]

- peak-peak voltage

Vpk-pk=4 div ×3 V=12 V						[2 Marks]

- time period of the waveform.

Period=5 div ×10 ms=50 ms					[2 Marks]

- Frequency.

f=1t

f= 150 ms=20 Hz						[2 Marks]

- A transformer steps up voltage from 110 V to 440 V. If the primary coil has 50 turns, how many turns does the secondary coil have?			[5 Marks]

Answer:

VSVP=NSNP=440110=NS50								[2 Marks]

4=NS50 										[2 Marks]

NS=200 turns									[1 Mark]

- List any two applications of Transistor.					[2 Marks]

Answer:

- Transistor as an amplifier							[1 Mark]

- Transistor as switch								[1 Mark]

Question 4 [25 Marks]

- For the circuit shown in fig5, Calculate the following

- Total resistance in the circuit			[6 Marks]

- Total current in the circuit.			[2 Marks]

- Current across each resistor		[10 Marks]

- the voltage across R7 resistor			[2 Marks]

Fig: 5

Answer:

- Total Resistance in the circuit

R1 and R2 are in parallel

Req1=11R1+1R2=116+13=10.5=2Ω						[1 Mark]

R4 and R5 are in parallel

Req2=11R4+1R5=115+110=10.3=3.33Ω						[1 Mark]

Req2 and R3 are in series

Req3=Req2+R3=4+3.33=7.33Ω 					[1 Mark]

R6+R7 in parallel with Req3

Req4=(R6+R7)×Req3R6+R7+Req3=2+4×7.332+4+7.33=3.3Ω					[2 Marks]

Req4 and Req1 are in series

Rt=Req4+Req1=3.3+2=5.3Ω					[1 Mark]

- Total current in the circuit

I=VRt=255.3									[1 Mark]

I=4.71A									[1 Mark]

- Current across each resistor

Current across R1

I1=IR2R1+R2= 4.7×36+3=1.57A							[1 Mark]

Current across R2

I2=IR1R1+R2= 4.7×66+3=3.14A							[1 Mark]

Current across R3

I3=I(R6+R7)R6+R7+Req3= 4.7×(2+4)2+4+7.33=2.1A						[2 Marks]

Current across R4

I4=I3R5R4+R5= 2.1×105+10=1.4A 							[2 Marks]

Current across R5

I5=I3R4R4+R5= 2.1×55+10=0.7A 							[2 Marks]

Current across R6 and R7

I6=IReq3R6+R7+Req3= 4.7×7.332+4+7.33=2.55A						[2 Marks]

- Calculate the current across R7 Resistor

V7=I6R7=2.44×4=10.2V 						[2 Marks]

- What happens when a PN junction diode is forward biased?			[5 Marks]

Answers:

- Depletion region narrows, allowing charge carriers to move across the junction.

- Barrier potential reduces, making it easier for electrons and holes to cross.

- Current flows through the diode due to the movement of majority charge carriers. Voltage drop occurs around 0.7V for silicon diodes.										[Each 2 marks, maximum of 5 Marks]

Diwedd/End