INTERNET PROTOCOLS

By: Ivaylo Ivanov

University of Wales Trinity Saint David

BSc Cloud Computing

Table of content

Introduction…………………………………………………………………..3

Types of IPv6………………………………………………………………….4

Why is IPv6 Better………………………………………………………….5

Conclusion……………………………………………………………………..6

References……………………………………………………………………..7

Introduction

As it has been predicted quite a few years ago, the internet is running out of IPv4 addresses to distribute. Furthermore the “Réseaux IP Européens” or commonly known as RIPE- the European regional internet registry, stated that as of 25th of November 2019 it has run out of IPv4 addresses to allocate (RIPE 2018). Even though they have run out, addresses are still being traded, sold, leased to users and companies alike.

With that being said, this day was anticipated, and a successor was developed all the way back 1995 called IPv6.Taking this into consideration, the transition from one version to the other will not be a quick and easy task to achieve. Most common operating systems such as Windows, OSX and Linux support IPv6.Backward compatibility must be ensured for routers and switches currently operating in order for the transition from one protocol to another to be smooth and seamless as possible (Ben Parr 2011).

To give additional clarity as to why this situation came to be, we will look at the structure and features of both protocols in the following text.

Being developed since the early days of the internet, the Internet protocol IP has been and still is the base for communication between devices on the internet. Today, IPv4 is the main protocol used for these communications. Based on a 32-bit format, comprised by 4 binary octets ranging from 00000001.00000000.00000000.00000000 to 11111111.11111111.1111111.11111111 or translated in to decimal 1.0.0.0 to 255.255.255.255

The number of unique addresses that can be attributed to an IPv4 address are 4^32 or 4,294,967,296 billion (techterms 2009). With the introduction of the internet to home users in the late 1990’s the number of allocated IP’s grew from 16 million in 1995 to 3.3 billion by 2015(Alan Kemp 2016).

IPv6 on the other hand is based on a 128-bit format comprised of 8 groups of 4 hexadecimal digits. Using decimal digits from “0” to “9” the addresses are also comprised of letters “a”,”b”,”c”,”d”,”e”,”f”. Each group Is separated by a colon “:”.

An IPv6 Address looks something like 2001:0db8:0000:0000:abcd:0000:0000:1234, but since this sequence is quite hard to write and pronounce, a shortened version is available which is 2001:db8::abcd:0:0:1234.

With this version of IP address theoretically allows up to 2^128 or 3.4x10^38. Even though the actual number is smaller, because some addresses are either reserved or are excluded of use (Rick Graziani 2017).

Types of IPv6 addresses

Unicast- Used for communication from a single device to another with a single destination, very common to public IPv4 Addresses(Graziani 2017).

Global unicast

Link-local address – Made for communication between two devices in a local link

Loopback address – An address designed for the ability of a host to send a

packet back to himself

Unspecified address – Used only to indicate the absence of an IPv6 address

Unique local- Much like an IPv4 local address and cannot be routed in a IPv6 internet

IPv4 embedded address- An address that has a IPv4 within itself, placed in the last 32 bits.

Multicast

Designed to transmit information from one host to several devices. It serves the purpose of the IPv4 Broadcast address, because IPv6 does not have such a designated one.

Solicited-node address

An automatically created address, assigned to a special multicast interface

Well known multicast address

Used to assign an address to a group of devices such as IPv6 routers and multicast groups.

Anycast address.An address that can be assigned to multiple different devices. This means that multiple devices can have the same anycast address (Graziani 2017)

Why IPv6 is better?

A much bigger pool of addresses

With the virtually endless possible addresses that can be assigned, every single device can have its own uniquely assigned address. While with IPv4 most homes and small businesses are assigned with one IP on their router, and that router would in turn assign internal IPs to the users connected to it and it must always oversee which traffic goes to which device. The transition to IPv6 will make things such as  automation, online gaming, etc. much more easier, not having to do complex setting on your router(Bradley 2012).

Better security

Due to the possibility of IPv4 ICMP packets to carry malware the addresses are often victims of firewalls. While with ICMPV6 may be permitted, because IPSec is pretty much built in to IPV6.(nce 2012)

Better packet processing

Ipv6 has more efficient packet processing due to the fact that it has a simplified packet header. Ipv6 has no IP-level checksum, so not recalculating the checksum on every router hop cuts down time

Directed data flow

IPv6 uses Multicаst rаther then Broadcast. Multicast allows to send intensive packet flows such as multimediа, which in turn saves on bandwith.In addition to that, IPv6 has a field named flow label, which knows what packets belong where.

Better services

With the absence of NAT (Network Address Translation) a true peer-to-peer connectivity will be available which will bring with itself very valuable services, such as Voice over IP (VoIP).

A simpler network connection

In IPv6 addresses are assigned automatically. A host is able to generate it’s own IP address by appending it’s own Media access control address(MAC) and convert it in to an EUI (Extended universal identifier) in a 64 bit format, to the local 64 bit prefix.

Conclusion

Created solely to replace IPv4, because of the latter’s somewhat limited numbers, it has become more than just an IP format with more available addresses. It brings multiple and improved features to the table, which were previously inherently lacking. While it may be a bumpy ride to fully transfer from IPv4, some companies, such as T-mobile USA have already migrated 90% of traffic over IPv6, with more tech giants following suit (Keith Shaw 2018). As we are moving closer to the future of IP addressing, we are yet to see all the perks that this new way of addressing

References:

Bradley T. (2012) ”IPv6: 5 Things you should know” for https://www.pcworld.com/article/257037/ipv6_five_things_you_should_know.html

accessed:01.2020

Editor (2019) “The RIPE NCC has run out of IPv4 addresses” for “Réseaux IP Européens” https://www.ripe.net/publications/news/about-ripe-ncc-and-ripe/the-ripe-ncc-has-run-out-of-ipv4-addresses

accessed: 01.2020

Graziani.R (2017) “IPv6 Address representation and address types” for http://www.ciscopress.com/articles/article.asp?p=2803866&seqNum=4

Accessed: 01.2020

Parr B.(2011)”IPv4 and IPv6: A short guide” for https://mashable.com/2011/02/03/ipv4-ipv6-guide/?europe=true

Accessed: 01 2020

Shaw K. (2018) “What is IPv6 and why aren’t we there yet?” for https://www.networkworld.com/article/3254575/what-is-ipv6-and-why-aren-t-we-there-yet.html

accessed: 01.2020

| School of Applied computing -Network address: 190.168.0.0/22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Room | Subnet mask | Network address | first host | last host | Broadcast |  |
| Open access lab- 250 pc | 255.255.255.0 | 190.168.0.0/24 | 190.168.0.1 | 190.168.0.254 | 190.168.0.255 |  |
| test centre 22 pc | 255.255.255.224 | 190.168.1.0/27 | 190.168.1.1 | 190.168.1.30 | 190.168.1.31 |  |
| Lab 301- 60pc | 255.255.255.192 | 190.168.1.32/26 | 190.168.1.33 | 190.168.1.94 | 190.168.1.95 |  |
| lab 302 - 30pc | 255.255.255.224 | 190.168.1.96/27 | 190.168.1.97 | 190.168.1.126 | 190.168.1.127 |  |
| Lab 303- 36pc | 255.255.255.192 | 190.168.1.128/26 | 190.168.1.129 | 190.168.1.190 | 190.168.1.191 |  |
| Lab 304- 45pc | 255.255.255.192 | 190.168.2.0/26 | 190.168.2.1 | 190.168.2.62 | 190.168.2.63 |  |
| Lab 306 - 40 | 255.255.255.192 | 190.168.2.64/26 | 190.168.2.65 | 190.168.2.127 | 190.168.2.128 |  |
| Lab 308 - 140pc | 255.255.255.0 | 190.168.3.0/24 | 190.168.3.1 | 190.168.3.254 | 190.168.3.255 |  |
| Lab 309 - 50 pc | 255.255.255.192 | 190.168.2.129/26 | 190.168.2.130 | 190.168.2.192 | 190.168.2.193 |  |
| Lab 310 - 18pc | 255.255.255.224 | 190.168.1.192/27 | 190.168.1.193 | 190.168.1.224 | 190.168.1.225 |  |
| Staff 20 pc | 255.255.255.224 | 190.168.2.194/27 | 190.168.2.195 | 190.168.2.226 | 190.168.2.227 |  |

| School of Automotive engineering - Network Address : 190.168.4.0/23 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Room | Subnet mask | Network address | first host | last host | Broadcast |
| Lab 101-40pc | 255.255.255.192 | 190.168.4.0/26 | 190.168.4.1 | 190.168.4.62 | 190.168.4.63 |
| lab 102 -25pc | 255.255.255.224 | 190.168.4.64/27 | 190.168.4.65 | 190.168.4.94 | 190.168.4.95 |
| lab 103-35 pc | 255.255.255.192 | 190.168.4.160/26 | 190.168.4.161 | 190.168.4.222 | 190.168.4.223 |
| lab 205- 40pc | 255.255.255.192 | 190.168.4.96/26 | 190.168.4.97 | 190.168.4.158 | 190.168.4.159 |
| lab 206 - 48 pc | 255.255.255.192 | 190.168.5.0/26 | 190.168.5.1 | 190.168.5.62 | 190.168.5.63 |
| lab 301- 50pc | 255.255.255.192 | 190.168.5.64/26 | 190.168.5.65 | 190.168.5.127 | 190.168.5.128 |
| CAD lab - 20 pc | 255.255.255.224 | 190.168.5.129/27 | 190.168.5.130 | 190.168.5.159 | 190.168.5.160 |
| Staff 25 pc | 225.255.255.224 | 190.168.5.161/27 | 190.168.5.162 | 190.168.5.191 | 190.168.5.192 |

| School of Engineering - 190.168.6.0/23 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Room | Subnet mask | Network address | first host | last host | Broadcast |
| LAB 201- 20PC | 255.255.255.224 | 190.168.6.0/27 | 190.168.6.1 | 190.168.6.30 | 190.168.6.31 |
| LAB 202 - 80PC | 255.255.255.128 | 190.168.6.32/25 | 190.168.6.33 | 190.168.6.158 | 190.168.6.159 |
| LAB 203 - 65 | 255.255.255.128 | 190.168.7.0/25 | 190.168.7.1 | 190.168.7.126 | 190.168.7.127 |
| LAB 205 - 25PC | 255.255.255.224 | 190.168.6.160/27 | 190.168.6.161 | 190.168.6.190 | 190.168.6.191 |
| LAB 404 - 45PC | 255.255.255.192 | 190.168.7.128/26 | 190.168.7.129 | 190.168.7.190 | 190.168.7.191 |
| LAB 4088 - 28PC | 255.255.255.224 | 190.168.7.192/27 | 190.168.7.193 | 190.168.7.222 | 190.168.7.223 |
| LAB 409 - 80 PC | 255.255.255.128 | 190.168.8.0/25 | 190.168.8.1 | 190.168.8.126 | 190.168.8.127 |
| LAB 410 - 45 PC | 255.255.255.192 | 190.168.8.128/26 | 190.168.8.129 | 190.168.8.190 | 190.168.8.191 |
| STAFF - 18 | 255.255.255.224 | 190.168.6.192 | 190.168.6.193 | 190.168.6.222 | 190.168.6.223 |

| Server | IPv6 Address |
| --- | --- |
| DNS | 0:0:0:0:0:ffff:bea8:6e0 |
| email | 0:0:0:0:0:ffff:bea8:6e1 |
| FINANCE | 0:0:0:0:0:ffff:bea8:6e2 |
| FILE | 0:0:0:0:0:ffff:bea8:6e3 |
| REGISTRY | 0:0:0:0:0:ffff:bea8:6e4 |
| PRINT | 0:0:0:0:0:ffff:bea8:6e5 |
| MP | 0:0:0:0:0:ffff:bea8:6e1 |