Draft client letter and outline BEP

BIM – ACBA5010

Cameron Evans 1807303

Word count: 810

Contents

1.0 Client letter………………………………..……………...….3

Outline BEP…………………………………………..…….....…5

3.0 References……………………………………………...….18

4.0 Bibliography……………………………………………….19

Client letter

Dear Swansea council,

Within the letter I will be detailing all the requirements and standards which have to be adhered to in order to achieve BIM level 2 compliance, the project in question being the proposed construction of Ysgol Bae Abertawe. These standards include 1192:2007, BS 8536-1:2015, BS8536-2:2016, PAS 1192-3:2014, BS 1192-4:2014 and PAS 1192-5:2015; with the standard for the BEP being PAS 1192-2. I, myself, am the lead consultant in charge of delivering this ‘state of the art’ infant school and due to the project being joint funded with the welsh government BIM level-2 is the requirement, as of the 4th April 2016, for all government construction projects. Appended after this letter will be the outline BIM execution plan (BEP) which includes the project information, management, planning and documentation, standards, methods and procedures; and IT solutions.

The first stage is everything involved within the pre-construction stage, this includes following the BIM protocol which is simply a legal document included in all government projects to ensure the BIM level 2 requirements are understood and met. Secondly, a project plan or BIM execution plan (BEP) will need to be created in accordance with the exchange information requirements (EIR) and will include the parameters set out in the first paragraph. EIRs cover 3 distinct areas being technical, management and commercial.  Set out in the BEP is the requirements set in BIM level 2 and the agreed formats by stakeholders and other members on how information will be shared, coordinated and managed. The sharing of data will use a common data environment (CDE), set out in PAS 1192-2, which can be split into different information environments specific for user weather that is the project design team, the employer or for the suppliers. For this project the CDE and clash detection programme chosen for this is Revizto which can be used by all members of the project design team and other parties indirectly involved, if data needs to be shared with external parties cloud based drives can be used to record and share this data.

The way in which we aim to coordinate data correctly following standards is by allowing information exchange to the employers to ensure the project is on track, as mention above Revizto will be used for clash detection and the sharing of data/volume zones between the internal team members, it also allows for the structural and architectural drawings to be implemented together so any clash design errors can be seen relatively easy and prior to any construction. Stated in the appended outline BEP is the software and versions that will be used by the separate teams involved in the project delivery team which ensures all the data can be coordinated in a reliable and applicable manner.

Set out in the outline BEP appended it clearly states the roles, responsibilities and authorities set out within the project design team following ISO 19650 standard; all agreed parameters within the BEP should not be changed once the project has begun in the design and construction stage. Within the BEP is the standards, methods and procedures to be used on this project, this includes file naming convections, layer naming conventions, agreed construction tolerances and drawing sheet scales; all these parameters must be included and specified to not only ensure data can be read and found correctly but to prevent any confusion between different members involved in the project team as they may usually work using different standards. This data will be defined in the EIR and then set out in the BEP in accordance with PAS 1192-2.

Once the parameters for the pre-construction stage have been met successfully the project will move into its design and constriction phase. Prior to this stage all the variables set out within the design and construction phase have been set out and agreed within the BEP allowing for problematic free progression. The already agree parameters for this section will be uses in relation to collaboration in the design team, sharing of data though a common data environment (CDE), using the same language on drawings or models and by incorporating the use of a construction-operations building information exchange (COBie). COBie is basically a spreadsheet data format used to store digital information about a building due to the government requirement for all information documentation and data to be electronic. The COBie is aimed to pull all the information like specifications , bills of quantities and schedules into one format so it can be shared between the different construction teams and the correct stages of a project.

After successfully completing these stages whilst following the correct requirements and standards the project comes to the hand over stage, there is no real mention for this in the BEP appended however the aim is always to have a smooth hand over with all the project data being delivered to the client, Swansea council.

Outline BIM Execution Plan (BEP)

5010 – Ysgol Bae Abertawe – 21st Century Schools

Author – Cameron Evans

Word count: 1427

1.0 Outline BIM Execution Plan

This BEP is structured in accordance with PAS1192-2:2013.

The objectives set out in a BEP are to ensure that the correct implementation of BIM is used following the requirements set by the Welsh Assembly. BIM is defined by Autodesk as:

‘Building Information Modeling (BIM) is an intelligent 3D model-based process that gives architecture, engineering, and construction (AEC) professionals the insight and tools to more efficiently plan, design, construct, and manage buildings and infrastructure.’

2.0 Project Information

| Project Owner | Swansea Council |
| --- | --- |
| Project Name | Ysgol Bae Abertawe |
| Project Location | Green site adjacent to Swansea Observatory located in the Maritime Quarter |
| Project Description | Plan and design a new 1 form entry infant school for 90 pupils |

Contents

2.0 Project information…………………………………………..2

3.0 Management…………………………………………………..4

3.1 Roles, Responsibilities and Authorities……………….4

3.2 Existing Legacy Data Use………………………..…….4

3.3 PIM Authorisation Process……………………………..4

4.0 Planning and Documentation……………………….……..5

4.1 Revised Project Implementation Plan……….…….…..5

4.2 Approved Software and Versions………….…………..6

4.3 Common Data Environment………………..……….…..6

4.4 Task Information Delivery Plan……….………………..8

4.5 Master Information Delivery Plan…………..………….8

5.0 Standards, Methods and Procedures……….……………9

5.1 Volume Strategy………………………………………….9

5.2 PIM Origin and Orientation………………………..……9

5.3 File Naming Convections………………………..………9

5.4 Layer Naming Convections………………………...….10

5.5 Agreed Construction Tolerances for all Disciplines…10

5.6 Drawing Sheet Templates…………………………..…10

5.7 Annotations, Dimensions, Abbreviations and Symbols………………………………………………………11

5.8 Attribute Data……………………………………………11

6.0 IT Solutions…………………………………………………..12

6.1 Software Versions……………………………..……….12

6.2 Exchange Formats…………………………………..…12

6.3 Process and Data Management Systems……………13

3.0 Management

This section covers the requirements of PAS1192-2 Clause 7.2.1 a).

3.1 Roles, Responsibilities and Authorities

Table 1 – Roles and responsibilities

| Role | Name | Organisation |
| --- | --- | --- |
| Client | Swansea Council | Welsh Government |
| End user | TBC |  |
| Lead Consultant | Cameron Evans | UWTSD |
| Lead designer | TBC |  |
| Project Information manager | TBC |  |
| Task Team manager | TBC |  |
| Interface manager | TBC |  |
| Task team information manager | TBC |  |
| CAD coordinator | TBC |  |

Table 2 – Role authorities

| Role | Authority |
| --- | --- |
| Project information manager | Enforce the project BIM standard and ensure delivery of the information requirement in the EIR |
| Lead designer | Enforce spatial coordination |
| Task team manager | Enforce documentation standards |
| Interface manager | Negotiate space allocation |
| Task team information manager | Reject non-compliant models, drawings and documents |
| CAD coordinator | Enforce CAD related project BIM standards |

3.2 Existing Legacy Data Use

As approved and defined by the EIR

3.3 PIM Authorization Process

The design and construction process should be specified, agreed and documented as early as possible in the project. This includes the sign off of specialist design completion information.

Sign off and authorization process to be agreed and published in coordination with the abilities of the collaboration or EDMS solution.

4.0 Planning and Documentation

This section covers the requirements of PAS1192-2 Clause 7.2.1 b).

4.1 Revised Project Implementation Plan (PIP)

The PIP created in the pre-contract BEP consists of the following completed CPIx documentation:

- Supply chain capability summary form

- Supplier building information management assessment form(s)

- Supplier IT assessment form(s)

- Supplier resource assessment form(s)

Table 3 - Revised PIP confirming competence and capability of supply chain

| Supplier | Discipline | Resource number | Levels of competence | Years of experience | Name of individuals |
| --- | --- | --- | --- | --- | --- |
| TBC | Qualified Architect | 2 | RIBA, CAD/BIM, Specification Author | TBC | TBC |
| TBC | Architectural Technologist | 2 | CIAT, CAD/BIM trained | TBC | TBC |
| TBC | Architectural Technician | 4 | Certificate of CAD or model competence | TBC | TBC |
| TBC | Qualified structural Engineer | 3 | MISTructE | TBC | TBC |
| TBC | Quantity Surveyor | 2 | MRICS, CAD/BIM trained | TBC | TBC |
| TBC | Project construction Manager | 1 | MCIOB | TBC | TBC |

4.2 Agreed Project Processes for Collaboration and Information Modelling

Table 4 – Processes of Collaboration and Information Modelling

| Company | Solution | Network | Database | File Based | Comments |
| --- | --- | --- | --- | --- | --- |
| Design team | TBC | TBC | Revizto | TBC |  |
| Client | TBC | TBC | Revizto | TBC |  |
| TBC |  |  |  |  |  |

The clash rendition viewer to be used across the whole project.

Table 5 – Clash rendition viewer

| Agreed clash rendition viewer | Version |
| --- | --- |
| Revizto | 5.3 |

4.3 Common Data Environment

To support BIM workflows, communication must be controlled and methodical. Team members should communicate frequently and interactions (including digital communication and records of meetings) should be stored on a CDE and made available to appropriate members of the project team. Access to information should be determined through engagement between the information manager, Swansea council and other project stakeholders.

Table 6 – Common Data Environment (CD)

| CDE for this project | Version |
| --- | --- |
| Revizto | 5.3 |

Figure 1 – CDE Collaboration Process

4.4 Task Information Delivery Plan (TIDP)

A TIDP for each task within the project shall be prepared using the template below so that the format is consistent with the format required for the master information delivery plan.

Table 7 – Template for Task Information delivery Plan

| File Identifier |  |  |  |  |  |  | Model/ drawing title | Delivery dates |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project | Originator | Volume | Level | File type | Discipline | No. |  | Milestone 1 | Milestone 2 |  |  |

4.5 Master Information Delivery Plan (MIDP)

The master information delivery plan shall be developed from the separate TIDPs produced for each task within the project. This more detailed, co-ordinated MIDP must be developed and agreed with stakeholders.

5.0 Standards, Methods and Procedures

This section covers the requirements of PAS1192-2 Clause 7.2.1 c).

5.1 Volume Strategy

Each individual team will provide a copy of their volume strategy in the form of a drawing of each floor layout, section or site with volumes clearly marked, named and inserted below. For volumes relating to infrastructure refer to PAS1192-2:2013.

5.2 PIM Origin and Orientation

Table 8 – Record of Information Model Origin and Orientation

| Point | Grid intersection | Easting (m) | Northing (m) | Elevation of site datum |
| --- | --- | --- | --- | --- |
| Site local grid origin | TBC | TBC | TBC | TBC |
| Grid origin bottom left intersection | TBC | TBC | TBC | TBC |
| Grid intersection bottom right | TBC | TBC | TBC | TBC |
| Grid intersection top left | TBC | TBC | TBC | TBC |
| Grid intersection top right | TBC | TBC | TBC | TBC |

5.3 File Naming Convection

Table 9 – template for file naming(BS1192:2007)

| Project | Originator | Volume | Level or locations | File type | Role | Number |
| --- | --- | --- | --- | --- | --- | --- |

5.4 Layer Naming Convention

Each discipline should provide the design manager and the CAD manager with a full list of all layer names to be used on the project. This list is to be published to all members of the project team for information.

Table 10 – Layer Naming Convention Template

| Field |  | Discipline |  | Classification |  | Presentation |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

5.5 Agreed Construction Tolerances for all Disciplines

Table 11 – Agreed Tolerances for construction elements according to project Discipline template

| Discipline | Element | Tolerance |
| --- | --- | --- |

5.6 Drawing Sheet Templates

All drawing templates must be rendered and presented at one of a number of approved scales, which are typically defined by the ‘CAD Manager’. Scales other than those approved should not be used. The templates shall also be in the standard format for sharing and interoperability.

Table 12 – List of drawing sheet templates

| Sheet Size | Sheet file Name |
| --- | --- |

Table 13 – Drawing sheet scales

| Scale | Description of detail |
| --- | --- |
| 1:1000 | 1:1000 scale detail shows shape and layout |
| 1:500 |  |
| 1:200 |  |
| 1:100 | 1:100 scale detail shows shape, layout and construction elements |
| 1:50 |  |
| 1:20 |  |
| 1:10 |  |
| 1:5 | 1:5 scaled detail shows shape, dimension and assembly of the separate construction elements |
| 1:2 |  |
| 1:1 | All model files must be modelled at 1:1 scale |

5.7 Annotations, Dimensions, Abbreviations and Symbols

The project team should agree common units of measurements to be used on the project. (BS8541 parts 1, 2, 3 and 4)

Table 14 – Agreed units of measurement

| Type of information | Millimetres | Meters | Degrees | Radians | Clockwise | Counter |
| --- | --- | --- | --- | --- | --- | --- |
| Survey |  |  |  |  |  |  |
| Roads |  |  |  |  |  |  |
| Models |  |  |  |  |  |  |
| Drawings |  |  |  |  |  |  |

5.8 Attribute data

Attribute dada for the project as defined in the EIR and appropriate COBie templates.

6.0 IT Solutions

This section covers the requirements of PAS1192-2 Clause 7.2.1 d).

6.1 Software versions

Software versions shall be agreed at the outset and not deviated from. Please note any changes to software and upgrades to versioned software to be used on this project must be agreed by all parties prior to actioning any upgrade.

Only tools identified below shall be used to author, coordinate and validate BIMs. This is to facilitate interoperability between software platforms and enable the successful execution of the interoperability strategy.

Table – 8 Agreed software versions

| Discipline | Software and versions |
| --- | --- |
| Architectural | Autodesk Revit 2021 / Revizto 5.3 |
| Interior Design | Autodesk Revit 2021 / Enscape3D 3.0 |
| Landscape Architecture | Autodesk Revit 2021 / Revizto 5.3 |
| Structural | Autodesk Revit 2021 / Revizto 5.3 |
| Buildings Services Engineer | Autodesk Revit 2021 / Revizto 5.3 |
| Civil Engineer | Autodesk Revit 2021 / Revizto 5.3 |
| Structural Engineer | Autodesk Revit 2021 / Revizto 5.3 |
| Cost Manager | Autodesk Revit 2021 |
| Facilities Manager | Autodesk Revit 2021 / Revizto 5.3 |
| Collaboration | Revizto 5.3/ Enscape3D 3.0 |

6.2 Exchange Formats

The agreed formats for model and drawing file exchange.

Table 9 – Agreed exchange formats for models and drawings

|  | DWG | DGN | DWF | PDF | IFC | Other |
| --- | --- | --- | --- | --- | --- | --- |
| Models |  |  |  |  |  |  |
| Drawings |  |  |  |  |  |  |
| Final drawing format |  |  |  |  |  |  |
| Schedules or spreadsheets |  |  |  |  |  |  |

6.3 Process and Data Management Systems

The process of data management systems shall be described under section 4.2 – Agreed Project Process for Collaboration and Information Modelling.

References

CPIx on Line (2013) Post Contract-Award Building Information Modelling (BIM) Execution Plan (BEP). Available at: http://www.cpic.org.uk/wp-content/uploads/2013/06/cpix_post_contract_bim_execution_plan_bep_r1.0.pdf (Accessed: 20th April)

GlobalCAD (2019) The BIM execution plan, how important is it? Available at: https://www.globalcad.co.uk/the-bim-execution-plan-how-important-is-it/ (Accessed: 20th April)

Cambridge University (2015) Building information modelling (BIM) execution plan. Available at:  https://www.em.admin.cam.ac.uk/files/3_uoc_bimexecutionplan_v1.1.1.pdf (Accessed: 20th April)

Autodesk BIM 360. What is BIM Level 2. Available at: https://bim360.autodesk.com/what-is-bim-level-2/ (Accessed: 20th April)

Linkedin (2016) What is a common data environment. Available at: https://www.linkedin.com/pulse/what-common-data-environment-himani-shah?trk=mp-reader-card (Accessed: 28th April)

Bibliography

NBS (2019) National BIM report 2019. Available at: https://www.thenbs.com/knowledge/national-bim-report-2019 (Accessed: 21st April)

BSi (2020) Little book of BIM 2020 international edition. Available at: https://www.bsigroup.com/globalassets/localfiles/en-us/brochures/bim/little-book-bim.pdf (Accessed: 21st April)

UK BIM Framework (2020) Open data, building SMART and COBie. Available at: https://ukbimframework.org/wp-content/uploads/2020/09/Guidance-Part-B_Open-data-buildingSMART-and-COBie_Edition-1.pdf (Accessed: 21st April)

UK BIM Framework (2020) Information Protocol to support BS EN ISO 19650-2 the delivery phase of asset. Available at: https://ukbimframework.org/wp-content/uploads/2020/06/Information-Protocol-to-support-BS-EN-ISO19650-2.pdf (Accessed: 21st April)

UK BIM Framework (2021) Information management according to BS EN ISO 19650. Available at: https://www.ukbimframework.org/wp-content/uploads/2021/02/Guidance-Part-2_Parties-teams-and-processes-for-the-delivery-phase-of-assets_Edition-6.pdf (Accessed: 21st April)

NBS (2017) What is a BIM execution plan (BEP)? Available at: https://www.thenbs.com/knowledge/what-is-a-bim-execution-plan-bep (Accessed: 21st April)

NBS (2018) What is COBie? Available at: https://www.thenbs.com/knowledge/what-is-cobie (Accessed: 26th April)

BIM Portal Scottish Futures trust. Stage 3, Definition, Prepare MIDP/RACI/TIDP, Task information delivery plan. Available at: https://bimportal.scottishfuturestrust.org.uk/level2/stage/3/task/39/task-information-delivery-plan (Accessed: 27th April)

BIM Wiki (2020) Project Information model PIM. Available at: https://www.designingbuildings.co.uk/wiki/Project_information_model_PIM (Accessed: 27th April)

BIM Portal Scottish Futures Trust. Stage 1, Brief, BIM level 1 approach, File Naming Convention. Available at: https://bimportal.scottishfuturestrust.org.uk/level2/stage/1/task/47/file-naming-convention (Accessed: 27th April)

BIM Wiki (2020) BIM Volume. Available at: https://www.designingbuildings.co.uk/wiki/BIM_volume (Accessed: 27th April)

BIM Wiki (2021) Employers Information Requirements EIR. Available at: https://www.designingbuildings.co.uk/wiki/Employer%27s_information_requirements_EIR#:~:text=Building%20Information%20Modelling%20(BIM)%20is,bridge%2C%20tunnel%20and%20so%20on (Accessed: 27th April)

BIM Wiki (2020) Project Implementation plan for construction projects. Available at: https://www.designingbuildings.co.uk/wiki/Project_implementation_plan_for_construction_projects (Accessed: 27th April)

Asite (2021) Why a common data environment is essential for your project? Available at: https://www.asite.com/blogs/what-is-a-common-data-environment#:~:text=A%20Common%20Data%20Environment%20(CDE,updated%20throughout%20the%20project%20lifecycle (Accessed: 27th April)

BAM vdc (2016) Achieving level 2 BIM – Schools bundle 4. Available at: https://www.bam.com/sites/default/files/domain-606/documents/achieving_level_2_bim-606-1488892515854407486.pdf (Accessed: 27th April)

BIM Wiki (2021) Exchange information requirements. Available at: https://www.designingbuildings.co.uk/wiki/Exchange_information_requirements (Accessed: 27th April)

BIM Wiki (2020) PAS1192-2. Available at: https://www.designingbuildings.co.uk/wiki/PAS_1192-2 (Accessed: 28th April)

BIM Wiki (2021) Common Data Environment CDE. Available at: https://www.designingbuildings.co.uk/wiki/Common_data_environment_CDE (Accessed: 28th April)

Building information modelling Scottish Futures trust. Determin the info management & CDE strategy. Available at: https://bimportal.scottishfuturestrust.org.uk/level2/stage/1/task/2/determine-the-info-management-cde-strategy (Accessed: 28th April)