Contents

a. Calculate the resistance of E and H for the circuit’s configuration. 	[3]	1

b. B and C are identical and the current passing through A is 0.5A, calculate the current passing through B and C. [2]	1

c (i) The Resistance of A. [2]	1

c (ii) The Resistance of B and C [3]	1

c (iii) The potential difference across A, B and C [1]	1

c (iv) Since the total potential difference across L to M is 6V, calculate the potential difference across D and find its Resistance. [2]	2

d. The ‘lost volts across one of the cells in the parallel arrangement is 2 V. Calculate r and hence the total resistance of the parallel combination of cells. [3]	2

e. Given that the resistance F is 6Ω, calculate the voltage drop across it. [2]	2

f. Calculate the total resistance of the circuit, this includes total internal and the resistance cause by the reverse polarity battery and the open diodes in given scenario (ignore any “non-open diodes”). [2]	2

g. Calculate the emf of each of the cells in the parallel combination. [3]	2

## a. Calculate the resistance of E and H for the circuit’s configuration. 	[3]

VE=0.6V

I=1A

Rearrange Ohms Law for resistance:R =VI

V=0.6

RE= 0.61

RE=0.6Ω

Diode H will not have any resistance as the diode E has the lower breakdown voltage and therefore a lower resistance for the circuit to flow.

## b. B and C are identical and the current passing through A is 0.5A, calculate the current passing through B and C. [2]

Kirchoffs Current Law is applied to this question I1=I2+ I3

I=1A

I=1-0.5

I =0.5

I= 0.52

Current for B and C is equal to 0.25Amps.

## c (i) The Resistance of A. [2]

1.25 Js=1.25 Watts

P=1.25 Watts

P= I2 ×R

Rearange to find R

R= PI2

R= 1.250.52

R=5Ω

## c (ii) The Resistance of B and C [3]

R= 2.50.25

R= 10Ω

Resistors B and C have a resistance of 10Ω.

## c (iii) The potential difference across A, B and C [1]

Resistors in parrallel formula; R= 11R+1R+1R

R= 115+210

R=2.5Ω

V= 2.5×1

V=2.5V

## c (iv) Since the total potential difference across L to M is 6V, calculate the potential difference across D and find its Resistance. [2]

V=6-2.5-0.6

Potential difference=2.9V

R= 2.91

R=2.9Ω

Resistor D has a resistance value of 2.9Ω.

## d. The ‘lost volts across one of the cells in the parallel arrangement is 2 V. Calculate r and hence the total resistance of the parallel combination of cells. [3]

R= 20.25

r=8Ω

RP=148

RP=2Ω

## e. Given that the resistance F is 6Ω, calculate the voltage drop across it. [2]

V=I×R

V=1×6

Voltage drop=6V

## f. Calculate the total resistance of the circuit, this includes total internal and the resistance cause by the reverse polarity battery and the open diodes in given scenario (ignore any “non-open diodes”). [2]

RT=0.6+115+210+2.9+6+6+2

RT=20Ω

## g. Calculate the emf of each of the cells in the parallel combination. [3]

E.M.F

E.M.F=2.5+2.9+0.6+6+2+6

E.M.F=20V