| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2023 | 2023 |

MODULE CODE: ACDC5009

Electromagnetic Radiation

Programme:

B.Eng.(Hons.) iN Embedded Electronic Systems

TIME:  2 HOURS

answer Any Five questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ANY 5 QUESTIONS

Question 1 [20 marks]

- Define an Electromagnetic wave. 						[2 marks]

- Describe the properties of Electromagnetic waves. 				[6 marks]

- How does an Electromagnetic wave propagate.					[2 marks]

- What is an Electromagnetic Spectrum? Describe the features and applications of Radio Frequency waves.									[8 marks]

- Define a TE mode.								[2 marks]

Question 2 [20 marks]

- Define Polarisation. Describe the types of Polarisation with the aid of necessary diagram.											[10 marks]

- A Charge Q1= -20μC is located at P(-6,4,6) and a charge Q2= 50μC is located at R (5,8,-2) in a free space. Find the force exerted on Q2 by Q1. The distance given are in meters.										[10 marks]

Question 3 [20 marks]

- Describe the following terms used in Electromagnetic Radiation with necessary diagram,

- Diffraction, 								[2 marks]

- Total Internal Reflection, 						[2 marks]

- Scattering 								[2 marks]

- Interference								[2 marks]

- Refraction									[2 marks]

- What is a waveguide? Describe the types of optic fibre waveguide with necessary diagrams.										[10 marks]

Question 4 [20 marks]

- A Volume charge density is expressed as ρv=10Z2xsinπy. Find the total charge inside the volume (-1≤x≤2), (0≤y≤1) and (3≤Z≤3.6)			[6 marks]

- A silica optical fiber with a core diameter large enough to be considered by ray theory analysis has a core refractive index of 1.50 and a cladding refractive index of 1.47. Determine the critical angle at the core–cladding interface and the acceptance angle in air for the fiber.									[4 marks]

- Describe the construction and working of a laser with the aid of necessary diagrams.												[8 marks]

- Describe the function of an isolator in a waveguide.				[2 marks]

Question 5 [20 marks]

- Find the inductance per unit length of a co-axial cable radius of inner and outer conductors are 1mm and 3mm respectively. Assume relative permeability as unity.

[4 marks]

- Calculate the characteristic impedance , the attenuation constant and phase constant of a transmission line if the following measurements have been made on the line as, ZOC=550Ω∠-60oΩ and ZSC=500Ω∠-14oΩ.					[16 marks]

Question 6 [20 marks]

- A plane TEM wave has a power density of 12ω/m2 in a medium with εr=3,μr=1. Find E and H.									[6 marks]

- Define Doppler Effect. With example						[ 4 marks

- Describe the working of a Pulse radar, with the aid of a functional Block diagram.												[10 marks]

Diwedd/End