Control & Automation revision questions

- A temperature-measuring system has the following component sensitivities;

Transducer -		350 mΩ/°C

Wheatstone bridge - 	10 mV/Ω

Amplifier gain -	250 V/V

Recorder - 		0.2 cm/V

Determine the overall system sensitivity, and what is the temperature change if the recorder registered a movement of 8cm?  [0.175 cm/°C; 45.71°C]

- An angular displacement measuring system has the following component sensitivities;

Displacement transducer -	120 mV/mm

Amplifier gain -		100 V/V

Pen recorder - 		0.5 deg/V

Determine the overall system sensitivity, and if the recorder displayed a movement of 15 deg, what would the displacement have been?  [6 deg/mm; 2.5 mm]

- A simple hydraulic fluid system comprises of a resistance to flow (R) in series with an accumulator that acts as the hydraulic capacitance (C) in the system.  When subject to a sudden supply pressure (P), i.e. when a valve is suddenly opened fully, it is found to produce an output pressure drop across the hydraulic capacitance, pc which is given by the differential equation;

RC dpcdt+ pc=P

What is the solution of this differential equation, i.e. how does pc vary with time?

[Ans:  pc=P 1- e- tRC]

What is the time constant of the system, and show that when time, t, is equal to 1 time constant, the pressure drop across the hydraulic capacitance is approximately 63% of the supply pressure P.

- A simple float-controlled water level system within an enclosed tank is found to have the following differential equation;

1kdhdt+h=H

Where, k is a constant of proportionality, h is the instantaneous height of the water level in the tank, and H is the final level of the water.  What is the solution of the system differential equation, and if k has a value of 4, what will be the approximate time for the system to reach H?

[ h=H(1- e-kt); 1.25s]

- A second-order system is described by the following differential equation

0.1 d2ydt2+0.2 dydt+2.5 y=2.5 x

Where, x is the input of the system and y the output.  Determine

- The undamped natural frequency of the system, and		[5Hz]

- The damping ratio.							[0.2]

- A second-order mechanical system is modelled as a mass/spring/damper arrangement, whereby the differential equation is given as;

m d2xdt2+c dxdt+kx=F

Force F is the applied step input to the system, where mass m = 5kg, coefficient of damping C = 25 Ns/m, spring stiffness k = 200 N/m, and F = 50N.

Determine the damping ratio, and hence state whether the system is under-damped, over-damped, or critically damped.				[0.395, under-damped]

Determine the percentage overshoot for the system.	[approx. 25%]

Solution:

First convert given system differential equation into standard form by dividing through by m, so that;
 d2xdt2+cmdxdt+kmx=Fm

Now input given data and simplify;

d2xdt2+5 dxdt+40x=10

Comparing directly with the second-order standard form yields;

ωn= 40=  6.325 Hz

And,		2ζωn=5   ∴ ζ=0.395	hence, system is under-damped.

Using this value of damping ratio in equation for percentage maximum overshoot will yield a value of approximately 25.4%.