Fundamentals of Electrical and Electronic systems

ACBD4018

Rory Bowen

Astable 555 Timer Logbook

Supervisor:	 Olubenga Olubodun

July 2025

Academic integrity declaration

I declare that:

- the thesis submitted has been written by me alone.

- the thesis submitted has not been previously submitted to this university or any other.

- that all content, including primary and/or secondary data, is true to the best of my knowledge.

- that all quotations and references have been duly acknowledged according to the requirements of academic research.

I understand that to knowingly submit work in violation of the above statement will be considered by examiners as academic misconduct.

Abstract

This report outlines the design and evaluates the students as table 555 timer. The resistor values were calculated using equations, listed in the report, from assumed LED currents. Proteus Professional 8 was used to generate simulation to produced timing tables and that matched expected theoretical predictions. The 555-timer circuit was then assembled on a breadboard, and a multi-meter was used to measure capacitance charge and discharge. The report shows troubleshooting steps to further improve reproducibility of results to have a higher confidence in results. The results confirm the 555 timer’s ability to produce astable pulse shown by a blinking of the different coloured LEDs. The value of capacitance it was then plotted to show how the simulated and practical circuits produced the same rough graphs.

Table of Contents

Academic integrity declaration	i

Abstract	ii

List of Figures	iv

List of Tables	iv

List of Equations	iv

List of Abbreviations	v

1 Circuit	6

1.1 Description	6

2 Schematics	7

2.1 Simulated	7

3 Design Calculations	9

3.1 Resistance	9

3.2 Selected Components	9

3.3 Timing Equations	9

3.4 On/Off Time	9

3.5 Period	10

3.6 Frequency	10

4 Simulated Measurements	11

4.1 Timing	11

4.2 Graphing	11

5 Breadboard	14

5.1 Building	14

5.2 Testing	15

5.3 Analyse of Data	16

5.4 Graphical Analysis – Breadboard Results	17

6 Conclusion	18

7 References	19

Appendix A Graphing Data	20

# List of Figures

Figure 1 - Circuit for Astable 555 Timer simulated on Proteus	6

Figure 2 - Lecturer given example of 555 Circuit	7

Figure 3 - Graph of capacitor (C1) voltage and output voltage against time for simulated circuit	10

Figure 4 - Graph made from Proteus simulation Data points	11

Figure 5 - Breadboard 555 Timer Circuit	14

Figure 6 - Graph showing the C1 Capacitor Voltage and Output Voltage against time on the built bread board circuit.	16

List of Tables

Table 1 - Proteus simulation data points	10

Table 2 - Breadboard Data Points	15

List of Equations

Equation 1 - t_on=0.693 × (R_3+R_4) × C_1	8

Equation 2 - t_off=0.693 × R_4 × C_1	8

Equation 3 - T= t_on+ t_off	8

Equation 4 - f= 1/T	8

# Circuit

## Description

This report contains a computer-based simulation, and a physical construction of an astable 555 timer integrated circuit (IC). The astable configuration, of the 555 timer operates as an oscillator, continuously generating a square waveform without the need for an external trigger.

The timing of the circuit is determined by two resistors (R3 and R4) and a capacitor (C1). These components determine the charging and discharging cycles of the capacitor, which defined the pulse duration and frequency of the output wave. The function of each component was necessary to the generation of consistent and predictable oscillations in square waves. The following bullet points list outlines the functional roles of the key components.

- Functional Role of Key Components

- 555 Timer IC: Functions as the core of the pulse square wave, which produced a high and low output at regular intervals.

- Resistors R3 and R4: Determine the timings of the charging and discharging phases of the capacitor C1.

- Capacitor C1: Controls the rate of voltage change across pin 6 and 2, which influenced the timings of the output.

# Schematics

## Simulated

Figure 1 - Circuit for Astable 555 Timer simulated on Proteus

The circuit shown in the photo above was drawn with the assistance of the given 555 timer circuit listed below.

Figure 2 - Lecturer given example of 555 Circuit

The proposed circuit was modelled and simulated using Proteus Professional 8 simulation software. The software’s component library was used to build the schematic. The simulation was designed with a 9V DC power supply. Voltage probes and oscilloscope channels were placed across the output (pin 3) and the timing capacitor (C1) to observe square waveforms.

The simulation parameters included time-base adjustments and a waveform capture setting which allowed the student to gather an accurate reading of the voltage across C1 charging, discharging and timing intervals.

# Design Calculations

## Resistance

To determine the timings of the astable multivibrator, the timing equations for 555 timers were applied. Resistors R3 and R4 were calculated with the said timing equations. The LED voltage was cross referenced with references [1] [2] [3] [4]

## Selected Components

- R = Vsupply- VLEDA

- R_3 = 9- 320x10-3 = 300Ω

- R_4 = 9- 220x10-3 = 350Ω

## Timing Equations

Equation 1 - t_on=0.693 × (R_3+R_4) × C_1

ton=0.693 × R3+R4 × C1

Equation 2 - t_off=0.693 × R_4 × C_1

toff=0.693 × R4 × C1

Equation 3 - T= t_on+ t_off

T= ton+ toff

Equation 4 - f= 1/T

f= 1T

## On/Off Time

ton=0.693 × R3+R4 × C1

ton=0.693 × 300+350 × 100μF = 0.045s

toff=0.693 × R4 × C1

toff=0.693 × 350 × 100μF = 0.024s

## Period

T= ton+ toff

T= ton+ toff = 0.069s

## Frequency

f= 1T

f= 1T = 14.43Hz

These calculations provided a basis for verifying the simulation and experimental results.

# Simulated Measurements

## Timing

Due to Proteus simulation software having its own graphing software the following graph in section 4.2 was produced. However, the data of said graph is not produced so the following table shows the voltage at second intervals which still produced a good graph as seen in figure 4

## Graphing

Figure 3 - Graph of capacitor (C1) voltage and output voltage against time for simulated circuit

Table 1 - Proteus simulation data points

|  | Proteus |
| --- | --- |
| Time (s) | Voltage across C2 100uF (v) |
| 0 | 9.00 |
| 1 | 3.35 |
| 2 | 5.58 |
| 3 | 4.93 |
| 4 | 3.47 |
| 5 | 5.76 |
| 6 | 5.05 |
| 7 | 3.57 |
| 8 | 5.67 |
| 9 | 4.91 |
| 10 | 3.53 |
| 11 | 5.71 |
| 12 | 4.64 |
| 13 | 3.55 |
| 14 | 5.87 |
| 15 | 4.85 |
| 16 | 3.62 |
| 17 | 5.85 |
| 18 | 4.96 |
| 19 | 3.48 |
| 20 | 5.71 |
| 21 | 4.80 |
| 22 | 3.52 |
| 23 | 6.07 |
| 24 | 4.84 |
| 25 | 3.56 |
| 26 | 6.12 |
| 27 | 4.81 |
| 28 | 3.36 |
| 29 | 6.05 |
| 30 | 4.72 |

Figure 4 - Graph made from Proteus simulation Data points

The output waveform from pin 3 displays a charging and discharging wave with a predictable cycle and frequency. The consistent wave corresponding with the expected exponential charge and discharge profiles. The confirmed the proper operation of the astable 555 timer. The transition between voltage levels is sharp, which help to indicate that the capacitor and resistors are functioning and are acceptable.

# Breadboard

## Building

The breadboard circuit was constructed on a solderless breadboard using an identical component value to those used in the simulation. A regulated 9V DC power source was used for powering the circuit. The following bullet points list the components used in the circuit

- Component List

- 555 Timer IC

- R_1: 1kΩ

- R_2: 10KΩ

- R_3: 10KΩ

- R_4: 10KΩ

- C_1: 100µF

- Breadboard

- Power supply: 9v bench power supply

## Testing

Figure 5 - Breadboard 555 Timer Circuit

A systematic approach was used to validate the astable 555 timer performance. The following steps were executed:

- The 555 Timer IC was properly oriented and inserted into the breadboard.

- Resistors R3 and R4 were connected between Vcc, pin 7 (discharge), and pin 6 (threshold).

- The timing capacitor C1 was connected from pin 6/2 to ground.

- Pins 6 and 2 were connected to facilitate threshold-trigger coupling.

- Pin 3 (output) was monitored using an oscilloscope.

- The power supply was connected, and the circuit was activated.

- Measurements of waveform parameters were obtained using an oscilloscope and multimeter.

- Observations were made to evaluate consistency with theoretical and simulated results.

## Analyse of Data

The following table outlines that C1 charges and discharges between 1/3 and 2/3 of the supply voltage, this helps to show the circuit was functioning as intended. This behaviour matches an astable 555 timer when the data is plotted the voltage pattern forms a square-like waveform alternating between 2.6v and 5.6v.

Table 2 - Breadboard Data Points

|  | BreadBoard |
| --- | --- |
| Time (s) | Voltage across C2 100uF (v) |
| 0 | 0 |
| 1 | 4.58 |
| 2 | 2.58 |
| 3 | 5.46 |
| 4 | 2.76 |
| 5 | 5.55 |
| 6 | 2.64 |
| 7 | 5.6 |
| 8 | 2.61 |
| 9 | 5.63 |
| 10 | 4.37 |
| 11 | 2.66 |
| 12 | 4.31 |
| 13 | 5.67 |
| 14 | 2.6 |
| 15 | 5.66 |
| 16 | 4.13 |
| 17 | 2.62 |
| 18 | 4.12 |
| 19 | 2.66 |
| 20 | 5.62 |
| 21 | 2.69 |
| 22 | 5.62 |
| 23 | 2.7 |
| 24 | 4.18 |
| 25 | 2.7 |
| 26 | 5.26 |
| 27 | 2.7 |
| 28 | 4.06 |
| 29 | 5.6 |
| 30 | 3.95 |

## Graphical Analysis – Breadboard Results

Figure 6 - Graph showing the C1 Capacitor Voltage and Output Voltage against time on the built bread board circuit.

Analyse of this graph shows the oscillations are periodic, which matches the simulated results of a repeating charge-discharge cycle. The consistency of the peaks and troughs implied the capacitor charges and discharges in response to a standard capacitor. The oscillations are due to a square wave, in which the regular oscillations suggest the circuit is working correctly.

# Conclusion

The measured wave on the physical breadboard showed a correlation between both the theoretical predictions and simulation results, however there were some minor differences were observed. firstly, the differences could be attributed to the component tolerances, and potential breadboard connection resistances. Despite these small differences, the output waveform maintained a stable frequency and consistent cycle, which help to validate the design.

# References

| [1] | D. Lida, “Demonstration of low forward voltage InGaN-based red LEDs,” Applied physics express, vol. 3, no. 13, p. 031001, 2020. |
| --- | --- |
| [2] | R.-H. Horng, “Fabrication and study on red light micro LED displays,” IEEE Journal of the Electron devices Society, no. 6, pp. 1064-1069, 2018. |
| [3] | S. Nakamura, “Blue-green light-emitting diode and violet laser diodes,” MRS Bulletin, vol. 2, no. 22, pp. 29-35, 1997. |
| [4] | S. Hisahiro, “Extremely low operating voltage green phosphorescent organic light-emitting devices,” Advacnced Functional Materials, vol. 44, no. 23, pp. 5550-5555, 2013. |

Graphing Data

|  | BreadBoard | Proteus |
| --- | --- | --- |
| Time (s) | Voltage across C2 100uF (v) | Voltage across C2 100uF (v) |
| 0 | 0 | 9.00 |
| 1 | 4.58 | 3.35 |
| 2 | 2.58 | 5.58 |
| 3 | 5.46 | 4.93 |
| 4 | 2.76 | 3.47 |
| 5 | 5.55 | 5.76 |
| 6 | 2.64 | 5.05 |
| 7 | 5.6 | 3.57 |
| 8 | 2.61 | 5.67 |
| 9 | 5.63 | 4.91 |
| 10 | 4.37 | 3.53 |
| 11 | 2.66 | 5.71 |
| 12 | 4.31 | 4.64 |
| 13 | 5.67 | 3.55 |
| 14 | 2.6 | 5.87 |
| 15 | 5.66 | 4.85 |
| 16 | 4.13 | 3.62 |
| 17 | 2.62 | 5.85 |
| 18 | 4.12 | 4.96 |
| 19 | 2.66 | 3.48 |
| 20 | 5.62 | 5.71 |
| 21 | 2.69 | 4.80 |
| 22 | 5.62 | 3.52 |
| 23 | 2.7 | 6.07 |
| 24 | 4.18 | 4.84 |
| 25 | 2.7 | 3.56 |
| 26 | 5.26 | 6.12 |
| 27 | 2.7 | 4.81 |
| 28 | 4.06 | 3.36 |
| 29 | 5.6 | 6.05 |
| 30 | 3.95 | 4.72 |