#include <main.h>						//main header file

/* pin definitions for the LCD */

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

unsigned int a,b,c,d=0;						//initialising global variables

/* function prototypes */

void swrot(void);						//reads user input from the rotary switch

unsigned int adc_reading(unsigned int channel);			//reads analog input

void rotateCW(void);						//rotates motor clockwise

void rotateCCW(void);						//rotates motor counter clockwise

void stallMotor(void);						//stops the motor

void display_ADC_value(void);					//shows the ADC value on LCD

void knightrider(void);						//LED pattern display

void main(){

setup_adc_ports(sAN0);					//sets up the ADC pin AN0 to be analog

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);			//configures the ADC clock speed and ADC sample time

lcd_init();							//initialises LCD display

while(TRUE){							//main program loop

swrot();							//calls switch rotation control function

}

}

unsigned int adc_reading(unsigned int channel){

set_adc_channel(channel);					//specifies the channel to use for the next read_adc() call

delay_us(500);						//500ms delay

return read_adc();						//reads the digital value from the analog to digital converter

}

void swrot(void){

if(input(pin_a2)){						//when SW1-A is selected, the value of RC1 will drive motor A

if((adc_reading(0)>76) && (adc_reading(0)<174)){ 		//if the value of RC1 is between 76 and 174

stallMotor();						//stops motor

display_ADC_value();					//displays the conversion value of RV1 on it's own LCD line

lcd_gotoxy(7,2);					//positions the cursor to the centre of line 2 for direction message

printf(lcd_putc, "Stalled");				//displays rotation direction

delay_ms(300);						//300ms delay for display visibilty

printf(lcd_putc, "\f");				//clears LCD display

}

if(adc_reading(0)<75){ 					//if the value of RC1 is below 75

rotateCCW();

display_ADC_value();					//displays the conversion value of RV1 on it's own LCD line

lcd_gotoxy(3,2);					//positions the cursor to the centre of line 2 for direction message

printf(lcd_putc, "Counter Clockwise");			//displays rotation direction

delay_ms(300);						//300ms delay for display visibility

printf(lcd_putc, "\f");				//clears LCD display

}

if(adc_reading(0)>175){ 					//if the value of RC1 is aboive 175

rotateCW();						//rotates the motor clockwise

display_ADC_value();					//displays the conversion value of RV1 on it's own LCD line

lcd_gotoxy(7,2);					//positions the cursor to the centre of line 2 for direction message

printf(lcd_putc, "Clockwise");				//displays rotation direction

delay_ms(300);						//300ms delay for display visibility

printf(lcd_putc, "\f");				//clears LCD display

}

}

if(input(pin_a3)){						//checks if SW1-B is pressed

if((a==0) || (a!=adc_reading(0) + 1)){			//checks if ADC value has changed

printf("ADC Value = %3u ", adc_reading(0));		//prints ADC value on virtual terminal

a=(adc_reading(0) + 1);				//updates previous value tracker

delay_ms(300);						//300ms display delay

}

stallMotor();						//stops the motor

output_b(adc_reading(0));					//outputs the conversion value of the analogue signal from RV1 on the LEDs

display_ADC_value();					//displays the conversion value of RV1 on it's own LCD line

}

if(input(pin_a4)){						//checks if SW1-C is pressed

rotateCW();						//rotates motor clockwise

if(input(pin_a1)){					//checks button state

b=0;							//resets state tracker for button off

if(c==0){						//checks if status needs updating

printf("button on ");				//echoed onto the virtual terminal

c=1;						//updates status tracker

}

lcd_gotoxy(26,1);					//positions the cursor to the centre of line 3 for button message

printf(lcd_putc, "button on ");			//displays button status

delay_ms(300);						//300ms display delay

printf(lcd_putc, "\f");				//clears display

}

else{							//handles button off state

c=0;							//resets status tracker for button on

if(b==0){						//checks if status needs updating

printf("button off ");				//echoed onto the virtual terminal

b=1;						//updates status tracker

}

stallMotor();						//stops motor

lcd_gotoxy(26,1);					//positions the cursor to the centre of line 3 for button message

printf(lcd_putc, "button off");			//displays button status

delay_ms(300);						//300ms display delay

printf(lcd_putc, "\f");				//clears display

}

}

if(input(pin_a5)){						//checks if SW1-D is pressed

if(d==0){							//checks if status needs updating

printf("kitt mode ");					//echoed onto the virtual terminal

d=1;							//updates tracker, ensuring it isnt't repeatedly displayed on the virtual terminal

}

stallMotor();						//stops motor

lcd_gotoxy(26,2);						//positions the cursor to the centre of line 4 for the current mode message

printf(lcd_putc, "Kitt mode");				//displays current mode

knightrider();						//runs LED pattern sequence

printf(lcd_putc, "\f");					//clears display

}

}

void rotateCW(void){

output_high(pin_C0);						//sets direction pin high

output_low(PIN_C1);						//sets opposite direction pin low to rotate the motor clockwise

}

void rotateCCW(void){

output_low(pin_C0);						//sets the direction pin low

output_high(PIN_C1); 					//sets opposite direction pin high to rotate the motor counter clockwise

}

void stallMotor(void){

output_low(pin_C0);						//sets the direction pin low

output_low(PIN_C1); 						//sets opposite direction pin low to stop the motor

}

void display_ADC_value(void){

lcd_gotoxy(4,1);						//the x coordinate '4' ensures the output is as centred as possible; 'y' coordinate ensures it is displayed on the first line on the LCD

printf(lcd_putc, "ADC Value = %3u", adc_reading(0));		//conversion value of the potentiometer from port AN0

}

void knightrider(void){

unsigned int kr[14]=0x80,0x40,0x20,0x10,0x08,0x04,0x02,0x01,0x02,0x04,0x08,0x10,0x20,0x40,;		//LED pattern sequence in hex code

unsigned int q;						//loop counter

for(q=0;q<=13;q++){						//cycles through the LED pattern

output_b(kr[q]);        				//sends values to b ports

delay_ms(200);            				//200ms delay

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2.. Your code has a number of your functions and all work. Your statements indicate what each line does 75%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%

Total 75%