Specific Stream Submission Tom Schultze 1802180

5th Engine Workshop Session with John Kinsella

29th October 2019

Engine Workshop

Reassembly of Inline 4 CBR 600R Engine

Health and Safety

-ALWAYS before entering workshop wear appropriate safety equipment.

Equipment worn this session:

-Eye protection: safety goggles

-Protective clothing: dust coat provided by uni and steel toe caps

- Put in generator 1x14mm

- Put on generator cover 8x8mm

- Put on top end 10x12mm

- Put in coolant pump 2x10mm

  - Different sized bolts left one is the longer bolt

- To put in cam shafts put the first piston at top dead centre (TDC) and put in cams and cam timing chain like in fig1.

Figure 1: Picture of camshaft position with timing chain

- To put piston at TDC remove spark plug, put in screwdriver in spark plug hole and turn motor until screwdriver starts dipping again => piston is at TDC

- Put back rocker housing 16x10mm

Figure 2: Rocker housing

Watching out for inner and outer bolt lengths (Fig. 2)

- Put on clutch cover 14x10mm

- Put on top 4x10mm

Engine is fully rebuilt again! (Fig.3)

Figure 3: Fully rebuilt engine

Figure 5: Inner workings of the fork

As the session was not qu

Figure 4: Opening the CRF forks

ite finished John let us take apart the forks of the CRF 450 (Fig. 4 & 5)

End of session