Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

Blocks the user if they get the password wrong twice within 120 second time period.

What does the block-for 30 mean in the above command?

Blocks the user from typing in password for 30 seconds.

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

Router 1 has accepted the telnet connection because it has configured to allow remote access with the local username and password.

Use Tera Term, establish an SSH session to R1. Use the username admin and password adminpass12. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection? Yes

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

The router blocked new connections for 30 seconds after 2 failed log in attempts within 120 seconds

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources. Tom – it is only showing Normal-Mode

R1#

After the 30 seconds has expired, SSH to R1 again and login using the SSHadmin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed? The username was created with no privileges set.  R1> was displayed and was waiting for entering privileged EXEC mode

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain.

I was allowed three attempts of entering the privileged EXEC password and it did not disconnect from the SSH session, I was presented back to User EXEC (R1>) mode and able to try the privileged EXEC password again.

Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

- username SSHadmin secret 5 $1$mERr$uMzC7If3IbCZVifeGQ3rg/

- enable secret 5 $1$mERr$5wXi8fVAu3dNc0gNCjvIQ1

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret adminpass

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch?

I can SSH to the switch on Vlan1 IP address 192.168.1.11

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

SSH version 1.99 and 2.0 are supported from the CLI

How would you provide multiple users, each with their own username, access to a network device?

You would create individual usernames and passwords for each of the multiple users, this can be accomplished by being in privilege EXEC mode, Configuration Terminal and entering the following for example:

S1(config)# username jbloggs password bloggsp@55

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g AES, SHA, DES, RSA.

SHA

There is a family of secure hashing algorithm types which are widely used (depending on various hardware and software capabilities), these being SHA-1, SHA-2(SHA-224, SHA-256), SHA-3(SHA-384) and SHA-5(SHA-512, SHA-512/224, SHA-512/256). [1]

SHA-1 is a legacy algorithm and is adequately safe to use, but leading security advisors suggest the minimum hashing level should be set to SHA-256 on appliances where it available for use due to it being more secure.

SHA-3 and SHA-5 are being labelled as the next-generation of cryptographic algorithms.

SHA-0 was the first of type, but this was found to contain major vulnerabilities and was withdrawn. Secure Hash Algorithm is a means of securing data enclosed within a message. [2] These algorithms are used to covert the data into secure messages. The output of the hash is called a message direct.

The message digests range in length from 160 to 512 bits depending on which algorithm type is used. Secure hash algorithms are commonly used in conjunction with other cryptographic algorithms.

This type of algorithm ensures the message’s integrity, any adjustment to a message will output a different message digest, this results in a verification failure. [3]

The algorithms strength in securing the data differ considerably, in other words the bigger the algorithm used, the better. SHA1, SHA-224 and SHA-256, each message block has 512 bits which are represented as a sequence of sixteen 32-bit words.  For SHA-384, SHA-512, SHA 512/224 and SHA-512/256 each message block has 1024 bits, which are represented as a sequence of sixteen 64-bit words. [4]

RSA

Rivest, Shamir, Adleman, are the names of the engineers who discovered this algorithm that is used for digital signatures and key establishment. Its primary use is for asymmetric algorithm which is used for authentication. [5]  RSA can also be used as a Public-Key Cryptography Standards (PKCS) series and is a reversible algorithm used for encryption.  This form of algorithm does not use the same keys for encrypting or decrypting the data, it uses two different keys that mathematically work together as a pair, commonly known as the public and private keys or RSA key pair.  RSA is quite a resource hungry algorithm, it requires high CPU utilization in order to create the keys, encrypting and decrypting the data.  However, where multi-prime RSA specification can be supported, it will reduce the computational cost for the decryption. With the use of single processor hardware, but better on multiprocessor hardware, the performance of using such techniques can be improved greatly.

But still his type of encryption is best used for tasks that are infrequent because of the resources required, it would be impractical to use for constantly encrypting day-to-day data. [6]

AES and DES

Advanced Encryption Service (AES) and Digital Encryption Standard (DES) are both referred to as block symmetric key cipher or symmetric encryption algorithms. They use the same key to encrypt and decrypt data and are not as resource hungry as asymmetric algorithms. This type of encryption may take a 64-bit block of plain text (unencrypted) and produce a 64-bit block of cipher text (encrypted). [7]  If there is not enough data to encrypt then this algorithm can use padding to fill up the block space, this is not ideal since there will be a small overhead of waste because the padding will be processed along with the real data. [8] An example of the type of service for this would be two hosts connected via a VPN, they both require the same keys in order to encrypt and decrypt the data that is secured using symmetric encryption algorithm. These forms of algorithms are much faster to use and take less CPU resources, the VPN example above is a prime use of encrypting the data. [9]  To ensure the data is secure and difficult to intercept/decrypt, it is important to make sure that the key is difficult for someone who does not have a key to not understand the data and try to create a key should they attempt to decrypt, this can be achieved by having a longer key. Ideally the minimum key length should be at least 128 bits for symmetric encryption algorithms to be secure. [10] DES was found to be a weak cryptographic algorithm and NIST withdrew it as a standard in 2005, it was replaced with AES. [11]

References

| [1] | "NIST," [Online]. Available: https://csrc.nist.gov/glossary/term/secure-hash-algorithm. [Accessed 11 December 2019]. |
| --- | --- |
| [2] | "NIST," [Online]. Available: https://csrc.nist.gov/glossary/term/secure-hash-algorithm. [Accessed 11 December 2019]. |
| [3] | P. Jones, "Tools," September 2001. [Online]. Available: https://tools.ietf.org/html/rfc3174. [Accessed 11 December 2019]. |
| [4] | J. M. S. D. C. Omar Santos, "CCNA Cyber Ops SECFND," pp. CCNA Cyber Ops SECFND 210-250. |
| [5] | J. M. S. D. C. Omar Santos, "CCNA Cyber Ops SECFND," pp. 210-250. |
| [6] | "NIST," [Online]. Available: https://csrc.nist.gov/glossary/term/RSA. [Accessed 11 December 2019]. |
| [7] | J. M. S. D. C. Omar Santos, "CCNA Cyber Ops SECFND," pp. 210-250. |
| [8] | P. Jones, "Tools," September 2001. [Online]. Available: https://tools.ietf.org/html/rfc3174. [Accessed 11 December 2019]. |
| [9] | F. Maino, "Tools," June 2004. [Online]. Available: https://tools.ietf.org/html/rfc3826. [Accessed 11 December 2019]. |
| [10] | J. M. S. D. C. Omar Santos, "CCNA Cyber Ops SECFND," pp. 210-250. |
| [11] | "NIST," [Online]. Available: https://csrc.nist.gov/News/1993/Revision-of-FIPS-46-1-DES-as-FIPS-46-2. [Accessed 11 December 2019]. |