| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Floor screeding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0003 |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | Employees |  | Y | Contractor |  |  |  | Public |  |  |  | Y | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Slips, trips and falls when walking through construction site |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |
| 2 | Musculoskeletal injuries caused by incorrect lifting and handling of screed and associated work equipment |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |
| 3 | Use of screed mixer and pump – risk of entrapment with moving part |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  | 6 |  |  |  |
| 4 | Falls over screed pipes |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |
| 5 | Burns from exposure to screed |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |
| 6 | Eye injury |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  | 6 |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Slips, trips and falls – ensure provision of safe access/egress to and from and within the workplace |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Musculoskeletal injuries – where the weight of materials/work equipment cannot be reduced, lifting is necessary by trained persons. Consider using mechanical aids where possible e.g trolley |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Use of screed mixer & pump – equipment to be used by trained persons, and in accordance with the manufacturer’s instruction manual. Where fitted, guards must be maintained in good order and must be in place during the operation of the equipment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Falls over screed pipes – pipes to be routed where possible in locations that minimise the risk of trip hazards. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Burns – refer to COSHH assessment (skin to be covered. Exposure to be minimised where possible/ RPE to be worn when in dust form) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Eye Injury – eye protection to be worn where is a foreseeable risk to injury, in particular when the screed is in dust form or where there is a risk of splashing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Slips, trips and falls when walking through construction site |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| 2 | Musculoskeletal injuries caused by incorrect lifting and handling of screed and associated work equipment |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  | 2 |  |  |  |
| 3 | Use of screed mixer and pump – risk of entrapment with moving part |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| 4 | Falls over screed pipes |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  | 2 |  |  |  |
| 5 | Burns from exposure to screed |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  | 2 |  |  |  |
| 6 | Eye injury |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| Name: Nathan Price Position: Quantity Surveyor |  |  |  |  |  |  | Signed: N PRICE |  |  |  |  |  |  |  |  |  | Date of Completion: 5/5/2020 |  |  |  |  |  |  |  |  |
| Reviews |  |  |  | Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Date: |  |  |  | Likelihood |  |  |  | Severity |  |  | Severity of Injury |  | 5 | 5Y |  |  |  | 10R |  | 15 |  |  | 20 |  | 25 |
| Reviewed by: |  |  |  | 1 very unlikely 2 unlikely 3 likely 4 very likely 5 certainty |  |  |  | 1 nuisance 2 minor 3 medical treatment 4 major 5 fatal |  |  |  |  | 4 | 4 |  |  |  | 8 |  | 12 |  |  | 16 |  | 20 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 3 |  |  |  | 6G |  | 9 |  |  | 12 |  | 15 |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 |  |  |  | 4 |  | 6Y |  |  | 8 |  | 10Y |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5G |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood of Injury |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  | Medium Risk |  |  |  |  | High Risk |  |