Electromagnetic Radiation

Robert Webb

2308625

Embedded Electronic Degree Apprenticeship

L5 – Electromagnetic Radiation

ACDC5009

Abstract

This report presents a comprehensive simulation study of two types of rectangular waveguides and a half-wave dipole antenna, conducted using ANSYS HFSS and CST Studio Suite. The waveguides were designed based on specific dimensional parameters derived from the author’s student number, with one modelled using vacuum as the dielectric material and the other using Teflon. Electromagnetic performance, including field distributions and cut-off frequencies for TE modes, was analysed and compared against theoretical predictions. Simulation results demonstrated good agreement, with TE10 mode confirmed as the dominant propagation mode and minimal variation between simulated and theoretical cut-off frequencies.

Additionally, a half-wave dipole antenna was designed to resonate at approximately 1.306 GHz. The antenna exhibited an impedance close to 73 ohms, a broadside radiation pattern, and a return loss below -20 dB at resonance, validating its suitability for practical communication applications. Overall, the simulations effectively confirmed theoretical expectations and highlighted the influence of material properties on waveguide behaviour, as well as the importance of precise dimensional design in antenna performance.

Table of Contents

Table of Figures	4

Waveguides and Halfwave Dipole Antenna Simulations	5

1.1.	Introduction	5

1.1.1.	Waveguide Theory:	5

1.1.2.	Half wave Dipole Theory:	5

1.2. Methodology	5

1.2.1. Waveguides Methodology	5

1.2.2. Half-Wave Dipole Antenna Methodology:	6

1.3. Results	7

1.3.1 PEC Waveguide Port Field Displays	7

1.3.2. PEC Waveguide E & H Magnitude and Vector Plots	8

1.3.3. Teflon Waveguide Port Field Displays	12

1.3.4. PEC Waveguide E & H Magnitude and Vector Plots	13

1.4. Discussion	20

1.4.1. Waveguides Results:	20

1.4.2. Halfwave Dipole Results:	20

1.5. Conclusion	21

2.0. Drone Radar Literature Review	22

2.1. Introduction	22

2.2. Literature Review	22

2.2.1 Micro-Doppler Effect:	22

2.2.2 Modulation of the RCS:	23

2.3. Discussion	24

2.3.1 Future Improvements:	25

2.4. Conclusion	25

2.5. All References	26

# Table of Figures

Figure 1: PEC Waveguide	6

Figure 2: Teflon Waveguide	6

Figure 3: Halfwave Dipole Antenna	7

Figure 4: Port Field Display 1 Mode 1 – Vacuum	7

Figure 5: Port Field Display 1 Mode 2 - Vacuum	7

Figure 6: Port Field Display 1 Mode 3 - Vacuum	7

Figure 7: Port Field Display 2 Mode 1 - Vacuum	8

Figure 8: Port Field Display 2 Mode 2 - Vacuum	8

Figure 9: Port Field Display 2 Mode 3 – Vacuum	8

Figure 10: Mode 1 E Field Magnitude Plot	8

Figure 11: Mode 1 E Field Vector Plot	9

Figure 12: Mode 1 H Field Magnitude Plot	9

Figure 13: Mode 1 H Field Vector Plot	9

Figure 14: Mode 2 E Field Magnitude Plot	10

Figure 15: Mode 2 E Field Vector Plot	10

Figure 16: Mode 2 H Field Magnitude Plot	10

Figure 17: Mode 2 H Field Vector Plot	11

Figure 18: Mode 3 E Field Magnitude Plot	11

Figure 19: Mode 3 E Field Vector Plot	11

Figure 20: Mode 3 H Field Magnitude Plot	12

Figure 21: Mode 3 H Field Vector Plot	12

Figure 22: Port Field Display 1 Mode 1 – Teflon	12

Figure 23: Port Field Display 1 Mode 2 – Teflon	12

Figure 24: Port Field Display 1 Mode 3 – Teflon	12

Figure 25: Port Field Display 2 Mode 1 – Teflon	13

Figure 26: Port Field Display 2 Mode 2 – Teflon	13

Figure 27: Port Field Display 2 Mode 3 – Teflon	13

Figure 28: Mode 1 E Field Magnitude Plot Teflon	13

Figure 29: Mode 1 E Field Vector Plot Teflon	13

Figure 30: Mode 1 H Field Magnitude Plot Teflon	14

Figure 31: Mode 1 H Field Vector Plot Teflon	14

Figure 32: Mode 2 E Field Magnitude Plot Teflon	14

Figure 33: Mode 2 E Field Vector Plot Teflon	15

Figure 34: Mode 2 H Field Magnitude Plot Teflon	15

Figure 35: Mode 2 H Field Vector Plot Teflon	15

Figure 36: Mode 3 E Field Magnitude Plot Teflon	16

Figure 37: Mode 3 E Field Vector Plot Teflon	16

Figure 38: Mode 3 H Field Magnitude Plot Teflon	16

Figure 39: Mode 3 H Field Vector Plot Teflon	17

Figure 40: Vacuum Waveguide Gamma Plot	17

Figure 41: Teflon Waveguide Gamma Plot	18

Figure 42: Halfwave Dipole Antenna Radiation Pattern	19

Figure 43: S1.1 against Frequency Plot for Dipole Antenna	19

Figure 44: Dipole Antenna Phi Directivity	19

Figure 45: Dipole Antenna Theta Directivity	20

Figure 46: Micro-Doppler Signature Dataset of Small Unmanned Aerial Vehicle (SUAV) [12]	23

Figure 47: Doppler effect of a bird flapping its wings [13]	23

Figure 48: ISAR images [15]	24

# Waveguides and Halfwave Dipole Antenna Simulations

## Introduction

Within this report, the author was tasked to create, simulate and validate data on two types of waveguides and a half-wave dipole Antenna. These simulations were done in Ansys and CST, dedicated simulation software for testing antennas and waveguides, to name but a few. The author had to design the waveguide around their student number to decide the width of the waveguide. This was then done again for the second waveguide, with the only difference being the material change from Vacuum to Teflon.

### Waveguide Theory:

Waveguides are used to channel EM (Electromagnetic) signals within a structure, they are used to prevent losses and spreading of waves during transmission from one end to the other. For a rectangular waveguide, as used in this report, they are a hollow metallic conducting tube with a rectangular cross section, they are usually constructed with a length a > b, where b is the breadth of the waveguide. Waves are transmitted longitudinally through the waveguide, the waves are then reflected off the conducting walls, and depending on the total reflection inside, which would result in either the magnetic field or the electric field in the direction of wave propagation [1].

### Half wave Dipole Theory:

This design makes the half-wave dipole most efficient at its resonant frequency, where the total length of the antenna is equal to half the wavelength of the signal. [2]

When driven with an alternating current (AC) at the resonant frequency, the antenna produces oscillating electric and magnetic fields along its length. These oscillations radiate outward into an area as electromagnetic waves. The resulting radiation pattern is toroidal or doughnut also known as omnidirectional, with the strongest signal emitted perpendicular to the axis of the antenna and minimal radiation along the antenna’s ends. At resonance, a centre-fed half-wave dipole typically presents an input impedance of approximately 73 ohms (Ω) [3]. This value allows for relatively straightforward matching with common transmission lines, such as coaxial cables, minimising signal reflection and power loss.

## 1.2. Methodology

In tasks 1 and 2, the author had to design and create a Waveguide from 2 different materials; these materials were a vacuum and Teflon, they were both designed and developed using Ansys software.

### 1.2.1. Waveguides Methodology

The rectangular waveguide was modelled and simulated using ANSYS software to simulate its electromagnetic performance. The waveguide was designed using the author's student number with a width (a) of 23 mm, a height (b) of 4 mm, and a total length (L) of 50 mm. The interior region was assigned as air, and the walls were treated as perfect electric conductors (PEC) to represent an idealised metallic structure. This was repeated for the other waveguide, but the only difference is the material of Teflon.

Wave ports were applied to both ends of the waveguide to excite and receive the dominant TE10 mode. Each port was defined to match the cross-sectional dimensions of the waveguide, with appropriate integration lines placed to ensure accurate mode excitation. A frequency sweep around the expected operational band was implemented to observe the behaviour of the waveguide across the spectrum.

Simulation outputs included S-parameters, specifically S11​ and S21​, to characterise the reflection and transmission properties. Additionally, electric field distribution plots were analysed to verify proper excitation of the TE10​ mode. The observed cut-off frequency and field patterns were compared against theoretical numbers to validate the accuracy of the simulation model.

Figure 1: PEC Waveguide

Figure 2: Teflon Waveguide

### 1.2.2. Half-Wave Dipole Antenna Methodology:

When constructing the waveguide, the author had to use their student number to aid the sizing/measurements, with the Length being 230mm, due to the author's student number, the length would be 115mm on each side with an excitation in the middle (1mm gap) this can be known as a 50Ω lumped port which is used to allow the dipole to radiate as current and voltage is distributed up and down the antenna also known as a feeding gap. The length of the conductive elements is half of the maximum possible wavelength, λ/2 [4]. The dipole was placed in a rectangular radiating surface environment.

Figure 3: Halfwave Dipole Antenna

## 1.3. Results

This section shows all of the results for the waveguides and the Dipole Antenna

### 1.3.1 PEC Waveguide Port Field Displays

Figure 4: Port Field Display 1 Mode 1 – Vacuum

Figure 5: Port Field Display 1 Mode 2 - Vacuum

Figure 6: Port Field Display 1 Mode 3 - Vacuum

Figure 7: Port Field Display 2 Mode 1 - Vacuum

Figure 8: Port Field Display 2 Mode 2 - Vacuum

Figure 9: Port Field Display 2 Mode 3 – Vacuum

### 1.3.2. PEC Waveguide E & H Magnitude and Vector Plots

Figure 10: Mode 1 E Field Magnitude Plot

Figure 11: Mode 1 E Field Vector Plot

Figure 12: Mode 1 H Field Magnitude Plot

Figure 13: Mode 1 H Field Vector Plot

Figure 14: Mode 2 E Field Magnitude Plot

Figure 15: Mode 2 E Field Vector Plot

Figure 16: Mode 2 H Field Magnitude Plot

Figure 17: Mode 2 H Field Vector Plot

Figure 18: Mode 3 E Field Magnitude Plot

Figure 19: Mode 3 E Field Vector Plot

Figure 20: Mode 3 H Field Magnitude Plot

Figure 21: Mode 3 H Field Vector Plot

### 1.3.3. Teflon Waveguide Port Field Displays

Figure 22: Port Field Display 1 Mode 1 – Teflon

Figure 23: Port Field Display 1 Mode 2 – Teflon

Figure 24: Port Field Display 1 Mode 3 – Teflon

Figure 25: Port Field Display 2 Mode 1 – Teflon

Figure 26: Port Field Display 2 Mode 2 – Teflon

Figure 27: Port Field Display 2 Mode 3 – Teflon

### 1.3.4. PEC Waveguide E & H Magnitude and Vector Plots

Figure 28: Mode 1 E Field Magnitude Plot Teflon

Figure 29: Mode 1 E Field Vector Plot Teflon

Figure 30: Mode 1 H Field Magnitude Plot Teflon

Figure 31: Mode 1 H Field Vector Plot Teflon

Figure 32: Mode 2 E Field Magnitude Plot Teflon

Figure 33: Mode 2 E Field Vector Plot Teflon

Figure 34: Mode 2 H Field Magnitude Plot Teflon

Figure 35: Mode 2 H Field Vector Plot Teflon

Figure 36: Mode 3 E Field Magnitude Plot Teflon

Figure 37: Mode 3 E Field Vector Plot Teflon

Figure 38: Mode 3 H Field Magnitude Plot Teflon

Figure 39: Mode 3 H Field Vector Plot Teflon

After simulating the 2 waveguides, PEC and Teflon, the author then plotted two graphs which can be used to help visualise the cut-off frequencies for the modes. The graphs in Figures 40 and 41 show the cut-off frequencies for each mode when simulating the waveguide. These then show that Mode 1 (TE10) is the dominant mode due to it having the smallest cut-off frequency, allowing it to propagate with the least loss. This is the same across both materials; the difference between the two cut-off frequencies is 1.98 GHz.

Figure 40: Vacuum Waveguide Gamma Plot

c=1εμ=3 x 108m/s

TE 1:0=fc=c210.0232+0π0.0042= 6.52x10^9 Hz = 6.5 GHz

TE 2:0=fc=c220.0232+0π0.0042= 1.30x10^10 = 13 GHz

TE 3:0=fc=c230.0232+0π0.0042= 1.97 x10^9 Hz = 19.7 GHz

Figure 41: Teflon Waveguide Gamma Plot

c=1εμ = (3x108)1x2.1=2.07 x 108

TE 1:0=fc=c210.0232+0π0.0042 = 4.5 GHz

TE 2:0=fc=c220.0232+0π0.0042 = 9 GHz

TE 3:0= fc=c230.0232+0π0.0042 = 13.5 GHz

After completing the calculations for the cut-off frequencies, there was a bigger difference between the two different materials the higher the frequency became during the simulation. This is due to the materials changing the magnetic field, enabling the transmission to be different through the waveguide.

Halfwave Dipole Antenna

Length of the halfwave dipole =  230mm2 = 115mm

f=λc

Operating Frequency, f=230mm3x108m/s=1.306 GHz

Figure 42: Halfwave Dipole Antenna Radiation Pattern

Figure 43: S1.1 against Frequency Plot for Dipole Antenna

The red plot shown in Fig. 43 is the original from the simulation, whilst this is correct, it is just the wavelength divided by 2, the green plot is the scaled frequency /frequency shift from having to increase the dipole length slightly to push the operating frequency up to a more precise frequency, the increase in the length of a dipole increases the frequency in which is outputted. Figures 44 and 45 show the directivity on two different planes, Phi and Theta, phi being a ‘side view’, which shows the radiation outwards and the nulls at the top and bottom where it has a weaker field of radiation. Whereas Theta directivity is a ‘top-down’ view which shows the strength of fields radiating out and direction, in this instance it displays that the dipole radiated at 360 degrees.

Figure 44: Dipole Antenna Phi Directivity

Figure 45: Dipole Antenna Theta Directivity

## 1.4. Discussion

### 1.4.1. Waveguides Results:

The simulation results showed differences between the PEC vacuum and Teflon rectangular waveguides. The PEC waveguide showed faster wave propagation due to the vacuum’s permittivity (εᵣ = 1), which led to a higher cut-off frequency and reduced signal delay. However, the Teflon waveguide, with εᵣ ≈ 2.1, showed a lower cut-off frequency, which was confirmed when the author used theoretical calculations, and more confined electromagnetic fields, which resulted in slower velocity. While both waveguides effectively guided the dominant TE₁₀ mode, the Teflon dielectric increased the effective electrical length, which may be an advantage if used in compact designs. However, increased dielectric losses in Teflon could degrade performance in high-power or long-distance applications. Overall, the choice of dielectric material significantly impacts waveguide behaviour.

### 1.4.2. Halfwave Dipole Results:

The half-wave dipole antenna that had a total length of 230 mm (2 x 115mm) was analysed both theoretically and through simulation, using a theoretical formula the operating frequency of around 1.306 GHz was calculated after adjusting the length due to having a lesser frequency at 115mm because of fringing effect [5] this was counteracted by increasing the length slightly to increase its inductance. The simulation resonant frequency for the half-wave dipole of this length is approximately 1.304 GHz, confirming that the antenna is appropriately dimensioned for resonance at the intended frequency.

Theoretical research shows an input impedance of around 73 ohms [6], and a radiation pattern with a gain of approximately 2.15 dBi. These characteristics were confirmed through simulation using CST, where the half-wave dipole antenna demonstrated a resonant frequency closely matching 1.306 GHz. Simulated results showed an input impedance of 70–75 ohms, return loss (S₁₁) below -20 dB at resonance, and a gain of about 2.1–2.3 dBi. The radiation pattern maintained the expected doughnut shape the author expected.

Overall, the antenna exhibits high efficiency and excellent matching performance at the target frequency, making it well-suited for use in communication systems operating around 1.3 GHz.

## 1.5. Conclusion

Overall, the simulations confirmed that material properties significantly influence waveguide performance, with the PEC (vacuum) waveguide exhibiting higher cut-off frequencies and faster wave propagation compared to the Teflon-filled counterpart. Both supported the dominant TE₁₀ mode effectively. The half-wave dipole antenna, designed with a total length of 230 mm, achieved resonance near the operating frequency of 1.306 GHz, showing good impedance matching and a typical 360-degree radiation pattern. Overall, the results align closely with theoretical expectations, demonstrating accurate modelling and reliable performance of both waveguides and the antenna.

# 2.0. Drone Radar Literature Review

## 2.1. Introduction

Within this literature review of the report, you will read about the reasons why radars, known for detecting and tracking aircraft within the airspace, fail to detect and track drones as well as some other UAS platforms. Drones are a fairly new and emerging technology which is advancing at an extremely fast pace, with market leaders such as DJI, Autel and Parrot to name but a few. Each specialises in different aspects of life in which a drone can be used. In this literature review, the author will evaluate the technological advancements to help detect drones and discuss the societal and environmental impacts of the systems.

Drones are an ever-increasing security threat not to just the UK but to the world in general, with incidents including things like Gatwick Airport back in 2018 causing significant disruption and grounding hundreds of flights and stopping about 110,000 passengers from traveling overnight [6], this caused significant issues and shone the light onto drones as being an upcoming security threat for years to come. During 2023 alone, there were over 6000 incidents involving drone use reported to the police, and a lot more which went unreported. These incidents pose a significant risk to the public and all modes of transport, predominantly air travel.

The use of drone radar from a societal point of view would be to allow the tracking and tracing of platforms which could cause disruption to things like UK airport by completely disregarding the no-fly zones, this is essential for things like airports and the consequences for an aircraft being in a collision with a drone could end up being catastrophic. This could also be used to help counter the use of drones to drop contraband into prisons for inmates. Due to them being small and quiet in a loud prison environment, they would be hard to detect by prison officers. 31% of the reports stated above were related to drones being suspected of delivering items within prisons. [7] [8]

## 2.2. Literature Review

Radar has been around since World War II, with the British developing the systems and then providing the US Armed Forces allowing for joined support within the Battle. With the word ‘Radar’ meaning Radio Detection and Ranging [9], the use of radar has become critical within the 20th century, with aircraft and ships being the leading users of radar systems.

New advancements in drone radar enable the use of radio energy to detect smaller objects such as drones, i.e. DJI platforms. These new advancements utilise the same idea as standard aircraft radar, which sends out signal waves that are then reflected from the platform. The radar then ‘listens’ for the echo, which is the waves being reflected from the platform back to the radar. Advancements in this field have seen a lot of drone detection kits using the Micro-Doppler effect technology [10], this effect is utilised in most drone radars as it allows the detection of drones rather than birds, etc to be traced and tracked.

### 2.2.1 Micro-Doppler Effect:

The ‘Micro-Doppler’ is generated due to the periodic movement of a main structural component of an item, device or object. This movement creates ‘micro-motion’ which created side bands about the bulk Doppler Frequency [11]. The phase of the signal that is returned to the radar from such an object will change depending on what that object could be. Drones have propellers which spin to keep them in flight whereas bird use their wings and flap; radar struggles to know the difference between the two types of movement, due to them being about the same size (depending on what platform is being flow, DJI Mini-series etc), other methods of drone detection would include the modulation of the RCS (Radar Cross-Section) but the Micro Doppler radar overcomes most of these issues by primary focusing on the signatures being emitted during its flight.

Figure 46: Micro-Doppler Signature Dataset of Small Unmanned Aerial Vehicle (SUAV) [12]

Figure 47: Doppler effect of a bird flapping its wings [13]

Figure 23 shows the Micro-Doppler signatures on a range of different UAS systems, this is what the radar uses to distinguish the differences between drones and birds, figure 24 shows the doppler effect of a bird flapping its wings, most drones are quadcopters so using image ‘D’ in fig23 you can see that there is a major difference in the effect being given off by the rotors compared to the wings of the bird. The radar looks into the modulation of the object rather than the speed as it is more straightforward to distinguish between the two instead of by speed.

### 2.2.2 Modulation of the RCS:

Another recent advancement in Drone radar technology has been the use of identifying the modulation of the RCS (Radar Cross Section). The cross section of a drone would be extremely small due to the small size of a drone and the materials used when manufacturing it. [14] When in flight, the RCS will be the same but will be hard to locate and track due to the constant and sudden movement in which drones can do, radar which track the RCS of the drone are new to the market and have proven themselves well and can differentiate between platforms and birds with significant accuracy which is a big selling point in the current market.

Figure 48: ISAR images [15]

The above images show what the modulation of an RCS looks like, visually you can just about work out it’s a platform, but new radar with this technology can take this data and analyse it in seconds and decide whether it’s a platform or a drone, this is market changing technology which will be used increasingly.

## 2.3. Discussion

The application of the radars to drones is the fact that they can locate and enable the tracking of drones right from small to large sizes without getting confused between birds and the platforms. These applications have allowed more incidents to be prevented or acted upon to reduce crime rates or disruption. The use of Micro-Doppler radars is a great technological advancement which should continue to be utilised and deployed in drone radar as it seems to be a very good system and makes good detection decisions between birds and drone platforms. With this, the modulation of the RCS would still be a great technology to continue to develop and improve, especially with different types of platforms being designed and manufactured, which could be even smaller in the future.

The application of both the micro-doppler and RCS modulation radars has seen great advancements in drone radar technology. Drones can now track more and smaller platforms, which poses good societal impacts. Due to the radar being over the air, effects on the environment are minimal, so this also poses a good look in the environment area due to negligible impacts being produced. Overall, the applications of these radars are fantastic due to the fact that it is able to track at any time of the day, no matter the weather and can detect much further than VLOS (Visual Line of Sight), as well as in most cases they can detect multiple drones simultaneously.

### 2.3.1 Future Improvements:

AI-Assisted Drone Tracking Radar:

Improvements for the future could involve utilising AI (Artificial Intelligence) in a radar system, this could be trained to recognise all different types of signatures drones give off and be used to search and track. Once it matches the signatures, it could then track the location of the drone until it lands or gets turned off. With it being trained AI, it could be able to recognise the exact platform and provide all of its details to the user of the radar all of the details such as location, model, and ground speed, to name but a few. this would have good impacts due to it being able to potentially help the Police with searching and investigations when drone incidents occur. This would have no bad effect on the environment due to no waste being created, etc, only making the airspace safer for members of the public.

## 2.4. Conclusion

Overall, the current drone radars are doing an adequate job of identifying drones as well as determining the difference between a drone and birds. The societal impacts of the drone radars also leans very positive as it helps to reduce illegal drone use as the platforms can be tracked right from the start of their flight as well as keeps air travel safer. Using other methods like the modulation of the RCS is also a good style which is effective and can scan vast amounts of airspace, processing the differences of drones to birds and ground clutter amongst it. As more drones are purchased by consumers and no doubt more incidents occur, these forms of radars will be great to utilise and improve going forward to ensure they can be tracked and the new advancements have been market-changing with new technology being unlocked to help detect platforms.

# 2.5. All References

| [1] | Cadence System Analysis, “Basic Rectangular Waveguide Theory”. |
| --- | --- |
| [2] | Tutorialspoint, “Antenna Theory - Half-Wave Dipole,” tutorialspoint, [Online]. Available: https://www.tutorialspoint.com/antenna_theory/antenna_theory_half_wave_dipole.htm. [Accessed 20 Apr 2025]. |
| [3] | Wikipedia, “Dipole antenna,” Wikipedia, [Online]. Available: https://en.wikipedia.org/wiki/Dipole_antenna#:~:text=Ideally%2C%20a%20half%2Dwave%20dipole,65%E2%80%9370%20%CE%A9%20input%20impedance.. [Accessed 20 Apr 2025]. |
| [4] | JEM Engineering, “Dipole Antennas: The Simplest, Most Common Antenna,” JEM Engineering, Aug 2021. [Online]. Available: https://jemengineering.com/blog-dipoles/#:~:text=A%20half%2Dwave%20dipole%2C%20also,at%20the%20frequency%20of%20operation.. [Accessed 16 Apr 2025]. |
| [5] | P. Kathryn L. Smith, “The Half-Wave Dipole: Design,” 2020. [Online]. Available: chrome-extension://efaidnbmnnnibpcajpcglclefindmkaj/https://innovationspace.ansys.com/courses/wp-content/uploads/sites/5/2022/07/TheHalfWaveDipole-Design.pdf. [Accessed 21 Apr 2025]. |
| [6] | “5.2.1.1 Dipole Antenna Input Impedance Canonical Validation Problem”. |
| [7] | BBC, “Gatwick Airport: Drones ground flights,” BBC News, 20 Dec 2018. [Online]. Available: https://www.bbc.co.uk/news/uk-england-sussex-46623754. [Accessed 02 Apr 2025]. |
| [8] | AARTOS, “Drone Incidents,” AARTOS, [Online]. Available: https://drone-detection-system.com/drone-incidents/. [Accessed 04 Apr 2025]. |
| [9] | ProtectUK, “Threat from Drones in the UK,” ProtectUK, 08 Aug 2024. [Online]. Available: https://www.protectuk.police.uk/threat-risk/threat-analysis/threat-drones-uk#:~:text=The%20vast%20majority%20of%20users,approximately%2011%25)%20constituted%20offences.. [Accessed 04 Apr 2025]. |
| [10] | “History of Radar,” Wikipedia, [Online]. Available: https://en.wikipedia.org/wiki/History_of_radar. [Accessed 04 March 2025]. |
| [11] | “What is Counter-UAS?,” Robin Radar System, 7 March 2024. [Online]. Available: https://www.robinradar.com/blog/what-is-counter-uas#:~:text=Drone%20detection%20radar%20identifies%2C%20classifies,moving%20components%20like%20rotor%20blades.. [Accessed 28 March 2025]. |
| [12] | robinradar, “Why Micro-Doppler Radar is Essential for Drone Detection,” robinradar, 06 Dec 2022. [Online]. Available: https://www.robinradar.com/blog/micro-doppler-radar-drone-detection. [Accessed 02 Apr 2025]. |
| [13] | M. C. A. A. B. R. S. V. D. Harish Chandra Kumawat, DIAT-µSAT: micro-Doppler Signature Dataset of Small Unmanned Aerial Vehicle (SUAV), 2022. |
| [14] | W. &. W. Y. &. L. Y. &. L. Y. &. S. W. Jiang, “Radar Target Characterization and Deep Learning in Radar Automatic Target Recognition: A Review. Remote Sensing,” 2024. |
| [15] | F. F. D. A. Jarez S. Patel, “Review of radar classification and RCS characterisation techniques for small UAVs or drones,” IET, 2018. |
| [16] | C. J. Li and H. Ling, “An Investigation on the Radar Signatures of Small Consumer Drones,” 2016. |