Evaluating the Effectiveness of Chosen Maintenance Strategies for Dedicated Equipment

Course: BEng Mechanical and Manufacturing Engineering

Module: Plant and Asset Engineering, (ACMA 6007)

Aneurin Bamsey (2014505)

Andrew Thorn & Dr Kelvin Lake

Monday, 3rd of April 2023

Contents

1.0 Introduction	3

1.1 Report Outline	3

1.2 Business Background	3

1.3 Chosen Equipment	3

2.0 Critical review of current maintenance practice	4

2.1 Depaneling Machine	4

2.2 Current Maintenance Practice	5

2.2.1 What is Maintenance and its benefits?	5

2.3 Types of maintenance	6

Reactive	6

Proactive	6

Aggressive	6

2.4 Review	7

2.5 Why has this approach been chosen.	8

3.0 A full evaluation of how the current approaches to maintenance could be improved.	9

3.1 How the current maintenance strategy can be improved? Followed with suggested alternative approaches that could be adopted/implemented in future maintenance strategy.	9

4.0 Consideration of costs and resources required to implement proposed changes.	10

Costs are resources.	10

Change management implications.	10

Justification	10

5.0 Conclusions/ Recommendations.	11

6.0 References	12

7.0 Appendices	13

7.1 Current Depanelar schedules (January – December)	13

List of figures

Figure.1, Depaneler external image ..…………………………………….………………………. 4

Figure.2, Depaneler internal image ..…………………………………………………….……….. 4

Figure.3, Ruwac extraction unit ……………………………………………….……………………. 4

Figure.4, Panel entering Depanelar ……………………………………………….………………. 5

Figure.5, Z-Axis milling head ……………………………………………….…………………………. 5

Figure.6, Z-Axis handling assembly and gripper station …………………………….…… 5

Figure.7, PCBs in toting ……………………………………………………………….………………… 5

Figure.8, PCB dust in Ruwac bin ……………………………………………………………………. 8

Figure.9, Maintenance time tab on ASYS software ……………………………………….. 9

# 1.0 Introduction

## 1.1 Report Outline

This report will be delving into the current maintenance practice surrounding the ASYS Depaneling System Divisio 5000/5100 machine. This will involve investigating the current maintenance strategy. By Highlighting its pros and cons, its weaknesses can then be exploited and suggesting for improvements can be made to ensure the maintenance is carried out with greater repeatability and quality, while considering any implication changes as a result.

## 1.2 Business Background

Renishaw is a global, high precision metrology and healthcare technology group. The company has over 5000 employees covering 77 offices in 36 different countries. The company started in 1973 by the two founders Sir David McMurtry and John Deer and since then the company has become one of the world leaders in fields such as additive manufacturing and encode produces. (Renishaw, 2023)

## 1.3 Chosen Equipment

The ASYS Depaneling machine is a machine manufactured by ASYS Group, to deliver an optimal depaneling solution for all printed circuit boards (PCB). The manufacturers states, regardless of material, size, or contour. This depaneling machine can depanel boards from their panels with a precision of 10µm. (GROUP, 2023)

Fig. 2, Depaneler internal image

Fig. 1, Depaneler external image

Fig. 3, Ruwac extraction unit

# 2.0 Critical review of current maintenance practice

## 2.1 Depaneling Machine

The Depaneling machine is the process of cutting and removing individual PCB boards from their panels to be transported individually into a separation toting for the next process stage. The machine includes a transportation conveyor for both Panels and toting, a Z-Axis Milling head equipped with a fiducial camera, and a Z-Axis Handling assembly with 5 optional gripper types. The process starts by loading both panel and toting into the machine. They are then stopped through using pneumatic stoppers which correctly positioning ready for operation. The Fiducial camera then checks the panels fiducial marks ensuring safe to proceed. The handling assembly then attaches its selected gripper type ready for transportation. Once this is done the milling tool proceeds to cut out individual PCB’s which are held securely by the grippers and then transported into the toting. The reasons for this machine over traditional methods such as manual hand cutters or wheel cutters are that it is.

- Faster

- Repeatable

- Produces cleaner cuts, not leaving edges pointing out or cutting into the board.

- Greater accuracy

(LASER, 2023)

Fig. 7, PCBs in toting

Fig.4, Panel entering Depanelar.

Fig. 6, Z-Axis handling assembly and gripper station

Fig. 5, Z-Axis milling head.

## 2.2 Current Maintenance Practice

### 2.2.1 What is Maintenance and its benefits?

The Purpose of maintenance is to ensure plant and assets equipment are maintained correctly to prevent failure/interrupting being introduced to production (J., 2001). Maintenance practices can also extend the lifetime of an equipment along with increasing equipment efficiency and safety in the workplace (Gibson, 2023).

2.2.2 The current driving qualities and potential areas for improvement:

Pros

- Standard Job Plans, this ensures repeatability within the delivery in critical work. This also links with standard of training as the introduction to driving tests are essential before working competently alone.

- Using a software known as ‘Syteline’, all maintenance jobs are automatically scheduled in and notifies all about upcoming planned maintenance. This allows production to plan their work around.  Syteline also records all maintenances activities completed on depanelar allowing a history log to be created.

- Spears and lubricants are stored correctly and a system in place using ‘Coswin’ allows automatic store replacement when stock dops below minimum stock level to prevent extension of downtime looking for items.

Cons

- Does not include loader and unloader equipment within the maintenance schedule.

- Does not carry out industrial checks on machines vibration and noise management.

(GOV.UK, 2023)

### 2.3 Types of maintenance

Quick summary of three maintenance approaches.

Reactive – Known as Firefighting this approach is where equipment is run till failure in which maintenance personnel are then sent to repair/replace equipment to get operations running again, which may include temporarily repairs. This form allows to minimize manpower and money spent. But introduces unpredictable downtime and higher level of poor-quality products which is overall more expensive.

Proactive – The strategy is about avoiding breakdowns through actively maintain and monitoring any signs of deterioration and then carrying out activities (Minor repairs) to restore its correct conditions. This can be done in two forms, Preventive and predictive. Preventive occurs on the same schedule every cycle, whether is needed or not, while Predictive occurs as needed drawing on real-time data collection.

(ATS, 2023)

Aggressive – Is the strategy where Tools such as Total Productive Maintenance (TPM) are introduced to ensure all failures are avoided while seeking Overall Equipment Operation improvements. This is done though involvements in efforts to improve design. TPM is a management tool developed in Japan to support Just-in-Time (JIT) and eliminate the six losses. TPM can resemble a partnership approach maintenance where small groups/teams correspond between maintenance and production to help accomplish maintenance work through production workers becoming involved in preforming maintenance jobs allowing them to play a role in equipment monitoring and upkeeping.

(Swanson, 2001)

## 2.4 Review

All Current maintenance practice done on the ASYS Depanelar has to be carried out by a maintenance personnel, due to their correct training and competency on the equipment.

After reviewing the current maintenance strategy, it is clear the team has adopted a Proactive preventative approach primarily but when discussing with the senior maintenance engineer, understandably a reactive approach is sometimes practiced (not by choice) as not all breakdowns can be avoided by using maintenance strategies due to the nature of machines aging. Unlike traditional preventative maintenance strategies, where activities are split into monthly, quarterly, half yearly and yearly depending on its potential to cause failure. This is not the case for the current ASYS Depanelar strategy as the activities for the quarterly, half yearly and yearly has been spread over the 12 monthly schedules. This was made clear when comparing the current used scheduling with the provided manufacturers strategy found in the machine’s manual. This as a result can have many impacts of the utilisation of how the machine is maintained. Examples of this miss use with regards to the current maintenance will include.

- Over maintaining – The current maintenance requires the Z-Axis grease rail points to be topped up every month, but in the manufacturers guide it suggests once every quarter. This has in the past caused issues as too much grease has been pumped in causing the bearing to snap under pressure.

Fig. 8, PCB dust in Ruwac bin

Missing/skipping maintenance – As highlight the yearly maintenance has been split over the 12 monthly schedules. This devalues the importance of a yearly activity as monthly maintenance can sometime be overlooked and forgotten whereas a yearly maintenance is seen more as priority due to its infrequency and therefore stressing its importance. An example of a yearly on the depanelar that has potential to be overlooked due to this strategy, is the removal of PCB hazardous dust from RUWAC Extraction.          If this is missed it can cause the extraction bin to overfill, no longer extracting toxic PCB dust from inside machine and therefore becoming airborne. (Ruwac, 2023) Highlights its importance as this waste can cause cancer.

- Not designed for capacity – Within the ASYS computer software there is a maintenance page that has a list of activities needed to be done these are determined not on time base (Preventive) but on utilisation of the machine being ran (Predictive). If these activities are not completed and acknowledged on the software, then the machine will not operate resulting a breakdown. These activities could be tool changes, creating backups and others. These are also carried out by maintenance personnel.

Fig.9, Maintenance time tab on ASYS software

## 2.5 Why has this approach been chosen.

It is unclear as to why this choice has been chosen as this maintenance strategy was passed down along with the machine from the Woodchester site before Miskin had opened in 2012. Potentially due overwhelming number of equipment, the Woodchester maintenance team split the larger maintenances over the year to ease the pressure in downtime, so production wouldn’t get affected and engineers weren’t forced to stay on a yearly maintenance to prevent it being voided while they get called to breakdowns mid job.

# 3.0 A full evaluation of how the current approaches to maintenance could be improved.

## 3.1 How the current maintenance strategy can be improved, followed with suggested alternative approaches that could be adopted/implemented in future maintenance strategy.

After reviewing the current maintenance strategy its clean to see there are changes that need to be made as the situation has changed since the current strategy was created. Moving forward the proactive preventative strategy should be scrapped as it was never done properly. Which can see advised by manufacturers within the manual. If this preventive strategy was to continue it would have to return to the traditional Monthly, Quarterly, half yearly and yearly as the current method has proven too vulnerable for activities to be overlooked and therefore reducing safety and performance of asset.

Preferably after evaluating the equipment, a Proactive predictive maintenance approach should be adopted instead. For this strategy to work efficiently, data will be required collecting alongside using the ASYS maintenance software to create a more personal schedule to meet the equipment’s needs given its utilisation performance in-real-time.  This will provide greater use of maintenance resources and time as some months are not as busy as other therefore equipment will not require as much attention or removing the machine from production for maintenance purposes when not needed.

Moving forward for future Implementations an aggressive maintenance strategy might be good as it introduces TPM, this will involve more teams and individuals to improve the maintenance of the asset. For example, introducing operators to the more frequent maintenances that do not require having high-technical training/knowledge to carry out. Doing this provides beneficial for the machine and operators as less call outs will be required, and better skilled-working operators will be gained. This could involve tool changes, backing up of data and general inspection. TPM can also involve those in the design team in which new methods for maintenance can be developed.

# 4.0 Consideration of costs and resources required to implement proposed changes.

## Costs and resources.

For the implementation outlined within section 3.0 Evaluation and improvements to the current approach, costing and resources were assessed and looked over.

It was found that regarding costing, there will be no additional costings required to forward improvements, as the predictive approach will cut a large amount of unrequired preventative maintenance time out while still delivering the same high-level of maintenance. Using machine quantitative data to accurately determine when maintenance is required. Its also unclear to estimate the saving factors as each month will differ in volume production.

Another costing/resource factor that has been considered is the time spent on the machine by maintenance engineers as the goal will be to pass some responsibilities to operators to carry out while running the machine. This will reduce time spent on the maintenance carried out by maintenance engineers, allowing better utilisation of their time.

## Change management implications.

Changes within management of the depanelar maintenance will require a new process for notifying near maintenance schedules as the maintenance will no longer be the same time every month. New methods for predicting maintenance will need to be introduced while maintaining good communication with production management and other engineers. Training (driving tests) infrastructure will be implemented for operators that will take on greater responsibilities for the depanelar to ensure safety at work.

Leadership with good communication (show and tell) and Seek feedback with clear deadlines.

Crucial to change management;

Execute

Prepare for change.

- Manage (monitor) change.

- Reinforce change.

(Sumitani, 2023)

# 5.0 Conclusions/ Recommendations.

To conclude, the current maintenance approach has been highlighted as inappropriate and over the top for the given asset. Therefore, suggestions have been recommended to improve the strategy. This includes stepping away from a preventative approach and inheriting a predictive maintenance strategy, as the existing maintenance has shown limitation while the proposed option shows to be catered more to the machine’s nature while potentially saving in costs and resources. Long term improvements will look at slowly introducing an aggressive maintenance approach, specifically TPM as it’s a continuous improvement tool to look at improving the overall efficiency of the machine/maintenance through involvement of small teams to ease responsibilities. The only consideration within change management is a need for a new infrastructure to help notify when maintenance is required this can be achieved using the ASYS software. This is an example of using TPM.

# 6.0 References

ATS. (2023, March 27). Difference Between Preventive vs. Predictive Maintenance. Retrieved from Advanced Tech Service: https://www.advancedtech.com/blog/preventive-vs-predictive-maintenance/

Gibson, M. (2023, March 23). Top 5 Benefits of Maintenance Management. Retrieved from Cloudapper: https://www.cloudapper.com/computerized-maintenance-management/top-five-maintenance-management-benefits/

GOV.UK. (2023, March 29). Guidance Noise and vibration management: environmental permits. Retrieved from GOV.UK: https://www.gov.uk/government/publications/noise-and-vibration-management-environmental-permits/noise-and-vibration-management-environmental-permits

GROUP, A. (2023, March 20). High-end Depaneling Systems from the Market Leader. Retrieved from ASYSY GROUP: https://www.asys-group.com/en-us/products/electronics/depaneling/#c10208

J. E. (2001). The Present Status of Maintenance Strategies and the Impact of Maintenance on Reliability. IEEE Transactions on Power Systems, vol. 16, no. 4, 638-646.

LASER, C. (2023, March 27). Methods of PCB Depaneling: From Manual to Laser. Retrieved from CMS LASER: https://cmslaser.com/blog/methods-of-pcb-depaneling-from-manual-to-laser/

Renishaw. (2023, March 22). About Renishaw. Retrieved from Renishaw.com: https://www.renishaw.com/en/about-renishaw--6432

Ruwac. (2023, March 29). PCB-Process Deduster. Retrieved from Ruwac Asia: https://www.ruwac-asia.com/fileadmin/media/Products/ApplicationPackage/Ruwac_PCB_Deduster_17-8-2018.pdf

Sumitani, A. (2023, March 30). How to Implement Change Management: A Step-By-Step Guide to Successful Organizational Change. Retrieved from Tiny Pulse: https://www.tinypulse.com/blog/how-to-implement-change-management-a-step-by-step-guide-to-successful-organizational-change

Swanson, L. (2001). Linking maintenance strategies to performance. Int. J. Production Economics 70, 237-244.

# 7.0 Appendices

## 7.1 Current Depanelar schedules (January – December)