Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

The router will block further password attempts if the incorrect password has been entered incorrectly twice in 120 seconds

What does the block-for 30 mean in the above command?

This block will last for 30 seconds. The user will then be free to retry entering the correct password

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

R1 does not accept the telnet connection. Since SSH has been activated, only SSH connections will be accepted by R1

Use Tera Term, establish an SSH session to R1. Use the username admin and password adminpass. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection? Yes, the SSH connection has been accepted

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

Upon the second incorrect password attempt, the connection is terminated with error message “The current session has closed. Do you want to reestablish a new session with the previous session options?” If “yes” is clicked within 30 seconds, then the connection will not re-open. Waiting longer than 30 seconds allows connection to be re-opened and brings up password login option. This is due to the “block-for” command we configured earliersj

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources.

R1#

After the 30 seconds has expired, SSH to R1 again and login using the SSHadmin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed? ________________________

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain.

You are not disconnected. The security measures prevent further logon attempts after 2 incorrect passwords, but only when establishing the SSH connection. Once this connection has been established and the user has logged into the SSH connection, the incorrect password attempts trying to enter the Priv Exec mode do not count against the “block-for” security measure we

- Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

Current configuration : 1216 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 10

!

hostname R1

!

login block-for 30 attempts 2 within 120

!

!

enable secret 5 $1$mERr$5wXi8fVAu3dNc0gNCjvIQ1

enable password 7 0822455D0A16

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

username admin privilege 15 secret 5 $1$mERr$xONyqSvEtuqaiormJVqR7.

!

!

license udi pid CISCO1941/K9 sn FTX1524TO02-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name ccna-lab.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended sl_def_acl

deny tcp any any eq telnet

deny tcp any any eq www

deny tcp any any eq 22

permit tcp any any eq 22

!

banner motd ^CAdmin Access Only. No unauthorised access^C

!

!

!

!

!

line con 0

exec-timeout 5 0

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0822455D0A16

login local

transport input ssh

line vty 5 15

password 7 0822455D0A16

- login

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret adminpass

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch?

Yes. The configuration for the switch works in the same way as the router and the SSH connection is allowed

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

R1#ssh -v ?

1  Protocol Version 1

2  Protocol Version 2

How would you provide multiple users, each with their own username, access to a network device?

Using the “username” command, we could add new users to the local database we set up earlier

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g AES, SHA, DES, RSA.

Encryption is the process of encoding data so that only authorized users can access it. During the 1970’s, DES (Date encryption standard) was the encryption standard used. DES was based on an algorithm developed by IBM and modified by the National Security Agency (NSA).  However, due to advances in computing power and the relatively small block size of 64 bits, by the 1990’s, this standard became possible to break in just several days[1]. Despite DES now being obsolete, 3DES is still used and considered to be reasonably safe due to the fact it uses a 112 bit encryption key. This, however, will change in 2023 when, due to instruction by NIST (National Institute of Standards and Technology) 3DES will be officially retired. The standard that has taken its place is AES (Advanced Encryption Standard). AES became the standard in 2001 and AES can use Key lengths of 128, 192, and 256 bits [2]. AES is a military grade encryption, widely used by the American government as well as governments, agencies, militaries, businesses and people worldwide. The AES-256 block cipher has never been cracked and despite the best efforts of research scientists, no meaningful attacks have ever been carried out. A notable attempt by Andrey Bogdanov, Dmitry Khovratovich, and Christian Rechberger [3] lowered the brute force attack time by a factor of four, but this still meant that a brute force attack time on a 126 bit key would be billions of years [4]. NIST estimate that this encryption standard could remain unbreakable and hence relevant for beyond 20 years in the future [5]

Some of AES’s common applications include [6]:

- WinZip

- VeraCrypt

- Signal

- WhatsApp

- TLS

- SSH

AES is a private key encryption standard. This means a single key is shared between the sender and receiver.

A widely used public-key encryption method is RSA ( named after its creators Rivest–Shamir–Adleman). “RSA encryption is a system that solves what was once one of the biggest problems in cryptography: How can you send someone a coded message without having an opportunity to previously share the code with them?” [7].  With RSA encryption, data is encrypted with a public key which is shared openly. Once a message has been encrypted with the public key, it can only be decrypted by another key, known as the private key. Each RSA user has a key pair consisting of their public and private keys. Public key encryption schemes differ from symmetric-key encryption, where both the encryption and decryption process use the same private key. These differences make public key encryption like RSA useful for communicating in situations where there has been no opportunity to safely distribute keys beforehand [7]. RSA is quite resource heavy and less efficient than other encryption methods and as a result, isn’t generally used to encrypt entire files. “To make things more efficient, a file will generally be encrypted with a symmetric-key algorithm, and then the symmetric key will be encrypted with RSA encryption. Under this process, only an entity that has access to the RSA private key will be able to decrypt the symmetric key” [7]. RSA encryption is widely used when establishing connections between VPN clients and servers, with the RSA algorithm exchanging keys and establishing a secure connection. RSA is considered secure when keys of 2048 bit of more are implemented.

Hashing varies from encryption. “A hash is a string or number generated from a string of text” [8]. Where encryption allows for the decrypting of data, hashing is designed to be a one way function. Once a set of data has been hashed, there should be no way to determine what the original data was from the hash. One of the most commonly used hashing algorithms is SHA (Secure Hash algorithm). SHA-1 is now considered to be obsolete, whereas SHA-2 is still used and SHA-3 being the newest and most robust version. Hashing should be implemented for functions such as storing passwords. Instead of storing passwords in plain text, leaving them vulnerable to attackers, passwords should be hashed and salted. The hash of the password should be stored, then the next time a user enters their password, the hashes should be compared. This means if any bad faith actors ever compromised a system and were able to access passwords, the data they would find would be meaningless and offer no way for them to compromise a user account.

Hashing is also use to verify the integrity of files. A file can be hashed, providing a fixed length string that corresponds to that file. Even the smallest of changes made to that file will produce a vastly different string when hashed again. Software developers will often display the hash of their software online when offering a download so users can check whether the file they have downloaded matches exactly with what the developer has provided.

| [1] | B. K. S. a. C. R. Johnson, “A Brief History of Encryption,” 19 July 2010. [Online]. Available: https://www.technewsworld.com/story/70437.html. |
| --- | --- |
| [2] | Cisco, “Cisco,” 3 August 2019. [Online]. Available: https://community.cisco.com/t5/security-documents/aes/ta-p/3113918. [Accessed 11 December 2019]. |
| [3] | D. K. a. C. R. Andrey Bogdanov, “Biclique Cryptanalysis of the Full AES,” 2011. |
| [4] | B. Scott, “NordPass,” 19 April 2019. [Online]. Available: https://nordpass.com/blog/military-grade-encryption-explained/. [Accessed 11 December 2019]. |
| [5] | NIST, “NIST,” 4 December 2001. [Online]. Available: https://www.nist.gov/news-events/news/2001/12/commerce-secretary-announces-new-standard-global-information-security. [Accessed 11 December 2019]. |
| [6] | J. Lake, “comparitech,” 12 December 2018. [Online]. Available: https://www.comparitech.com/blog/information-security/encryption-types-explained/. [Accessed 11 December 2019]. |
| [7] | “SRI Tutrorials,” 11 December 2018. [Online]. Available: https://sritutorials.com/what-is-rsa-encryption-and-how-does-it-work/. [Accessed 11 December 2019]. |
| [8] | “Securityinnovationeruope,” 31 October 2016. [Online]. Available: https://securityinnovationeurope.com/blog/page/whats-the-difference-between-hashing-and-encrypting. [Accessed 11 December 2019]. |
| [9] | “Biclique Cryptanalysis of the Full AES”. |