Faculty of Applied Design and Engineering/Cyfadran Dylunio Cymhwysol a Pheirianneg

School of Applied Computing/Ysgol Cyfrifiadureg Gymhwysol

MSc Computer Networks

Implementing IP Routing & Switching Practical Assessment

Resit

Name			:

Student ID		:

Date & Time		:

This is an individual practical lab-test, held under examination conditions. You will be assigned four Routers, you should configure these Routers based on the following specifications.

The routers that you will configure are: R1, R2, R3, and R4.

This paper is marked out of 100.

You will have 2 hours to complete this task.

Network Topology

Addressing Scheme Table (5 marks)

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | Fast Ethernet 0/0 | 172.16.12.1 | 255.255.255.0 |
|  | Loopback0 | 172.16.1.1 | 255.255.255.0 |
| R2 | Fast Ethernet 0/0 | 172.16.12.2 | 255.255.255.0 |
|  | Serial 0/0/1 | 172.16.23.2 | 255.255.255.0 |
|  | Loopback120 | 172.16.120.1 | 255.255.255.0 |
|  | Loopback121 | 172.16.121.1 | 255.255.255.0 |
|  | Loopback122 | 172.16.122.1 | 255.255.255.0 |
|  | Loopback123 | 172.16.123.1 | 255.255.255.0 |
| R3 | Serial 0/0/1 | 172.16.23.3 | 255.255.255.0 |
|  | Serial 0/1/0 | 192.168.34.3 | 255.255.255.0 |
|  | Serial 0/1/1 | 192.168.43.3 | 255.255.255.0 |
|  | Loopback0 | 172.16.3.1 | 255.255.255.0 |
| R4 | Serial 0/0/0 | 192.168.34.4 | 255.255.255.0 |
|  | Serial 0/0/1 | 192.168.43.4 | 255.255.255.0 |
|  | Loopback 200 | 172.31.0.1 | 255.255.255.0 |
|  | Loopback 204 | 172.31.4.1 | 255.255.255.0 |
|  | Loopback 208 | 172.31.8.1 | 255.255.255.0 |
|  | Loopback 212 | 172.31.12.1 | 255.255.255.0 |

Requirements:

Task 1 – Network Connections (15 marks)

Physically cable the network as shown in the topology diagram.

Configure the router with hostname, Passwords (Console, Vty and privilege EXEC), line description, banner, disable DNS lookup, and synchronous logging.

Configure interfaces with the IP addresses shown in the diagram. Set the bandwidth to 128 kbps on the links between R1, R2, and R3 and set the clock rate on the DCE connections as appropriate. Add a description for each WAN interface. The T3 and T1 designations are simply labels; you do not need to change the bandwidth or clockrate on the interfaces to reflect a speed difference.

Task 2 – Configure Routing (60 marks)

- Configure interfaces with the IP addresses shown in the diagram.

- Add the loopback interface on R3 and the connection between R2 and R3 to OSPF area 0.

- Add the loopback interfaces on R2 into OSPF area 20. Summarize this area to the backbone with the most efficient mask.

- Put the connection between R1 and R2 in OSPF area 25. Make this area a totally NSSA area.

- Redistribute R1’s loopback interface into OSPF.

- Redistribute the T1 and T3 WAN links into OSPF on R3.

- Make sure that all loopback interfaces get advertised with the correct mask into OSPF.

- Configure R1 and R2 to be in BGP AS 65100. Configure R4 to be in BGP AS 65400.

- Peer R3 and R4 in BGP on both WAN links.

- Advertise R4’s loopback interfaces into BGP.

- Add a static route on R4 to 172.16.0.0/16 pointing to the T3 link to R3. Add a floating static route as a backup using the T1 link.

- Make AS 65100 prefer the T3 link to AS 65400 networks by modifying the local preference.

- Configure R2 to act as DHCP server on the 172.16.12.0 network. Make R2 the default gateway.

### Task3: Test network connectivity and path control (20 marks)

- Create a Tcl script and test connectivity from each router to the addresses shown in the diagram.

tclsh

foreach address {

192.168.1.1

192.168.3.1

192.168.110.1

192.168.111.1

192.168.112.1

192.168.113.1

192.168.120.1

192.168.121.1

192.168.122.1

192.168.123.1

192.168.12.1

192.168.12.2

192.168.23.2

192.168.23.3

172.16.34.3

172.16.34.4

172.16.43.3

172.16.43.4

198.133.219.1

209.165.200.225

FEC0::13:1

FEC0::13:3

FEC0::11:1

FEC0::31:1

} {

ping $address }

Note: One or more addresses will not be reachable from all routers after final configuration due to the use of distribute lists to filter routes and the use of IPv4 and IPv6 addressing in the same network.

- Verify that filtered routes are not present in routing tables of the correct routers.

- Verify that traffic from AS 65101 to AS 65401 uses the preferred T1 route.

- Verify that R4 has a backup route to the AS 65101 networks if the T1 link fails.

- Verify that R1 and R3 have learned of their respective IPv6 networks via RIPng.

- Verify that R1 can ping the R3 Lo3 interface via the IPv6 tunnel. R1 and R3 should also be able to ping the tunnel endpoints.

Submission Requirements:

All documentation for your solution should be submitted. Your name should be clearly printed on the documents, and signed.

At the end of the test a flash memory will be given to you, the configuration files for all routers should be copied to the media and handed in to your instructor.

The file name should be studentname_ID_routername.txt

Show running-configuration

Show ip interface brief

Show ip route

Show ip protocols

Show ip bgp

tclsh

Solution:

Final Configurations

R1#show run

hostname R1

!

interface Loopback0

ip address 172.16.1.1 255.255.255.0

!

interface FastEthernet0/0

ip address 172.16.12.1 255.255.255.0

no shutdown

!

router ospf 1

area 25 nssa

redistribute connected subnets

network 172.16.12.0 0.0.0.255 area 25

end

R2#show run

hostname R2

!

ip dhcp pool mypool

network 172.16.12.0 255.255.255.0

default-router 12.16.12.2

!

interface Loopback120

ip address 172.16.120.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback121

ip address 172.16.121.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback122

ip address 172.16.122.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback123

ip address 172.16.123.1 255.255.255.0

ip ospf network point-to-point

!

interface FastEthernet0/0

ip address 172.16.12.2 255.255.255.0

no shutdown

!

interface Serial0/0/1

ip address 172.16.23.2 255.255.255.0

no shutdown

!

router ospf 1

area 20 range 172.16.120.0 255.255.252.0

area 25 nssa no-summary

network 172.16.12.0 0.0.0.255 area 25

network 172.16.23.0 0.0.0.255 area 0

network 172.16.120.0 0.0.3.255 area 20

!

router bgp 65100

no synchronization

neighbor 172.16.23.3 remote-as 65100

no auto-summary

end

R3#show run

hostname R3

!

interface Loopback0

ip address 172.16.3.1 255.255.255.0

ip ospf network point-to-point

!

interface Serial0/0/1

ip address 172.16.23.3 255.255.255.0

no shutdown

!

interface Serial0/1/0

ip address 192.168.34.3 255.255.255.0

no shutdown

!

interface Serial0/1/1

ip address 192.168.43.3 255.255.255.0

no shutdown

!

router ospf 1

redistribute connected subnets

network 172.16.3.0 0.0.0.255 area 0

network 172.16.23.0 0.0.0.255 area 0

!

router bgp 65100

no synchronization

neighbor 172.16.23.2 remote-as 65100

neighbor 192.168.34.4 remote-as 65400

neighbor 192.168.43.4 remote-as 65400

neighbor 192.168.43.4 route-map IN-ON-T3 in

no auto-summary

!

route-map IN-ON-T3 permit 10

set local-preference 200

end

R4#show run

hostname R4

!

interface Loopback200

ip address 172.31.0.1 255.255.255.0

!

interface Loopback201

ip address 172.31.4.1 255.255.255.0

!

interface Loopback208

ip address 172.31.8.1 255.255.255.0

!

interface Loopback212

ip address 172.31.12.1 255.255.255.0

!

interface Serial0/0/0

ip address 192.168.34.4 255.255.255.0

no shutdown

!

interface Serial0/0/1

ip address 192.168.43.4 255.255.255.0

no shutdown

!

router bgp 65400

no synchronization

network 172.31.0.0 mask 255.255.255.0

network 172.31.4.0 mask 255.255.255.0

network 172.31.8.0 mask 255.255.255.0

network 172.31.12.0 mask 255.255.255.0

neighbor 192.168.34.3 remote-as 65100

neighbor 192.168.43.3 remote-as 65100

no auto-summary

!

ip route 172.16.0.0 255.255.0.0 192.168.43.3

ip route 172.16.0.0 255.255.0.0 192.168.34.3 10

end